LOTUS /INTEL /MICROSOFT

EXPANDED MEMORY SPECIFICATION

Version 4.0

300275-005

October, 1987

Copyright (c) 1987

Lotus Development Corporation

55 Cambridge Parkway

Cambridge, MA 02142

Intel Corporation

5200 NE Elam Young Parkway

Hillsboro, OR 97124

Microsoft Corporation

16011 NE 36TH Way

Box 97017

Redmond, WA 98073

This specification was jointly developed by Lotus Development

Corporation, Intel Corporation, and Microsoft Corporation.  Although

it has been released into the public domain and is not confidential or

proprietary, the specification is still the copyright and property of

Lotus Development Corporation, Intel Corporation, and Microsoft

Corporation.

DISCLAIMER OF WARRANTY

LOTUS DEVELOPMENT CORPORATION, INTEL CORPORATION, AND MICROSOFT

CORPORATION EXCLUDE ANY AND ALL IMPLIED WARRANTIES, INCLUDING

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.

NEITHER LOTUS NOR INTEL NOR MICROSOFT MAKE ANY WARRANTY OF

REPRESENTATION, EITHER EXPRESS OR IMPLIED, WITH RESPECT TO THIS

SPECIFICATION, ITS QUALITY, PERFORMANCE, MERCHANTABILITY, OR FITNESS

FOR A PARTICULAR PURPOSE.  NEITHER LOTUS NOR INTEL NOR MICROSOFT SHALL

HAVE ANY LIABILITY FOR SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES

ARISING OUT OF OR RESULTING FROM THE USE OR MODIFICATION OF THIS

SPECIFICATION.

This specification uses the following trademarks:

Intel is a trademark of Intel Corporation

Lotus is a trademark of Lotus Development Corporation

Microsoft is a trademark of Microsoft Corporation

1

CHAPTER 1

INTRODUCTION

Because even the maximum amount (640K bytes) of conventional memory

isn't always enough for large application programs, Lotus Development

Corporation, Intel Corporation, and Microsoft Corporation created the

Lotus/Intel/Microsoft (LIM) Expanded Memory Specification.

The LIM Expanded Memory Specification defines the software interface

between the Expanded Memory Manager (EMM) -- a device driver that

controls and manages expanded memory -- and application programs that

use expanded memory.

WHAT IS EXPANDED MEMORY?

Expanded memory is memory beyond DOS's 640K-byte limit.  The LIM

specification supports up to 32M bytes of expanded memory.  Because

the 8086, 8088, and 80286 (in real mode) microprocessors can

physically address only 1M byte of memory, they access expanded memory

through a window in their physical address range.  The next section

explains how this is done.

HOW EXPANDED MEMORY WORKS

Expanded memory is divided into segments called logical pages.  These

pages are typically 16K-bytes of memory.  Your computer accesses

logical pages through a physical block of memory called a page frame.

The page frame contains multiple physical pages, pages that the

microprocessor can address directly.  Physical pages are also

typically 16K bytes of memory.

This page frame serves as a window into expanded memory.  Just as your

computer screen is a window into a large spreadsheet, so the page

frame is a window into expanded memory.

A logical page of expanded memory can be mapped into (made to appear

in) any one of the physical pages in the page frame.  Thus, a read or

write to the physical page actually becomes a read or write to the

associated logical page.  One logical page can be mapped into the page

frame for each physical page.

The page frame is located above 640K bytes.  Normally, only video

adapters, network cards, and similar devices exist between 640K and

1024K.

This specification also defines methods for operating systems and

environments to access expanded memory through physical pages below

640K bytes.  These methods are intended for operating

system/environment developers only.

2

CHAPTER 2

WRITING PROGRAMS THAT USE EXPANDED MEMORY

This chapter describes what every program must do to use expanded

memory and describes more advanced techniques of using expanded

memory.

This chapter also lists programming guidelines you should follow when

writing programs that use expanded memory and provides the listings of

some example programs.

WHAT EVERY PROGRAM MUST DO

This section describes the steps every program must take to use

expanded memory.

In order to use expanded memory, applications must perform these steps

in the following order:

1.  Determine if EMM is installed.

2.  Determine if enough expanded memory pages exist for your

application. (Function 3)

3.  Allocate expanded memory pages.  (Function 4 or 18)

4.  Get the page frame base address.  (Function 2)

5.  Map in expanded memory pages.  (Function 5 or 17)

6.  Read/write/execute data in expanded memory, just as if it were

conventional memory.

7.  Return expanded memory pages to expanded memory pool before

exiting.  (Function 6 or 18)

Table 2-1 overviews the functions while Chapter 3 describes each of

these functions in detail.  Example programs at the end of this

chapter illustrate using expanded memory.

TABLE 2-1.  THE BASIC FUNCTIONS

----------------------------------------------------------------------

Function                      Description

----------------------------------------------------------------------

1   The Get Status function returns a status code indicating whether

the memory manager hardware is working correctly.

2   The Get Page Frame Address function returns the address where the

64K-byte page frame is located.

3   The Get Unallocated Page Count function returns the number of

unallocated pages (pages available to your program) and the total

number of pages in expanded memory.

3

4   The Allocate Pages function allocates the number of pages

requested and assigns a unique EMM handle to these pages.

5   The Map/Unmap Handle Page function maps a logical page to a

specific physical page anywhere in the mappable regions of system

memory.

6   The Deallocate Pages deallocates the logical pages currently

allocated to an EMM handle.

7   The Get Version function returns the version number of the memory

manager software.

ADVANCED PROGRAMMING

In addition to the basic functions, the Lotus/Intel/Microsoft Expanded

Memory Specification provides several advanced functions which enhance

the capabilities of software that uses expanded memory.

The following sections describe the advanced programming capabilities

and list the advanced EMM functions.

Note.............................................................

Before using the advanced functions, programs should first call

Function 7 (Get Version) to determine whether the installed version of

EMM supports these functions.

SAVING THE STATE OF MAPPING HARDWARE

Some software (such as interrupt service routines, device drivers, and

resident software) must save the current state of the mapping

hardware, switch mapping contexts, manipulate sections of expanded

memory, and restore the original context of the memory mapping

hardware.   Use Functions 8 and 9 or 15 and 16 to save the state of

the hardware.

RETRIEVING HANDLE AND PAGE COUNTS

Some utility programs need to keep track of how expanded memory is

being used; use Functions 12 through 14 to do this.

MAPPING AND UNMAPPING MULTIPLE PAGES

Mapping multiple pages reduces the overhead an application must

perform during mapping.  Function 17 lets a program map (or unmap)

multiple pages at one time.

In addition, you can map pages using segment addresses instead of

physical pages.  For example, if the page frame base address is set to

D000, you can map to either physical page 0 or segment D000.  Function

25 (Get Mappable Physical Address Array) returns a cross reference

between all expanded memory physical pages and their corresponding

segment values.

4

REALLOCATING PAGES

Reallocating pages (Function 18) lets applications dynamically

allocate expanded memory pages without acquiring another handle or

obtain a handle without allocating pages.  Reallocating pages is an

efficient means for applications to obtain and release expanded memory

pages.

USING HANDLES AND ASSIGNING NAMES TO HANDLES

This specification lets you associate a name with a handle, so a

family of applications can share information in expanded memory.  For

example, a software package consisting of a word processor,

spreadsheet, and print spooler can share the same data among the

different applications.  The print spooler could use a handle name to

reference data that either the spreadsheet or word processor put in

expanded memory and could check for data in a particular handle name's

expanded memory pages.

Use Function 20 (Set Handle Name subfunction) to assign a handle name

to an EMM handle or Function 21 (Search for Named Handle subfunction)

to obtain the EMM handle associated with the handle name.  In

addition, you can use Function 14 (Get Handle Pages) to determine the

number of expanded memory pages allocated to an EMM handle.

USING HANDLE ATTRIBUTES

In addition to naming a handle, you can use Function 19 to associate

an attribute (volatile or non-volatile) with a handle name.  A non-

volatile attribute enables expanded memory pages to preserve their

data through a warmboot.

With a volatile attribute, the data is not preserved.  The default

attribute for handles is volatile.

Because using this function depends on the capabilities of the

expanded memory hardware installed in the system, you should use the

Get Attribute Capability subfunction before attempting to assign an

attribute to a handle's pages.

ALTERING PAGE MAPS AND JUMPING/CALLING

You can use Functions 22 (Alter Page Map and Jump) and 23 (Alter Page

Map and Call) to map a new set of values into the map registers and

transfer program control to a specified address within expanded

memory.  These functions can be used to load and execute code in

expanded memory.  An application using this feature can significantly

reduce the amount of conventional memory it requires.  Programs can

load needed modules into expanded memory at run time and use Functions

22 and 23 to transfer control to these modules.

Using expanded memory to store code can improve program execution in

many ways.  For example, sometimes programs need to be divided into

small overlays because of conventional memory size limitations.

Overlays targeted for expanded memory can be very large because LIM

EMS 4.0 supports up to 32M bytes of expanded memory.  This method of

loading overlays improves overall system performance by conserving

5

conventional memory and eliminating conventional memory allocation

errors.

MOVING OR EXCHANGING MEMORY REGIONS

Using Function 24 (Move/Exchange Memory Region), you can easily move

and exchange data between conventional and expanded memory.  Function

24 can manipulate up to one megabyte of data with one function call.

Although applications can perform this operation without this

function, having the expanded memory manager do it reduces the amount

of overhead for the application.  In addition, this function checks

for overlapping regions and performs all the necessary mapping,

preserving the mapping context from before the exchange/move call.

GETTING THE AMOUNT OF MAPPABLE MEMORY

Function 25 enables applications to determine the total amount of

mappable memory the hardware/system currently supports.   Not all

expanded memory boards supply the same number of physical pages (map

registers).

The Get Mappable Physical Address Array Entries subfunction returns

the total number of physical pages the expanded memory hardware/system

is capable of supporting.  The Get Mappable Physical Address Array

subfunction returns a cross reference between physical page numbers

and the actual segment address for each of the physical pages.

OPERATING SYSTEM FUNCTIONS

In addition to the functions for application programs, this

specification defines functions for operating systems/environments.

These functions can be disabled at any time by the operating

system/environment, so programs should not depend on their presence.

Applications that avoid this warning and use these functions run a

great risk of being incompatible with other programs, including the

operating system.

TABLE 2-2.  THE ADVANCED FUNCTIONS

----------------------------------------------------------------------

Function                 Description

----------------------------------------------------------------------

8   The Save Page Map saves the contents of the page mapping registers

from all expanded memory boards in an internal save area.

9   The Restore Page Map function restores (from an internal save

area) the page mapping register contents on the expanded memory

boards for a particular EMM handle.

10  Reserved.

11  Reserved.

12  The Get Handle Count function returns the number of open EMM

handles in the system.

6

13  The Get Handle Pages function returns the number of pages

allocated to a specific EMM handle.

14  The Get All Handle Pages function returns an array of the active

EMM handles and the number of pages allocated to each one.

15  The Get/Set Page Map subfunction saves or restores the mapping

context for all mappable memory regions (conventional and

expanded) in a destination array which the application supplies.

16  The Get/Set Partial Page Map subfunction provides a mechanism for

saving a partial mapping context for specific mappable memory

regions in a system.

17  The Map/Unmap Multiple Handle Pages function can, in a single

invocation, map (or unmap) logical pages into as many physical

pages as the system supports.

18  The Reallocate Pages function can increase or decrease the amount

of expanded memory allocated to a handle.

19  The Get/Set Handle Attribute function allows an application

program to determine and set the attribute associated with a

handle.

20  The Get/Set Handle Name function gets the eight character name

currently assigned to a handle and can assign an eight character

name to a handle.

21  The Get Handle Directory function returns information about active

handles and the names assigned to each.

22  The Alter Page Map & Jump function alters the memory mapping

context and transfers control to the specified address.

23  The Alter Page Map & Call function alters the specified mapping

context and transfers control to the specified address.  A return

can then restore the context and return control to the caller.

24  The Move/Exchange Memory Region function copies or exchanges a

region of memory from conventional to conventional memory,

conventional to expanded memory, expanded to conventional memory,

or expanded to expanded memory.

25  The Get Mappable Physical Address Array function returns an array

containing the segment address and physical page number for each

mappable physical page in a system.

26  The Get Expanded Memory Hardware Information function returns an

array containing the hardware capabilities of the expanded memory

system.

27  The Allocate Standard/Raw Pages function allocates the number of

standard or non-standard size pages that the operating system

requests and assigns a unique EMM handle to these pages.

7

28  The Alternate Map Register Set function enables an application to

simulate alternate sets of hardware mapping registers.

29  The Prepare Expanded Memory Hardware for Warm Boot function

prepares the expanded memory hardware for an "impending" warm

boot.

30  The Enable/Disable OS/E function enables operating systems

developers to enable and disable functions designed for operating

system use.

8

PROGRAMMING GUIDELINES

The following section contains guidelines for programmers writing

applications that use EMM.

o   Do not put a program's stack in expanded memory.

o   Do not replace interrupt 67h.  This is the interrupt vector the

EMM uses.  Replacing interrupt 67h could result in disabling the

Expanded Memory Manager.

o   Do not map into conventional memory address space your application

doesn't own.  Applications that use the EMM to swap into

conventional memory space, must first allocate this space from the

operating system.  If the operating system is not aware that a

region of memory it manages is in use, it will think it is

available.  This could have disastrous results.  EMM should not be

used to 'allocate' conventional memory.  DOS is the proper manager

of conventional memory space.  EMM should only be used to swap

data in conventional memory space previously allocated from DOS.

o   Applications that plan on using data aliasing in expanded memory

must check for the presence of expanded memory hardware.  Data

aliasing occurs when mapping one logical page into two or more

mappable segments.  This makes one 16K-byte expanded memory page

appear to be in more than one 16K-byte memory address space.  Data

aliasing is legal and sometimes useful for applications.

Software-only expanded memory emulators cannot perform data

aliasing.  A simple way to distinguish software emulators from

actual expanded memory hardware is to attempt data aliasing and

check the results.  For example, map one logical page into four

physical pages.  Write to physical page 0.  Read physical pages 1-

3 to see if the data is there as well.  If the data appears in all

four physical pages, then expanded memory hardware is installed in

the system, and data aliasing is supported.

o   Applications should always return expanded memory pages to the

expanded memory manager upon termination.  These pages will be

made available for other applications.  If unneeded pages are not

returned to the expanded memory manager, the system could 'run

out' of expanded memory pages or expanded memory handles.

o   Terminate and stay resident programs (TSR's) should ALWAYS save

the state of the map registers before changing them.  Since TSR's

may interrupt other programs which may be using expanded memory,

they must not change the state of the page mapping registers

without first saving them.  Before exiting, TSR's must restore the

state of the map registers.

The following sections describe the three ways to save and restore

the state of the map registers.

1  Save Page Map and Restore Page Map (Functions 8 and 9).

This is the simplest of the three methods.  The EMM saves

the map register contents in its own data structures --

the application does not need to provide extra storage

9

locations for the mapping context.  The last mapping

context to be saved, under a particular handle, will be

restored when a call to Restore Page Map is issued with

the same handle.  This method is limited to one mapping

context for each handle and saves the context for only LIM

standard 64K-byte page frames.

2  Get/Set Page Map (Function 15).  This method requires the

application to allocate space for the storage array.   The

EMM saves the mapping context in an array whose address is

passed to the EMM.  When restoring the mapping context

with this method, an application passes the address of an

array which contains a previously stored mapping context.

This method is preferable if an application needs to do

more than one save before a restore.  It provides a

mechanism for switching between more than one mapping

context.

3  Get/Set Partial Page Map (Function 16).  This method

provides a way for saving a partial mapping context.  It

should be used when the application does not need to save

the context of all mappable memory.  This function also

requires that the storage array be part of the

application's data.

o   All functions using pointers to data structures must have those

data structures in memory which will not be mapped out.  Functions

22 and 23 (Alter Map and Call and Alter Map and Jump) are the only

exceptions.

10

CHAPTER 3

EMM FUNCTIONS

This chapter provides you with a standardized set of expanded memory

functions.  Because they are standardized, you avoid potential

compatibility problems with other expanded memory programs that also

adhere to the memory manager specification.  Programs that deal

directly with the hardware or that don't adhere to this specification

will be incompatible.

Table 3-1 presents a sequential list of the EMM functions. The

remainder of this chapter provides detailed descriptions of each

function.

TABLE 3-1.  LIST OF EMM FUNCTIONS

----------------------------------------------------------------------

Number       Function Name                        Hex Value Page

----------------------------------------------------------------------

1  Get Status                                     40h       14

2  Get Page Frame Segment Address                 41h       15

3  Get Unallocated Page Count                     42h       16

4  Allocate Pages                                 43h       17

5  Map/Unmap Handle Page                          44h       20

6  Deallocate Pages                               45h       22

7  Get Version                                    46h       24

8  Save Page Map                                  47h       26

9  Restore Page Map                               48h       28

10 Reserved                                       49h       30

11 Reserved                                       4Ah       31

12 Get Handle Count                               4Bh       32

13 Get Handle Pages                               4Ch       34

14 Get All Handle Pages                           4Dh       36

15 Get Page Map                                   4E00h     38

Set Page Map                                   4E01h     40

Get & Set Page Map                             4E02h     42

Get Size of Page Map Save Array                4E03h     44

11

16 Get Partial Page Map                           4F00h     46

Set Partial Page Map                           4F01h     48

Get Size of Partial Page Map Save Array        4F02h     50

17 Map/Unmap Multiple Handle Pages

(Physical page number mode)                    5000h     52

Map/Unmap Multiple Handle Pages

(Segment address mode)                         5001h     56

18 Reallocate Pages                               51h       59

19 Get Handle Attribute                           5200h     62

Set Handle Attribute                           5201h     65

Get Handle Attribute Capability                5202h     67

20 Get Handle Name                                5300h     68

Set Handle Name                                5301h     70

21 Get Handle Directory                           5400h     72

Search for Named Handle                        5401h     74

Get Total Handles                              5402h     76

22 Alter Page Map & Jump

(Physical page number mode)                    5500h     77

Alter Page Map & Jump

(Segment address mode)                         5501h     77

23 Alter Page Map & Call

(Physical page number mode)                    5600h     80

Alter Page Map & Call

(Segment address mode)                         5601h     80

Get Page Map Stack Space Size                  5602h     84

24 Move Memory Region                             5700h     86

Exchange Memory Region                         5701h     91

25 Get Mappable Physical Address Array            5800h     96

Get Mappable Physical Address Array Entries    5801h     99

26 Get Hardware Configuration Array               5900h     101

Get Unallocated Raw Page Count                 5901h     104

27 Allocate Standard Pages                        5A00h     106

12

Allocate Raw Pages                             5A01h     109

28 Get Alternate Map Register Set                 5B00h     112

Set Alternate Map Register Set                 5B01h     117

Get Alternate Map Save Array Size              5B02h     120

Allocate Alternate Map Register Set            5B03h     122

Deallocate Alternate Map Register Set          5B04h     124

Allocate DMA Register Set                      5B05h     126

Enable DMA on Alternate Map Register Set       5B06h     128

Disable DMA on Alternate Map Register Set      5B07h     130

Deallocate DMA Register Set                    5B08h     132

29 Prepare Expanded Memory Hardware for Warmboot  5Ch       134

30 Enable OS/E Function Set                       5D00h     135

Disable OS/E Function Set                      5D01h     138

Return OS/E Access Key                         5D02h     140

13

FUNCTION 1    GET STATUS

PURPOSE

The Get Status function returns a status code indicating whether the

memory manager is present and the hardware is working correctly.

CALLING PARAMETERS

AH = 40h

Contains the Get Status function.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager is present in the system, and the hardware is

working correctly.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

EXAMPLE

MOV  AH, 40h                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

14

FUNCTION 2    GET PAGE FRAME ADDRESS

PURPOSE

The Get Page Frame Address function returns the segment address where

the page frame is located.

CALLING PARAMETERS

AH = 41h

Contains the Get Page Frame function.

RESULTS

These results are valid only if the status returned is zero.

BX = page frame segment address

Contains the segment address of the page frame.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the page frame address in the BX

register.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

EXAMPLE

page_frame_segment                 DW  ?

MOV  AH, 41h                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  page_frame_segment, BX        ; save page frame address

15

FUNCTION 3    GET UNALLOCATED PAGE COUNT

PURPOSE

The Get Unallocated Page Count function returns the number of

unallocated pages and the total number of expanded memory pages.

CALLING PARAMETERS

AH = 42h   Contains the Get Unallocated Page Count function.

RESULTS

These results are valid only if the status returned is zero.

BX = unallocated pages

The number of expanded memory pages that are currently

available for use (unallocated).

DX = total pages

The total number expanded memory pages.

REGISTERS MODIFIED

AX, BX, DX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the number of unallocated pages

and the number of total pages in expanded memory.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

EXAMPLE

un_alloc_pages                     DW  ?

total_pages                        DW  ?

MOV  AH, 42h                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  un_alloc_pages, BX            ; save unallocated page count

MOV  total_pages, DX               ; save total page count

16

FUNCTION 4    ALLOCATE PAGES

PURPOSE

The Allocate Pages function allocates the number of pages requested

and assigns a unique EMM handle to these pages.  The EMM handle owns

these pages until the application deallocates them.

Handles which are assigned using this function will have 16K-byte

pages, the size of a standard expanded memory page.  If the expanded

memory board hardware isn't able to supply 16K-byte pages, it will

emulate them by combining multiple non-standard size pages to form a

single 16K-byte page.  All application programs and functions that use

the handles this function returns will deal with 16K-byte pages.

The numeric value of the handles the EMM returns are in the range of 1

to 254 decimal (0001h to 00FEh).  The OS handle (handle value 0) is

never returned by the Allocate Pages function.  Also, the uppermost

byte of the handle will be zero and cannot be used by the application.

A memory manager should be able to supply up to 255 handles, including

the OS handle.  An application can use Function 21 to find out how

many handles an EMM supports.

Allocating zero pages to a handle is not valid.  If an application

needs to allocate 0 pages to a handle it should use Function 27

provided for this purpose.

Note...............................................................

This note affects expanded memory manager implementers and operating

system developers only.  Applications should not use the following

characteristic of the memory manager.  An application violating this

rule will be incompatible with future versions of Microsoft's

operating systems and environments.

To be compatible with this specification, an expanded memory manager

will provide a special handle which is available to the operating

system only.  This handle will have a value of 0000h and will have a

set of pages allocated to it when the expanded memory manager driver

installs.  The pages that the memory manager will automatically

allocate to handle 0000h are those that backfill conventional memory.

Typically, this backfill occurs between addresses 40000h (256K) and

9FFFFh (640K).  However, the range can extend below and above this

limit if the hardware and memory manager have the capability.

An operating system won't have to invoke Function 4 to obtain this

handle because it can assume the handle already exists and is

available for use immediately after the expanded memory device driver

installs.  When an operating system wants to use this handle, it uses

the special handle value of 0000h. The operating system will be able

to invoke any EMM function using this special handle value.  To

allocate pages to this handle, the operating system need only invoke

Function 18 (Reallocate pages)..

There are two special cases for this handle:

17

1.  Function 4 (Allocate Pages).  This function must never return zero

as a handle value.  Applications must always invoke Function 4 to

allocate pages and obtain a handle which identifies the pages

which belong to it.  Since Function 4 never returns a handle value

of zero, an application will never gain access to this special

handle.

2.  Function 6 (Deallocate Pages). If the operating system uses it to

deallocate the pages which are allocated to this special handle,

the pages the handle owns will be returned to the manager for use.

But the handle will not be available for reassignment.  The

manager should treat a deallocate pages function request for this

handle the same as a reallocate pages function request, where the

number of pages to reallocate to this handle is zero.

CALLING PARAMETERS

AH = 43h

Contains the Allocate Pages function.

BX = num_of_pages_to_alloc

Contains the number of pages you want your program to

allocate.

RESULTS

These results are valid only if the status returned is zero.

DX = handle

Contains a unique EMM handle.  Your program must use this

EMM handle (as a parameter) in any function that requires

it.  You can use up to 255 handles.  The uppermost byte of

the handle will be zero and cannot be used by the

application.

REGISTERS MODIFIED

AX, DX

STATUS

AH = 0     SUCCESSFUL

The manager has allocated the requested pages to the

assigned EMM handle.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

18

AH = 85h   RECOVERABLE

All EMM handles are being used.

AH = 87h   RECOVERABLE

There aren't enough expanded memory pages present in the

system to satisfy your program's request.

AH = 88h   RECOVERABLE

There aren't enough unallocated pages to satisfy your

program's request.

AH = 89h   RECOVERABLE

Your program attempted to allocate zero pages.

EXAMPLE

num_of_pages_to_alloc              DW  ?

emm_handle                         DW  ?

MOV  BX, num_of_pages_to_alloc     ; load number of pages

MOV  AH, 43h                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  emm_handle, DX                ; save EMM handle

19

FUNCTION 5    MAP/UNMAP HANDLE PAGES

PURPOSE

The Map/Unmap Handle Page function maps a logical page at a specific

physical page anywhere in the mappable regions of system memory.

The lowest valued physical page numbers are associated with regions of

memory outside the conventional memory range.  Use Function 25 (Get

Mappable Physical Address Array) to determine which physical pages

within a system are mappable and determine the segment addresses which

correspond to a specific physical page number.  Function 25 provides a

cross reference between physical page numbers and segment addresses.

This function can also unmap physical pages, making them inaccessible

for reading or writing.  You unmap a physical page by setting its

associated logical page to FFFFh.

You might unmap an entire set of mapped pages, for example, before

loading and executing a program.  Doing so ensures the loaded program,

if it accesses expanded memory, won't access the pages your program

has mapped.  However, you must save the mapping context before you

unmap the physical pages.  This enables you to restore it later so you

can access the memory you mapped there.  To save the mapping context,

use Function 8, 15, or 16.  To restore the mapping context, use

Function 9, 15, or 16.

The handle determines what type of pages are being mapped. Logical

pages allocated by Function 4 and Function 27 (Allocate Standard Pages

subfunction) are referred to as pages and are 16K bytes long.  Logical

pages allocated by Function 27 (Allocate Raw Pages subfunction) are

referred to as raw pages and might not be the same size as logical

pages.

CALLING PARAMETERS

AH = 44h

Contains the Map Handle Page function.

AL = physical_page_number

Contains the number of the physical page into which the

logical page number is to be mapped.  Physical pages are

numbered zero relative.

BX = logical_page_number

Contains the number of the logical page to be mapped at the

physical page within the page frame.  Logical pages are

numbered zero relative.  The logical page must be in the

range zero through (number of pages allocated to the EMM

handle - 1).  However, if BX contains logical page number

FFFFh, the physical page specified in AL will be unmapped

(be made inaccessible for reading or writing).

DX = emm_handle

Contains the EMM handle your program received from Function

4 (Allocate Pages).

REGISTERS MODIFIED

20

AX

STATUS

AH = 0     SUCCESSFUL

The manager has mapped the page.  The page is ready to be

accessed.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 83h   NON-RECOVERABLE

The memory manager couldn't find the EMM handle your

program specified.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager isn't

defined.

AH = 8Ah   RECOVERABLE

The logical page is out of the range of logical pages which

are allocated to the EMM handle.  This status is also

returned if a program attempts map a logical page when no

logical pages are allocated to the handle.

AH = 8Bh   RECOVERABLE

The physical page number is out of the range of allowable

physical pages.  The program can recover by attempting to

map into memory at a physical page which is within the

range of allowable physical pages.

EXAMPLE

emm_handle                         DW  ?

logical_page_number                DW  ?

physical_page_number               DB  ?

MOV  DX, emm_handle                ; load EMM handle

MOV  BX, logical_page_number       ; load logical page number

MOV  AL, physical_page_number      ; load physical page number

MOV  AH, 44h                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

21

FUNCTION 6    DEALLOCATE PAGES

PURPOSE

Deallocate Pages deallocates the logical pages currently allocated to

an EMM handle.  Only after the application deallocates these pages can

other applications use them.  When a handle is deallocated, it name is

set to all ASCII nulls (binary zeros).

Note...............................................................

A program must perform this function before it exits to DOS. If it

doesn't, no other programs can use these pages or the EMM handle.

This means that a program using expanded memory should trap critical

errors and control-break if there is a chance that the program will

have allocated pages when either of these events occur.

Calling Parameters

AH = 45h

Contains the Deallocate Pages function.

DX = handle

Contains the EMM handle returned by Function 4 (Allocate

Pages).

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has deallocated the pages previously allocated

to the EMM handle.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 83h   NON-RECOVERABLE

The manager couldn't find the specified EMM handle.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 86h   RECOVERABLE

The memory manager detected a save or restore  page mapping

context error (Function 8 or 9).  There is a page mapping

register state in the save area for the specified EMM

handle.  Save Page Map (Function 8) placed it there and a

subsequent Restore Page Map (Function 9) has not removed

22

it.  If you have saved the mapping context, you must

restore it before you deallocate the EMM handle's pages.

EXAMPLE

emm_handle                         DW  ?

MOV  DX, emm_handle                ; load EMM handle

MOV  AH, 45h                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

23

FUNCTION 7    GET VERSION

PURPOSE

The Get Version function returns the version number of the memory

manager software.

CALLING PARAMETERS

AH = 46h

Contains the Get Version function.

RESULTS

These results are valid only if the status returned is zero.

AL = version number

Contains the memory manager's version number in binary

coded decimal (BCD) format.  The upper four bits contain

the integer digit of the version number.  The lower four

bits contain the fractional digit of version number.  For

example, version 4.0 is represented like this:

0100 0000

/   \

4  .  0

When checking for a version number, an application should

check for a version number or greater.  Vendors may use the

fractional digit to indicate enhancements or corrections to

their memory managers.  Therefore, to allow for future

versions of memory managers, an application shouldn't

depend on an exact version number.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager is present in the system and the hardware is

working correctly.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

EXAMPLE

24

emm_version                        DB  ?

MOV  AH, 46h                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  emm_version, AL               ; save version number

25

FUNCTION 8    SAVE PAGE MAP

PURPOSE

Save Page Map saves the contents of the page mapping registers on all

expanded memory boards in an internal save area.  The function is

typically used to save the memory mapping context of the EMM handle

that was active when a software or hardware interrupt occurred.  (See

Function 9, Restore Page Map, for the restore operation.)  If you're

writing a resident program, an interrupt service, or a device driver

that uses expanded memory, you must save the state of the mapping

hardware.  You must save this state because application software using

expanded memory may be running when your program is invoked by a

hardware interrupt, a software interrupt, or DOS.

The Save Page Map function requires the EMM handle that was assigned

to your resident program, interrupt service routine, or device driver

at the time it was initialized. This is not the EMM handle that the

application software was using when your software interrupted it.

The Save Page Map function saves the state of the map registers for

only the 64K-byte page frame defined in versions 3.x of this

specification.  Since all applications written to LIM versions 3.x

require saving the map register state of only this 64K-byte page

frame, saving the entire mapping state for a large number of mappable

pages would be inefficient use of memory.  Applications that use a

mappable memory region outside the LIM 3.x page frame should use

Function 15 or 16 to save and restore the state of the map registers.

CALLING PARAMETERS

AH = 47h

Contains the Save Page Map function.

DX = handle

Contains the EMM handle assigned to the interrupt service

routine that's servicing the software or hardware

interrupt.  The interrupt service routine needs to save the

state of the page mapping hardware before mapping any

pages.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has saved the state of the page mapping

hardware.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

26

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 83h   NON-RECOVERABLE

The memory manager couldn't find the EMM handle your

program specified.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Ch   NON-RECOVERABLE

There is no room in the save area to store the state of the

page mapping registers.  The state of the map registers has

not been saved.

AH = 8Dh   CONDITIONALLY-RECOVERABLE

The save area already contains the page mapping register

state for the EMM handle your program specified.

EXAMPLE

emm_handle                         DW  ?

MOV  DX, emm_handle                ; load EMM handle

MOV  AH, 47h                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

27

FUNCTION 9    RESTORE PAGE MAP

PURPOSE

The Restore Page Map function restores the page mapping register

contents on the expanded memory boards for a particular EMM handle.

This function lets your program restore the contents of the mapping

registers its EMM handle saved. (See Function 8, Save Page Map for the

save operation.)

If you're writing a resident program, an interrupt service routine, or

a device driver that uses expanded memory, you must restore the

mapping hardware to the state it was in before your program took over.

You must save this state because application software using expanded

memory might have been running when your program was invoked.

The Restore Page Map function requires the EMM handle that was

assigned to your resident program, interrupt service routine, or

device driver at the time it was initialized.  This is not the EMM

handle that the application software was using when your software

interrupted it.

The Restore Page Map function restores the state of the map registers

for only the 64K-byte page frame defined in versions 3.x of this

specification.  Since all applications written to LIM versions 3.x

require restoring the map register state of only this 64K-byte page

frame, restoring the entire mapping state for a large number of

mappable pages would be inefficient use of memory.  Applications that

use a mappable memory region outside the LIM 3.x page frame should use

Function 15 or 16 to save and restore the state of the map registers.

CALLING PARAMETERS

AH = 48h

Contains the Restore Page Map function.

DX = emm_handle

Contains the EMM handle assigned to the interrupt service

routine that's servicing the software or hardware

interrupt.  The interrupt service routine needs to restore

the state of the page mapping hardware.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has restored the state of the page mapping

registers.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

28

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 83h   NON-RECOVERABLE

The memory manager couldn't find the EMM handle your

program specified.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Eh   CONDITIONALLY-RECOVERABLE

There is no page mapping register state in the save area

for the specified EMM handle.  Your program didn't save the

contents of the page mapping hardware, so Restore Page Map

can't restore it.

EXAMPLE

emm_handle                         DW  ?

MOV  DX, emm_handle                ; load EMM handle

MOV  AH, 48h                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

29

FUNCTION 10   RESERVED

In earlier versions of the Lotus/Intel/Microsoft Expanded Memory

Specification the use of this function was documented.  This function

is now documented as "RESERVED" (i.e. not documented) in the public

versions of the specification and new programs should not use it.

Existing programs that use this function may still work correctly if

the hardware is capable of supporting them.  Since IBM Microchannel

hardware is not capable of supporting this function, it cannot be

implemented in the IBM Microchannel architecture.  However, programs

that use Functions 16 through 30 in Version 4.0 of this specification

must not use Functions 10 and 11.  These functions won't work

correctly if a program attempts to mix the use of the new functions

(Functions 16 through 30) and functions 10 and 11.  Functions 10 and

11 are specific to the hardware on Intel expanded memory boards and

will not work correctly on all vendors expanded memory boards.

30

FUNCTION 11   RESERVED

In earlier versions of the Lotus/Intel/Microsoft Expanded Memory

Specification the use of this function was documented.  This function

is now documented as "RESERVED" (i.e. not documented) in the public

versions of the specification and new programs should not use it.

Existing programs that use this function may still work correctly if

the hardware is capable of supporting them.  Since IBM Microchannel

hardware is not capable of supporting this function, it cannot be

implemented in the IBM Microchannel architecture.  However, programs

that use Functions 16 through 30 in Version 4.0 of this specification

must not use Functions 10 and 11.  These functions won't work

correctly if a program attempts to mix the use of the new functions

(Functions 16 through 30) and functions 10 and 11.  Functions 10 and

11 are specific to the hardware on Intel expanded memory boards and

will not work correctly on all vendors expanded memory boards.

31

FUNCTION 12   GET HANDLE COUNT

PURPOSE

The Get Handle Count function returns the number of open EMM handles

(including the operating system handle 0) in the system.

CALLING PARAMETERS

AH = 4Bh

Contains the Get Handle Count function.

RESULTS

These results are valid only if the status returned is zero.

BX = total_open_emm_handles

Contains the number of open EMM handles [including the

operating system handle (0)].  This number will not exceed

255.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the number of active EMM handles.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

32

EXAMPLE

total_open_emm_handles             DW  ?

MOV  AH, 4Bh                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  total_open_emm_handles, BX    ; save total active handle count

33

FUNCTION 13   GET HANDLE PAGES

PURPOSE

The Get Handle Pages function returns the number of pages allocated to

a specific EMM handle.

CALLING PARAMETERS

AH = 4Ch

Contains the Get Handle Pages function.

DX = emm_handle

Contains the EMM handle.

RESULTS

These results are valid only if the status returned is zero.

BX = num_pages_alloc_to_emm_handle

Contains the number of logical pages allocated to the

specified EMM handle.  This number never exceeds 2048

because the memory manager allows a maximum of 2048 pages

(32M bytes) of expanded memory.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the number of pages allocated to

the EMM handle.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 83h   NON-RECOVERABLE

The memory manager couldn't find the EMM handle your

program specified.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

34

EXAMPLE

emm_handle                         DW  ?

pages_alloc_to_handle              DW  ?

MOV  DX, emm_handle                ; load EMM handle

MOV  AH, 4Ch                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  pages_alloc_to_handle, BX     ; save number of pages allocated

; to specified handle

35

FUNCTION 14   GET ALL HANDLES PAGES

PURPOSE

The Get All Handle Pages function returns an array of the open emm

handles and the number of pages allocated to each one.

CALLING PARAMETERS

AH = 4Dh

Contains the Get All Handle Pages function.

handle_page_struct         STRUC

emm_handle              DW ?

pages_alloc_to_handle   DW ?

handle_page_struct         ENDS

ES:DI = pointer to handle_page

Contains a pointer to an array of structures where a copy

of all open EMM handles and the number of pages allocated

to each will be stored.  Each structure has these two

members:

.emm_handle

The first member is a word which contains the value of the

open EMM handle.  The values of the handles this function

returns will be in the range of 0 to 255 decimal (0000h to

00FFh).  The uppermost byte of the handle is always zero.

.pages_alloc_to_handle

The second member is a word which contains the number of

pages allocated to the open EMM handle.

RESULTS

These results are valid only if the status returned is zero.

BX = total_open_emm_handles

Contains the number of open EMM handles (including the

operating system handle [0]).  The number cannot be zero

because the operating system handle is always active and

cannot be deallocated.  This number will not exceed 255.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the array.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

36

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager isn't

defined.

EXAMPLE

handle_page                        handle_page_struct 255 DUP (?)

total_open_handles                 DW  ?

MOV  AX, SEG handle_page

MOV  ES, AX

LEA  DI, handle_page               ; ES:DI points to handle_page

MOV  AH, 4Dh                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  total_open_handles, BX        ; save total open handle count

37

FUNCTION 15   GET/SET PAGE MAP

GET PAGE MAP SUBFUNCTION

PURPOSE

The Get Page Map subfunction saves the mapping context for all

mappable memory regions (conventional and expanded) by copying the

contents of the mapping registers from each expanded memory board to a

destination array.  The application must pass a pointer to the

destination array.  This subfunction doesn't require an EMM handle.

Use this function instead of Functions 8 and 9 if you need to save or

restore the mapping context but don't want (or have) to use a handle.

CALLING PARAMETERS

AX = 4E00h

Contains the Get Page Map subfunction.

ES:DI = dest_page_map

Contains a pointer to the destination array address in

segment:offset format.  Use the Get Size of Page Map Save

Array subfunction to determine the size of the desired

array.

RESULTS

These results are valid only if the status returned is zero.

dest_page_map

The array contains the state of all the mapping registers

on all boards in the system.  It also contains any

additional information necessary to restore the boards to

their original state when the program invokes a Set

subfunction.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the array.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

38

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

dest_page_map                      DB  ?  DUP  (?)

MOV  AX, SEG dest_page_map

MOV  ES, AX

LEA  DI, dest_page_map             ; ES:DI points to dest_page_map

MOV  AX, 4E00h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

39

FUNCTION 15   GET/SET PAGE MAP (CONTINUED)

SET PAGE MAP SUBFUNCTION

PURPOSE

The Set Page Map subfunction restores the mapping context for all

mappable memory regions (conventional and expanded) by copying the

contents of a source array into the mapping registers on each expanded

memory board in the system.  The application must pass a pointer to

the source array.  This subfunction doesn't require an EMM handle.

Use this function instead of Functions 8 and 9 if you need to save or

restore the mapping context but don't want (or have) to use a handle.

CALLING PARAMETERS

AX = 4E01h

Contains the Set Page Map subfunction.

DS:SI = source_page_map

Contains a pointer to the source array address in

segment:offset format.  The application must point to an

array which contains the mapping register state.  Use the

Get Size of Page Map Save Array subfunction to determine

the size of the desired array.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has passed the array.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = A3h   NON-RECOVERABLE

The contents of the source array have been corrupted, or

the pointer passed to the subfunction is invalid.

EXAMPLE

source_page_map                    DB  ?  DUP  (?)

40

MOV  AX, SEG source_page_map

MOV  DS, AX

LEA  SI, source_page_map           ; DS:SI points to source_page_map

MOV  AX, 4E01h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

41

FUNCTION 15   GET/SET PAGE MAP (CONTINUED)

GET & SET PAGE MAP SUBFUNCTION

PURPOSE

The Get & Set subfunction simultaneously saves a current mapping

context and restores a previous mapping context for all mappable

memory regions (both conventional and expanded).  It first copies the

contents of the mapping registers from each expanded memory board in

the system into a destination array.  (The application must pass a

pointer to the destination array.)  Then, the subfunction copies the

contents of a source array into the mapping registers on each of the

expanded memory boards.  (The application must pass a pointer to the

source array.)

Use this function instead of Functions 8 and 9 if you need to save or

restore the mapping context but don't want (or have) to use a handle.

CALLING PARAMETERS

AX = 4E02h

Contains the Get & Set Page Map subfunction.

ES:DI = dest_page_map

Contains a pointer to the destination array address in

segment:offset format.  The current contents of the map

registers will be saved in this array.

DS:SI = source_page_map

Contains a pointer to the source array address in

segment:offset format.  The contents of this array will be

copied into the map registers.  The application must point

to an array which contains the mapping register state.

This address is required only for the Set or Get and Set

subfunctions.

RESULTS

These results are valid only if the status returned is zero.

dest_page_map

The array contains the mapping state.  It also contains any

additional information necessary to restore the original

state when the program invokes a Set subfunction.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has returned and passed both arrays.

42

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE the function code passed to the memory

manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = A3h   NON-RECOVERABLE

The contents of the source array have been corrupted, or

the pointer passed to the subfunction is invalid.

EXAMPLE

dest_page_map                      DB  ?  DUP  (?)

source_page_map                    DB  ?  DUP  (?)

MOV  AX, SEG dest_page_map

MOV  ES, AX

MOV  AX, SEG source_page_map

MOV  DS, AX

LEA  DI, dest_page_map             ; ES:DI points to dest_page_map

LEA  SI, source_page_map           ; DS:SI points to source_page_map

MOV  AX, 4E02h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

43

FUNCTION 15   GET/SET PAGE MAP (CONTINUED)

GET SIZE OF PAGE MAP SAVE ARRAY SUBFUNCTION

PURPOSE

The Get Size of Page Map Save Array subfunction returns the storage

requirements for the array passed by the other three subfunctions.

This subfunction doesn't require an EMM handle.

CALLING PARAMETERS

AX = 4E03h

Contains the Get Size of Page Map Save Array subfunction.

The size of this array depends on how the expanded memory

system is configured and how the expanded memory manager is

implemented.  Therefore, the size must be determined after

the memory manager is loaded.

RESULTS

These results are valid only if the status returned is zero.

AL = size_of_array

Contains the number of bytes that will be transferred to

the memory area an application supplies whenever a program

requests the Get, Set, or Get and Set subfunctions.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the array size.

AH = 80h   NON-Recoverable the manager detected a malfunction in the

memory manager software.

AH = 81h   NON-Recoverable the manager detected a malfunction in the

expanded memory hardware.

AH = 84h   NON-Recoverable the function code passed to the memory

manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

size_of_array                      DB  ?

MOV  AX, 4E03h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

44

MOV  size_of_array, AL             ; save array size

45

FUNCTION 16   GET/SET PARTIAL PAGE MAP

GET PARTIAL PAGE MAP SUBFUNCTION

PURPOSE

The Get Partial Page Map subfunction saves a partial mapping context

for specific mappable memory regions in a system.  Because this

function saves only a subset of the entire mapping context, it uses

much less memory for the save area and may be potentially faster than

Function 15.  The subfunction does this by copying the contents of

selected mapping registers from each expanded memory board to a

destination array.

The application must pass a pair of pointers.  The first points to a

structure which specifies which mappable segments to save; the second

points to the destination array.

Use this function instead of Functions 8 and 9 if you need to save or

restore the mapping context but don't want (or have) to use a handle.

CALLING PARAMETERS

AX = 4F00h

Contains the Get Partial Page Map subfunction.

partial_page_map_struct     STRUC

mappable_segment_count   DW  ?

mappable_segment         DW  (?)  DUP  (?)

partial_page_map_struct     ENDS

DS:SI = partial_page_map

Contains a pointer to a structure which specifies only

those mappable memory regions which are to have their

mapping context saved.  The structure members are described

below.

.mappable_segment_count

The first member is a word which specifies the number of

members in the word array which immediately follows it.

This number should not exceed the number of mappable

segments in the system.

.mappable_segment

The second member is a word array which contains the

segment addresses of the mappable memory regions whose

mapping contexts are to be saved.  The segment address must

be a mappable segment.  Use Function 25 to determine which

segments are mappable.

ES:DI = dest_array

Contains a pointer to the destination array address in

Segment:Offset format.  To determine the size of the

required array, see the Get Size of Partial Page Map Save

Array subfunction.

RESULTS

46

These results are valid only if the status returned is zero.

dest_array

The array contains the partial mapping context and any

additional information necessary to restore this context to

its original state when the program invokes a Set

subfunction.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has saved the partial map context.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Bh   NON-RECOVERABLE

One of the specified segments is not a mappable segment.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = A3h   NON-RECOVERABLE

The contents of the partial page map structure have been

corrupted, the pointer passed to the subfunction is

invalid, or the mappable_segment_count exceeds the number

of mappable segments in the system.

EXAMPLE

partial_page_map                   partial_page_map_struct <>

dest_array                         DB ? DUP (?)

MOV  AX, SEG partial_page_map

MOV  DS, AX

LEA  SI, partial_page_map          ; DS:SI points to

MOV  AX, SEG dest_array            ; partial_page_map

MOV  ES, AX

LEA  DI, dest_array                ; ES:DI points to dest_array

MOV  AX, 4F00h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

47

FUNCTION 16   GET/SET PARTIAL PAGE MAP (CONTINUED)

SET PARTIAL PAGE MAP SUBFUNCTION

PURPOSE

The Set Partial Page Map subfunction provides a mechanism for

restoring the mapping context for a partial mapping context for

specific mappable memory regions in a system.  Because this function

restores only a subset of the entire mapping context and not the

entire systems mapping context, it uses much less memory for the save

area and is potentially faster than Function 15.  The subfunction does

this by copying the contents of the source array to selected mapping

registers on each expanded memory board.  The application passes a

pointer to the source array.  Use this function instead of Functions 8

and 9 if you need to save or restore the mapping context but don't

want (or have) to use a handle.

CALLING PARAMETERS

AX = 4F01h

Contains the Set Partial Page Map subfunction.

source_array DB  ?  DUP  (?)

DS:SI = source_array

Contains a pointer to the source array in segment:offset

format.  The application must point to an array which

contains the partial mapping register state.  To determine

the size of the required array, see the Get Size of Partial

Page Map Save Array subfunction.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has restored the partial mapping context.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

48

AH = A3h   NON-RECOVERABLE

The contents of the source array have been corrupted, or

the pointer passed to the subfunction is invalid.

EXAMPLE

MOV  AX, SEG source_array

MOV  DS, AX

LEA  SI, source_array              ; DS:SI points to source_array

MOV  AX, 4F01h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

49

FUNCTION 16   GET/SET PARTIAL PAGE MAP (CONTINUED)

GET SIZE OF PARTIAL PAGE MAP SAVE ARRAY SUBFUNCTION

PURPOSE

The Return Size subfunction returns the storage requirements for the

array passed by the other two subfunctions.  This subfunction doesn't

require an EMM handle.

CALLING PARAMETERS

AX = 4F02h

Contains the Get Size of Partial Page Map Save Array

subfunction.  The size of this array depends on the

expanded memory system configuration and the implementation

of the expanded memory manager.  Therefore, it will vary

between hardware configurations and implementations and

must be determined after a specific memory manager is

loaded.

BX = number of pages in the partial array

Contains the number of pages in the partial map to be saved

by the Get/Set Partial Page Map subfunctions.  This number

should be the same as the mappable_segment_count in the Get

Partial Page Map subfunction.

Results

These results are valid only if the status returned is zero.

AL = size_of_partial_save_array

Contains the number of bytes that will be transferred to

the memory area supplied by an application whenever a

program requests the Get or Set subfunction.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the array size.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

50

AH = 8Bh   NON-RECOVERABLE

The number of pages in the partial array is outside the

range of physical pages in the system.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

number_of_pages_to_map             DW  ?

size_of_partial_save_array         DB  ?

MOV  BX, number_of_pages_to_map

MOV  AX, 4F02h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  size_of_partial_save_array, AL; save array size

51

FUNCTION 17   MAP/UNAMP MULTIPLE HANDLE PAGES

PURPOSE

This function can, in a single invocation, map (or unmap) logical

pages into as many physical pages as the system supports.

Consequently, it has less execution overhead than mapping pages one at

a time.  For applications which do a lot of page mapping, this is the

preferred mapping method.

MAPPING MULTIPLE PAGES

The handle passed to this function determines what type of logical

pages are being mapped.  Logical pages that Function 4 and Function 27

(Allocate Standard Pages subfunction) allocate are referred to as

pages and are 16K bytes.  Logical pages that Function 27 (Allocate Raw

Pages subfunction) allocates are referred to as raw pages and might

not be the same size as the pages Function 4 and Function 27 (Allocate

Standard Pages subfunction) allocate.

UNMAPPING MULTIPLE PAGES

This function can make specific physical pages unavailable for reading

or writing.  A logical page which is unmapped from a specific physical

page cannot be read or written from that physical page.  The logical

page which is unavailable (unmapped) can be made available again by

mapping it, or a new logical page, at the physical page that was

unmapped. Unmapping a physical page is accomplished by setting the

logical page it is associated with to FFFFh.

You might unmap an entire set of mapped pages, for example, before

loading and executing a program.  This ensures that the loaded program

won't be able to access the pages your program has mapped.  However,

you must save the mapping context before you unmap the physical pages.

This enables you to restore it later so that you may access the memory

you had mapped there.  You can save the mapping context with Functions

8, 15, or 16.  You can restore the mapping context with Functions 9,

15, or 16.

MAPPING AND UNMAPPING MULTIPLE PAGES SIMULTANEOUSLY

Both mapping and unmapping pages can be done in the same invocation.

Mapping or unmapping no pages is not considered an error.  If a

request to map or unmap zero pages is made, nothing is done and no

error is returned.

ALTERNATE MAPPING AND UNMAPPING METHODS

You can map or unmap pages using two methods.  Both methods produce

identical results.

1   The first method specifies both a logical page and a physical page

at which the logical page is to be mapped.  This method is an

extension of Function 5 (Map Handle Page).

52

2   The second method specifies both a logical page and a

corresponding segment address at which the logical page is to be

mapped.  While this is functionally the same as the first method,

it may be easier to use the actual segment address of a physical

page than to use a number which only represents its location.  The

memory manager verifies whether the specified segment address

falls on the boundary of a mappable physical page.  The manager

then translates the segment address passed to it into the

necessary internal representation to map the pages.

MAP/UNMAP MULTIPLE HANDLE PAGES

LOGICAL PAGE/PHYSICAL PAGE METHOD

CALLING PARAMETERS

AX = 5000h

Contains the Map/Unmap Multiple Handle Pages subfunction

using the logical page/physical page method.

log_to_phys_map_struct   STRUC

log_page_number       DW  ?

phys_page_number      DW  ?

log_to_phys_map_struct   ENDS

DX = handle

Contains the EMM handle.

CX = log_to_phys_map_len

Contains the number of entries in the array.  For example,

if the array contained four pages to map or unmap, then CX

would contain 4.  The number in CX should not exceed the

number of mappable pages in the system.

DS:SI = pointer to log_to_phys_map array

Contains a pointer to an array of structures that contains

the information necessary to map the desired pages.  The

array is made up of the following two elements:

.log_page_number

The first member is a word which contains the number of the

logical page which is to be mapped.  Logical pages are

numbered zero relative, so the number for a logical page

can only range from zero to (maximum number of logical

pages allocated to the handle - 1).

If the logical page number is set to FFFFh, the physical

page associated with it is unmapped rather than mapped.

Unmapping a physical page makes it inaccessible for reading

or writing.

.phys_page_number

The second member is a word which contains the number of

the physical page at which the logical page is to be

mapped.  Physical pages are numbered zero relative, so the

number for a physical page can only range from zero to

53

(maximum number of physical pages supported in the system -

1).

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The logical pages have been mapped, or unmapped, at the

specified physical pages.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE

The manager couldn't find the specified EMM handle.  The

manager doesn't currently have any information pertaining

to the specified EMM handle.  The program has probably

corrupted its EMM handle.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Ah   RECOVERABLE

One or more of the mapped logical pages is out of the range

of logical pages allocated to the EMM handle.  The program

can recover by attempting to map a logical page which is

within the bounds for the specified EMM handle.  When this

error occurs, the only pages mapped were the ones valid up

to the point that the error occurred.

AH = 8Bh   RECOVERABLE

One or more of the physical pages is out of the range of

mappable physical pages, or the log_to_phys_map_len exceeds

the number of mappable pages in the system.  The program

can recover from this condition by attempting to map into

memory at a physical page which is in the range of the

physical page numbers supported by the system.  When this

error occurs, the only pages mapped were the ones valid up

to the point that the error occurred.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

log_to_phys_map                    log_to_phys_map_struct ? DUP (?)

emm_handle                         DW ?

MOV  AX, SEG log_to_phys_map       ; DS:SI points to

54

MOV  DS, AX                        ; log_to_phys_map

LEA  SI, log_to_phys_map

MOV  CX, LENGTH log_to_phys_map    ; set length field

MOV  DX, emm_handle                ; load handle

MOV  AX, 5000h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

55

MAP/UNMAP MULTIPLE HANDLE PAGES

LOGICAL PAGE/SEGMENT ADDRESS METHOD

CALLING PARAMETERS

AX = 5001h

Contains the Map/Unmap Multiple Handle Pages subfunction

using the logical page/segment address method.

log_to_seg_map_struct         STRUC

log_page_number            DW  ?

mappable_segment_address   DW  ?

log_to_seg_map_struct         ENDS

DX = handle

Contains the EMM handle.

CX = log_to_segment_map_len

Contains the number of entries in the array.  For example,

if the array contained four pages to map or unmap, then CX

would contain four.

DS:SI = pointer to log_to_segment_map array

Contains a pointer to an array of structures that contains

the information necessary to map the desired pages.  The

array is made up of the following elements:

.log_page_number

The first member is a word which contains the number of the

logical pages to be mapped.  Logical pages are numbered

zero relative, so the number for a logical page can range

from zero to (maximum number of logical pages allocated to

the handle - 1).

If the logical page number is set to FFFFh, the physical

page associated with it is unmapped rather than mapped.

Unmapping a physical page makes it inaccessible for reading

or writing.

.mappable_segment_address

The second member is a word which contains the segment

address at which the logical page is to be mapped.  This

segment address must correspond exactly to a mappable

segment address.  The mappable segment addresses are

available with Function 25 (Get Mappable Physical Address

Array)

REGISTERS MODIFIED

AX

STATUS

56

AH = 0     SUCCESSFUL

The logical pages have been mapped (or unmapped), at the

specified physical pages.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE

The manager could not find the specified EMM handle.  The

manager doesn't have any information pertaining to the

specified EMM handle.  The program has probably corrupted

its EMM handle.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Ah   RECOVERABLE

One or more of the logical pages to be mapped is out of the

range of logical pages allocated to the EMM handle.  The

program can recover from this condition by mapping a

logical page which is within the bounds for the specified

EMM handle.  When this error occurs, the only pages mapped

or unmapped were the ones valid up to the point that the

error occurred.

AH = 8Bh  RECOVERABLE

One or more of the mappable segment addresses specified is

not mappable, the segment address doesn't fall exactly on a

mappable address boundary, or the log_to_segment_map_len

exceeds the number of mappable segments in the system. The

program can recover from this condition by mapping into

memory on an exact mappable segment address.  When this

error occurs, the only pages mapped were the ones valid up

to the point that the error occurred.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

log_to_seg_map                     log_to_seg_map_struct 4 DUP (?)

emm_handle                         DW ?

MOV  AX, SEG log_to_seg_map

MOV  DS, AX

LEA  SI, log_to_seg_map            ; DS:SI points to log_to_seg_map

MOV  CX, LENGTH log_to_seg_map     ; load # of segments to

; map/unmap

MOV  DX, emm_handle                ; load handle

MOV  AX, 5001h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

57

JNZ  emm_err_handler               ; jump to error handler on error

58

FUNCTION 18   REALLOCATE PAGES

PURPOSE

This function allows an application program to increase or decrease

(reallocate) the number of logical pages allocated to an EMM handle.

There are four reallocation cases of interest:

1   A reallocation count of zero.  The handle assigned to application

remains assigned and is still available for use by the

application.  The memory manager won't reassign the handle to any

other application.  However, the handle will have any currently

allocated pages returned to the memory manager.  The application

must invoke the Deallocate Pages function (Function 6) before

returning to DOS, or the handle will remain assigned and no other

application will be able to use it.

2   A reallocation count equal to the current allocation count.  This

is not treated as an error, and a successful status is returned.

3   A reallocation count greater than the current allocation count.

The memory manager will attempt to add new pages to those pages

already allocated to the specified EMM handle.  The number of new

pages added is the difference between the reallocation count and

the current allocation count.  The sequence of logical pages

allocated to the EMM handle remains continuous after this

operation.  The newly allocated pages have logical page numbers

which begin where the previously allocated pages ended, and

continue in ascending sequence.

4   A reallocation count less than the current allocation count.  The

memory manager will attempt to subtract some of the currently

allocated pages and return them to the memory manager.  The number

of old pages subtracted is the difference between the current

allocation count and the re-allocation count.  The pages are

subtracted from the end of the sequence of pages currently

allocated to the specified EMM handle.  The sequence of logical

pages allocated to the EMM handle remains continuous after this

operation.

The handle determines what type of logical pages are being

reallocated.  Logical pages which were originally allocated with

Function 4 or Function 27 (Allocate Standard Pages subfunction) are

called pages and are 16K bytes long.  Logical pages which were

allocated with Function 27 (Allocate Raw Pages subfunction) are called

raw pages and might not be the same size as pages allocated with

Function 4 or Function 27 (Allocate Standard Pages subfunction).

CALLING PARAMETERS

AH = 51h

Contains the Reallocate Handle Pages function.

DX = handle

Contains the EMM handle.

59

BX = reallocation_count

Contains the total number of pages this handle should have

allocated to it after this function is invoked.

RESULTS

These results are valid only if the status returned is zero.

BX = number of pages allocated to handle after reallocation

Contains the number of pages now allocated to the EMM

handle after the pages have been added or subtracted.  If

the status returned is not zero, the value in BX is equal

to the number of pages allocated to the handle prior to the

invocation of this function.  This information can be used

to verify that the request generated the expected results.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 00h   SUCCESSFUL

The pages specified have been added or subtracted to or

from the handle specified.

AH = 80h   NON-Recoverable the manager has detected a malfunction in

the memory manager software.

AH = 81h   NON-Recoverable the manager has detected a malfunction in

the expanded memory hardware.

AH = 83h   NON-Recoverable the manager could not find the specified

EMM handle.  The manager doesn't have any information

pertaining to the specified EMM handle.  The program may

have corrupted its EMM handle.

AH = 84h   NON-Recoverable the function code passed to the manager is

not defined.

AH = 87h   Recoverable the number of pages that are available in the

system is insufficient for the new allocation request.  The

program can recover from this condition by specifying fewer

pages be allocated to the EMM handle.

AH = 88h   Recoverable the number of unallocated pages is insufficient

for the new allocation request.  The program can recover

from this condition by either requesting again when

additional pages are available or specifying fewer pages.

EXAMPLE

emm_handle                         DW   ?

realloc_count                      DW   ?

current_alloc_page_count           DW   ?

MOV  DX, emm_handle                ; specify EMM handle

60

MOV  BX, realloc_count             ; specify count

MOV  AH, 51h                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  current_alloc_page_count, BX

61

FUNCTION 19   GET/SET HANDLE ATTRIBUTE

DESIGN CONSIDERATIONS

This function is an option which will probably not be available in a

typical expanded memory manager, system, or memory board.  Most

personal computer systems disable memory refresh signals for a

considerable period during a warm boot.  This can corrupt some of the

data in memory boards, even though there is no problem with the design

of the memory board, its operation, or the memory chips.  This memory

refresh deficiency is present in the software design of the ROM BIOS

in most personal computer systems.

The majority of memory board designs, chip types, or personal computer

systems won't be able to support the non-volatility feature.  The

reason that this ROM BIOS deficiency is not evident in the

conventional or extended memory area is that the ROM BIOS always

initializes this area during a warm boot.  Memory data integrity is

not a problem with the conventional or extended memory region, because

it isn't physically possible to have data retained there across a warm

boot event -- the ROM BIOS sets it to zero.

Consequently, expanded memory board manufacturers should not supply

this function unless their board can guarantee the integrity of data

stored in the board's memory during a warm boot.  Generally, this

means the memory board has an independent memory refresh controller

which does not depend on the system board's memory refresh.

If the expanded memory manager, system, or memory board cannot support

this feature, it should return the not supported status described in

the function.

62

FUNCTION 19   GET/SET HANDLE ATTRIBUTE (CONTINUED)

GET HANDLE ATTRIBUTE SUBFUNCTION

PURPOSE

This subfunction returns the attribute associated with a handle.  The

attributes are volatile or non-volatile.  Handles with non-volatile

attributes enable the memory manager to save the contents of a

handle's pages between warm boots.  However, this function may be

disabled with a user option or may not be supported by the memory

board or system hardware.

If the handle's attribute has been set to non-volatile, the handle,

its name (if it is assigned one), and the contents of the pages

allocated to the handle are all maintained after a warm boot.

CALLING PARAMETERS

AX = 5200h

Contains the Get Handle Attribute subfunction.

DX = handle

Contains the EMM handle.

RESULTS

These results are valid only if the status returned is zero.

AL = handle attribute

Contains the EMM handle's attribute.  The only attributes a

handle may have are volatile or non-volatile.  A value of

zero indicates the handle is volatile.  A value of one

indicates that the handle is non-volatile.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The handles attribute has been obtained.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE

The manager couldn't find the specified EMM handle.  The

manager doesn't have any information pertaining to the

specified EMM handle.  The program may have corrupted its

EMM handle.

63

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 91h   NON-RECOVERABLE

This feature is not supported.

EXAMPLE

emm_handle                         DW   ?

handle_attrib                      DB   ?

MOV  DX, emm_handle                ; specify EMM handle

MOV  AX, 5200h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  handle_attrib, AL             ; save handle attribute

64

FUNCTION 19   GET/SET HANDLE ATTRIBUTE (CONTINUED)

SET HANDLE ATTRIBUTE SUBFUNCTION

PURPOSE

This subfunction can be used to modify the attribute which a handle

has associated with it.  The attributes which a handle may have are

volatile or non-volatile.  The non-volatile attribute enables the EMM

to save the contents of a handle's pages between warm boots.  However,

this function may be disabled with a user option or may not be

supported by the memory board or system hardware.

If the handle's attribute has been set to non-volatile, the handle,

its name (if it is assigned one), and the contents of the pages

allocated to the handle are all maintained after a warm boot.

CALLING PARAMETERS

AX = 5201h

Contains the Set Handle Attribute function.

DX = handle

Contains the EMM handle.

BL = new handle attribute

Contains the handle's new attribute.  A value of zero

indicates that the handle should be made volatile.  A value

of one indicates that the handle should be made non-

volatile.

A volatile handle attribute instructs the memory manager to

deallocate both the handle and the pages allocated to it

after a warm boot.  If all handles have the volatile

attribute (the default attribute) at warm boot, the handle

directory will be empty and all of expanded memory will be

initialized to zero immediately after a warm boot.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The handles attribute has been modified.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE

The manager could not find the specified EMM handle.  The

65

manager doesn't have any information pertaining to the

specified EMM handle.  The program may have corrupted its

EMM handle.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 90h   NON-RECOVERABLE

The attribute type is undefined.

AH = 91h   NON-RECOVERABLE

This feature is not supported.

EXAMPLE

emm_handle                         DW   ?

new_handle_attrib                  DB   ?

MOV  DX, emm_handle                ; specify EMM handle

MOV  BL, new_handle_attrib         ; specify the set attribute

MOV  AX, 5201h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

66

FUNCTION 19   GET/SET HANDLE ATTRIBUTE (CONTINUED)

GET ATTRIBUTE CAPABILITY SUBFUNCTION

PURPOSE

This subfunction can be used to determine whether the memory manager

can support the non-volatile attribute.

CALLING PARAMETERS

AX = 5202h

Contains the Get Attribute Capability subfunction.

RESULTS

These results are valid only if the status returned is zero.

AL = attribute capability

Contains the attribute capability.  A value of zero

indicates that the memory manager and hardware supports

only volatile handles.  A value of one indicates that the

memory manager/hardware supports both volatile and non-

volatile handles.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The attribute capability has been returned.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

attrib_capability                  DB  ?

MOV  AX, 5202h                     ; load function code

INT  67h                           ;  call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  attrib_capability, AL         ; save attribute capability

67

FUNCTION 20   GET/SET HANDLE NAME

GET HANDLE NAME SUBFUNCTION

PURPOSE

This subfunction gets the eight character name currently assigned to a

handle.  There is no restriction on the characters which may be used

in the handle name (that is, anything from 00h through FFh).

The handle name is initialized to ASCII nulls (binary zeros) three

times: when the memory manager is installed, when a handle is

allocated, and when a handle is deallocated.  A handle with a name

which is all ASCII nulls, by definition, has no name.  When a handle

is assigned a name, at least one character in the name must be a non-

null character in order to distinguish it from a handle without a

name.

CALLING PARAMETERS

AX = 5300h

Contains the Get Handle Name function.

DX = handle number

Contains the EMM handle.

ES:DI = pointer to handle_name array

Contains a pointer to an eight-byte array into which the

name currently assigned to the handle will be copied.

RESULTS

These results are valid only if the status returned is zero.

handle_name array

Contains the name associated with the specified handle.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The handle name has been returned.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE

The manager couldn't find the specified EMM handle.  The

68

manager doesn't have any information on the specified EMM

handle.  The program may have corrupted its EMM handle.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

handle_name                        DB 8 DUP (?)

emm_handle                         DW   ?

MOV  AX, SEG handle_name

MOV  ES, AX

LEA  DI, handle_name               ; ES:DI points to handle_name

MOV  DX, emm_handle                ; specify EMM handle

MOV  AX, 5300h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

69

FUNCTION 20   GET/SET HANDLE NAME

SET HANDLE NAME SUBFUNCTION

PURPOSE

This subfunction assigns an eight character name to a handle.  There

is no restriction on the characters which may be used in the handle

name.  The full range of values may be assigned to each character in a

name (that is, 00h through FFh).

At installation, all handles have their name initialized to ASCII

nulls (binary zeros).  A handle with a name consisting of all ASCII

nulls has no name.  When a handle is assigned a name, at least one

character in the name must be a non-null character in order to

distinguish it from a handle without a name.  No two handles may have

the same name.

A handle can be renamed at any time by setting the handles name to a

new value.  A handle can have its name removed by setting the handle's

name to all ASCII nulls.  When a handle is deallocated, its name is

removed (set to ASCII nulls).

CALLING PARAMETERS

AX = 5301h

Contains the Set Handle Name function.

DX = handle number

Contains the EMM handle.

DS:SI = pointer to handle_name

Contains a pointer to a byte array which contains the name

that is to be assigned to the handle.  The handle name must

be padded with nulls if the name is less than eight

characters long.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The handle name has been assigned.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE

The manager couldn't find the specified EMM handle.  The

manager doesn't currently have any information pertaining

70

to the specified EMM handle.  The program may have

corrupted its EMM handle.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = A1h   RECOVERABLE

A handle with this name already exists.  The specified

handle was not assigned a name.

EXAMPLE

handle_name                        DB "AARDVARK"

emm_handle                         DW   ?

MOV  AX, SEG handle_name

MOV  DS, AX

LEA  SI, handle_name               ; DS:SI points to handle_name

MOV  DX, emm_handle                ; specify EMM handle

MOV  AX, 5301h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

71

FUNCTION 21   GET HANDLE DIRECTORY

GET HANDLE DIRECTORY SUBFUNCTION

PURPOSE

This function returns an array which contains all active handles and

the names associated with each.  Handles which have not been assigned

names have a default name of all ASCII nulls (binary zeros).  When a

handle is first allocated, or when all the pages belonging to a handle

are deallocated (that is, an open handle is closed), its default name

is set to ASCII nulls.  It takes a subsequent assignment of a name for

a handle to have a name after it has been opened.  The full range of

values may be assigned to each character in a name (that is, 00h

through FFh).

The number of bytes required by the array is:

10 bytes * total number of handles

The maximum size of this array is:

(10 bytes/entry) * 255 entries = 2550 bytes.

CALLING PARAMETERS

AX = 5400h

Contains the Get Handle Directory function.

handle_dir_struct   STRUC

handle_value     DW  ?

handle_name      DB  8  DUP  (?)

handle_dir_struct   ENDS

ES:DI = pointer to handle_dir

Contains a pointer to an area of memory into which the

memory manager will copy the handle directory.  The handle

directory is an array of structures.  There are as many

entries in the array as there are open EMM handles.  The

structure consists of the following elements:

.handle_value

The first member is a word which contains the value of the

open EMM handle.

.handle_name

The second member is an 8 byte array which contains the

ASCII name associated with the EMM handle.  If there is no

name currently associated with the handle, it has a value

of all zeros (ASCII nulls).

RESULTS

These results are valid only if the status returned is zero.

72

handle_dir

Contains the handle values and handle names associated with

each handle value.

AL = number of entries in the handle_dir array

Contains the number of entries in the handle directory

array.  This is also the same as the number of open

handles.  For example, if only one handle is active, AL

will contain a one.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The handle directory has been returned.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

handle_dir                         handle_dir_struct 255 DUP (?)

num_entries_in_handle_dir          DB ?

MOV  AX, SEG handle_dir

MOV  ES, AX

LEA  DI, handle_dir                ; ES:DI points to handle_dir

MOV  AX, 5400h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  num_entries_in_handle_dir, AL ; save number of entries

73

FUNCTION 21   GET HANDLE DIRECTORY

SEARCH FOR NAMED HANDLE SUBFUNCTION

PURPOSE

This subfunction searches the handle name directory for a handle with

a particular name.  If the named handle is found, this subfunction

returns the handle number associated with the name.  At the time of

installation, all handles have their names initialized to ASCII nulls.

A handle with a name which is all ASCII nulls has, by definition, no

name.  When a handle is assigned a name, at least one character in the

name must be a non-null character in order to distinguish it from a

handle without a name.

CALLING PARAMETERS

AX = 5401h

Contains the Search for Named Handle subfunction.

DS:SI = handle_name

Contains a pointer to an 8-byte string that contains the

name of the handle be searched for.

RESULTS

These results are valid only if the status returned is zero.

DX = value of named handle

The value of the handle which matches the handle name

specified.

REGISTERS MODIFIED

AX, DX

STATUS

AH = 0     SUCCESSFUL

The handle value for the named handle has been found.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

74

AH = A0h   RECOVERABLE

No corresponding handle could be found for the handle name

specified.

AH = A1h   NON-RECOVERABLE

A handle found had no name (all ASCII nulls).

EXAMPLE

named_handle                       DB  'AARDVARK'

named_handle_value                 DW  ?

MOV  AX, SEG named_handle

MOV  DS, AX

LEA  SI, named_handle              ; DS:SI points to named_handle

MOV  AX, 5401h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  named_handle_value, DX        ; save value of named handle

75

FUNCTION 21   GET HANDLE DIRECTORY

GET TOTAL HANDLES SUBFUNCTION

PURPOSE

This subfunction returns the total number of handles that the memory

manager supports, including the operating system handle (handle value

0).

CALLING PARAMETERS

AX = 5402h

Contains the Get Total Handles subfunction.

RESULTS

These results are valid only if the status returned is zero.

BX = total_handles

The value returned represents the maximum number of handles

which a program may request the memory manager to allocate

memory to.  The value returned includes the operating

system handle (handle value 0).

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0     SUCCESSFUL

The total number of handles supported has been returned.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

total_handles                      DW     ?

MOV  AX, 5402h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  total_handles, BX             ; save total handle count

76

FUNCTION 22   ALTER PAGE MAP & JUMP

PURPOSE

This function alters the memory mapping context and transfers control

to the specified address.  It is analogous to the FAR JUMP in the 8086

family architecture.  The memory mapping context which existed before

the invocation of this function is lost.

Mapping no pages and jumping is not considered an error.  If a request

to map zero pages and jump is made, control is transferred to the

target address, and this function performs a far jump.

CALLING PARAMETERS

AH = 55h

Contains the Alter Page Map & Jump function.

log_phys_map_struct       STRUC

log_page_number        DW ?

phys_page_number_seg   DW ?

log_phys_map_struct       ENDS

map_and_jump_struct       STRUC

target_address            DD ?

log_phys_map_len          DB ?

log_phys_map_ptr          DD ?

map_and_jump_struct       ENDS

AL = physical page number/segment selector

Contains a code which indicates whether the value contained

in the .log_phys_map.phys_page_number_seg are physical page

numbers or are the segment address representation of the

physical page numbers.  A zero in AL indicates that the

values are physical page numbers.  A one in AL indicates

that the values in these members are the segment address

representations of the physical page numbers.

DX = handle number

Contains the EMM handle.

DS:SI = pointer to map_and_jump structure

Contains a pointer to a structure that contains the

information necessary to map the desired physical pages and

jump to the target address.  The structure consists of the

following elements:

.target_address

The first member is a far pointer which contains the target

address to which control is to be transferred.  The address

is represented in segment:offset format.  The offset

portion of the address is stored in the low portion of the

double word.

.log_phys_map_len

The second member is a byte which contains the number of

entries in the array of structures which immediately

77

follows it.  The array is as long as the application

developer needs to map the desired logical pages into

physical pages.  The number of entries cannot exceed the

number of mappable pages in the system.

.log_phys_map_ptr

The third member is a pointer to an array of structures

which contain the logical page numbers and  Alternate DMA

register sets physical page numbers or segment addresses at

which they are to be mapped.  Each entry in the array of

structures contains the following two elements:

.log_page_number

The first member of this structure is a word which contains

the number of the logical page to be mapped.

.phys_page_number_seg

The second member of this structure is a word which

contains either the physical page number or the segment

address representation of the physical page number at which

the previous logical page number is to be mapped.  The

value passed in AL determines the type of representation.

REGISTERS MODIFIED

AX

Note...............................................................

Values in registers which don't contain required parameters maintain

the values across the jump.  The values in registers (with the

exception of AX) and the flag state at the beginning of the function

are still in the registers and flags when the target address is

reached.

STATUS

AH = 0     SUCCESSFUL

Control has been transferred to the target address.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE

The manager could not find the specified EMM handle.  The

manager does not currently have any information pertaining

to the specified EMM handle.  The program may have

corrupted its EMM handle.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

78

AH = 8Ah   RECOVERABLE

One or more of the logical pages to map into a

corresponding physical page is out of the range of logical

pages which are allocated to the EMM handle.  The program

can recover from this condition by mapping a logical page

which is within the bounds for the EMM handle.

AH = 8Bh   RECOVERABLE

One or more of the physical pages is out of the range of

allowable physical pages, or the log_phys_map_len exceeds

the number of mappable pages in the system.  Physical page

numbers are numbered zero relative.  The program can

recover from this condition by mapping into memory at a

physical page which is in the range of supported physical

pages.

AH = 8Fh   RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

log_phys_map                       log_phys_map_struct (?) DUP (?)

map_and_jump                       map_and_jump_struct (?)

emm_handle                         DW ?

phys_page_or_seg_mode              DB ?

MOV  AX, SEG map_and_jump

MOV  DS, AX

LEA  SI, map_and_jump              ; DS:SI points to map_and_jump

MOV  DX, emm_handle

MOV  AH, 55h                       ; load function code

MOV  AL, phys_page_or_seg_mode     ; specify physical page or seg

; mode

INT  67h                           ; call memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

79

FUNCTION 23   ALTER PAGE MAP & CALL

ALTER PAGE MAP & CALL SUBFUNCTION

PURPOSE

This subfunction saves the current memory mapping context, alters the

specified memory mapping context, and transfers control to the

specified address.  It is analogous to the FAR CALL in the 8086 family

architecture.  Just as a return from a FAR CALL restores the original

value in the code segment register, this subfunction restores the

state of the specified mapping context after the return.

There is no explicit expanded memory subfunction which emulates a

return from a FAR CALL.  However, this facility is implicitly

available through the standard return from a FAR CALL.  The following

paragraphs describe how this works:

After this function is invoked, unless an error is detected, the

memory manager will transfer control to the address specified.  If an

error occurs, the memory manager returns immediately with the error

code in the AH register.  Otherwise, the memory manager pushes on the

stack information which enables it to restore the mapping context

after the return.

When the called procedure wants to return to the calling procedure, it

simply issues a standard FAR RETURN.  The memory manager traps this

return, restores the specified mapping context, and returns to the

calling procedure.  The memory manager also returns a status from a

successful return just as it does for all functions.

Developers using this subfunction must make allowances for the

additional stack space this subfunction will use.

CALLING PARAMETERS

AH = 56h

Contains the Alter Page Map & Call function.

log_phys_map_struct       STRUC

log_page_number        DW ?

phys_page_number_seg   DW ?

log_phys_map_struct       ENDS

map_and_call_struct       STRUC

target_address         DD ?

new_page_map_len       DB ?

new_page_map_ptr       DD ?

old_page_map_len       DB ?

old_page_map_ptr       DD ?

reserved               DW  4 DUP (?)

map_and_call_struct       ENDS

AL = physical page number/segment selector

Contains a code which indicates whether the value contained

in the

80

.new_page_map.phys_page_number_seg/.old_page_map.phys_page_number_seg

members are physical pages numbers or are the segment

address representation of the physical page numbers.  A

value of zero in AL indicates that the values in these

members are physical page numbers.  A value of one in AL

indicates that the values in these members are the segment

address representations of the physical page numbers.

DX = handle number

Contains the EMM handle.

DS:SI = pointer to map_and_call structure

Contains a pointer to a structure which contains the

information necessary to map the desired physical pages and

call the target address.  The structure members are

described here:

.target_address

The first member is a far pointer which contains the target

address to which control is to be transferred.  The address

is represented in segment:offset format.  The offset

portion of the address is stored in the low portion of the

pointer.  The application must supply this value.

.new_page_map_len

The second member is a byte which contains the number of

entries in the new mapping context to which

new_page_map_ptr points.  This number cannot exceed the

number of mappable pages in the system.

.new_page_map_ptr

The third member is a far pointer that points to an array

of structures which contains a list of the logical page

numbers and the physical page numbers/segments at which

they are to be mapped immediately after the call.  The

contents of the new array of structures are described at

the end of the map_and_call structure.

.old_page_map_len

The fourth member is a byte which contains the number of

entries in the old mapping context to which

old_page_map_ptr points.  This number cannot exceed the

number of mappable pages in the system.

.old_page_map_ptr

The fifth member is a far pointer that points to an array

of structures which contains a list of the logical page

numbers and the physical page numbers/segments at which

they are to be mapped immediately after the return.  The

contents of the old array of structures are described at

the end of the map_and_call structure.

.reserved

The sixth member is reserved for use by the memory manager.

Each entry in the old and new array of structures contains two

elements:

81

.log_page_number

The first member of this structure is a word which contains

a logical page number which is to be mapped at the

succeeding physical page number/segment immediately after

the CALL (in the case of the new array of structures) or

after the RETURN (in the case of the old array of

structures).

.phys_page_number_seg

The second member of this structure is a word which

contains either the physical page number or the segment

address representation of the physical page number/segment

at which the preceding logical page is to be mapped

immediately after the CALL (in the case of the new array of

structures) or after the RETURN (in the case of the old

array of structures).

REGISTERS MODIFIED

AX

Note..............................................................

Values in registers which don't contain required parameters maintain

the values across the call.  The values in registers (with the

exception of AX) and the flag state at the beginning of the function

are still in the registers and flags when the target address is

reached.

STATUS

AH = 0     SUCCESSFUL

Control has been transferred to the target address.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE

The manager couldn't find the specified EMM handle.  The

manager doesn't have any information pertaining to the

specified EMM handle.  The program may have corrupted its

EMM handle.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Ah   RECOVERABLE

One or more of the logical pages to map into a

corresponding physical page is out of the range of logical

pages which are allocated to the EMM handle.  The program

can recover from this condition by mapping a logical page

which is within the bounds for the EMM handle.

82

AH = 8Bh   RECOVERABLE

One or more of the physical pages is out of the range of

allowable physical pages, or you've specified more physical

pages than exist in the system.  Physical page numbers are

numbered zero relative.  The program can recover from this

condition by mapping a physical page which is in the range

from zero to three.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

new_page_map             log_phys_map_struct (?) DUP (?)

old_page_map             log_phys_map_struct (?) DUP (?)

map_and_call             map_and_call_struct (?)

emm_handle               DW ?

phys_page_or_seg_mode    DB ?

MOV  AX, SEG map_and_call

MOV  DS, AX

LEA  SI, map_and_call              ; DS:SI points to map_and_call

MOV  DX, emm_handle                ; specify EMM handle

MOV  AH, 56h                       ; load function code

MOV  AL, phys_page_or_seg_mode     ; specify physical page or seg

; mode

INT  67h                           ; control is actually

; transferred to the called

; procedure at this point

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

83

FUNCTION 23   ALTER PAGE MAP & CALL

GET PAGE MAP STACK SPACE SIZE SUBFUNCTION

PURPOSE

Since the Alter Page Map & Call function pushes additional information

onto the stack, this subfunction returns the number of bytes of stack

space the function requires.

CALLING PARAMETERS

AX = 5602h

Contains the Get Page Map Stack Space Size subfunction.

RESULTS

These results are valid only if the status returned is zero.

BX = stack space required

Contains the number of bytes which the Alter Page Map &

Call function will require.  In other words, BX contains

the number (including the return address) which has to be

added to the stack pointer to remove all elements from the

stack.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0     SUCCESSFUL

The size of the array has been returned.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

stack_space_reqd                   DW ?

MOV  AX, 5602h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

84

MOV  stack_space_reqd, BX          ; save required stack size count

85

FUNCTION 24   MOVE/EXCHANGE MEMORY REGION

MOVE MEMORY REGION SUBFUNCTION

PURPOSE

This subfunction copies a region of memory in the following memory

source/destination combinations.

o   conventional memory to conventional memory

o   conventional memory to expanded memory

o   expanded memory to conventional memory

o   expanded memory to expanded memory

You do not have to save and restore the expanded memory mapping

context to perform these move operations.  The current mapping context

is maintained throughout this operation.

The length of the region is limited by the amount of expanded memory

allocated to the handles specified.

However, in most practical applications, the region length will be

considerably smaller.  A region length of zero is not an error, and no

move will be performed.

A region length which exceeds 16K bytes is not an error.  In this case

the function assumes that a group of logical pages is the target for

the move.  The logical page specified represents the first logical

page in which the move will take place.  If the region length exceeds

16K bytes, or if the region is less than 16K bytes but spans logical

pages, there must be sufficient logical pages remaining after the

first logical page for the entire region to fit.

If your application needs to save a region of conventional memory in

expanded memory, you can move it without having to perform a save or

restore of the current mapping context.

The memory manager maintains the context.  A move of up to 1M byte may

be performed, although practical lengths are substantially less than

this value.

If the source and destination handles are identical, the source and

destination regions are tested for overlap before the move.  If they

overlap, the move direction is chosen so that the destination region

receives an intact copy of the source region.  A status will be

returned indicating that this overlap has occurred.

CALLING PARAMETERS

AX = 5700h

Contains the Move Memory Region function.

move_source_dest_struct      STRUC

region_length             DD  ?

86

source_memory_type        DB  ?

source_handle             DW  ?

source_initial_offset     DW  ?

source_initial_seg_page   DW  ?

dest_memory_type          DB  ?

dest_handle               DW  ?

dest_initial_offset       DW  ?

dest_initial_seg_page     DW  ?

move_source_dest_struct      ENDS

DS:SI = pointer to exchange_source_dest structure

Contains a pointer to a data structure which contains the

source and destination information for the exchange.  The

structure members are described here:

.region_length

The first member is a double word which specifies the

length of the memory region (in bytes) to be moved.

.source_memory_type

The second member is a byte which specifies the type of

memory where the source region resides.  A value of zero

indicates that the source region resides in conventional

memory (excluding the page frame segment).  A value of one

indicates that the source region resides in expanded

memory.

.source_handle

If the source region resides in expanded memory, the third

member is a word which specifies the handle number

associated with the source memory region.  If the source

region resides in conventional memory, this variable has no

meaning and should be set to zero for future compatibility.

.source_initial_offset

The fourth member is a word which specifies the offset

within the source region from which to begin the move.  If

the source region resides in expanded memory, the

source_initial_offset is relative to the beginning of the

16K logical page.  Because the offset is relative to the

beginning of a 16K expanded memory page, it may only take

on values between 0000h and 3FFFh.

If the source region resides in conventional memory, the

source_initial_offset is a word which specifies the offset,

relative to the beginning of the source segment, from which

to begin the move.  Because the offset is relative to the

beginning of a 64K-byte conventional memory segment, it may

take on values between 0000h and FFFFh.

.source_initial_seg_page

The fifth member  is a word which specifies the initial

segment or logical page number within the source region

from which to begin the move.  If the source region resides

in expanded memory, the value specifies the logical page

within the source region from which to begin the move.  If

the source region resides in conventional memory, the

87

source_initial_seg_page specifies the initial segment

address within conventional memory from which to begin the

move.

.dest_memory_type

The sixth member is a byte which specifies the type of

memory where the destination region resides.  A value of

zero indicates conventional memory; a value of one

indicates expanded memory.

.dest_handle

If the destination region resides in expanded memory, the

seventh member is a word which specifies the handle number

associated with the destination memory region.  If the

destination region resides in conventional memory, this

variable has no meaning and should be set to zero for

future compatibility.

.dest_initial_offset

The eighth member is a word which specifies the offset

within the destination region from which to begin the move.

If the destination region resides in expanded memory, the

dest_initial_offset is relative to the beginning of the

16K-byte logical page.  Because the offset is relative to

the beginning of a 16K-byte expanded memory page, it may

take on values between 0000h and 3FFFh.

If the destination region resides in conventional memory,

the dest_initial_offset is a word which specifies the

offset, relative to the beginning of the destination

segment, to begin the move.  Because the offset is relative

to the beginning of a 64K conventional memory segment, it

may take on values between 0000h and FFFFh.

.dest_initial_seg_page

The ninth member is a word which specifies the initial

segment or logical page number within the destination

region from which to begin the move.

If the destination region resides in expanded memory then

the value specifies the logical page within the destination

region from which to begin the exchange.

If the destination region resides in conventional memory,

the dest_initial_seg_page specifies the initial segment

address within conventional memory from which to begin the

move.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The memory regions have been moved.

88

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE

The manager couldn't find either the source or destination

EMM handles.  The memory manager doesn't have any

information on the handles specified.  The program may have

corrupted its EMM handles.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Ah   NON-RECOVERABLE

One or more of the logical pages is out of the range of

logical pages allocated to the source/destination handle.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 92h   SUCCESSFUL

The source and destination expanded memory regions have the

same handle and overlap.  This is valid for a move.  The

move has been completed and the destination region has a

full copy of the source region.  However, at least a

portion of the source region has been overwritten by the

move.  Note that the source and destination expanded memory

regions with different handles will never physically

overlap because the different handles specify totally

different regions of expanded memory.

AH = 93h   CONDITIONALLY-RECOVERABLE

The length of the source or destination expanded memory

region specified exceeds the length of the expanded memory

region allocated either the source or destination handle.

Insufficient pages are allocated to this handle to move a

region of the size specified.  The program can recover from

this condition by allocating additional pages to the

destination or source handle and attempting to execute the

function again.  However, if the application program

allocated as much expanded memory as it thought it needed,

this may be a program error and is not recoverable.

AH = 94h   NON-RECOVERABLE

The conventional memory region and expanded memory region

overlap.  This is invalid, the conventional memory region

cannot overlap the expanded memory region.

AH = 95h   NON-RECOVERABLE

The offset within the logical page exceeds the length of

the logical page.  The initial source or destination

offsets within an expanded memory region must be between

0000h and 3FFFh (16383 or (length of a logical page - 1)).

89

AH = 96h   NON-RECOVERABLE

Region length exceeds 1M byte.

AH = 98h   NON-RECOVERABLE

The memory source and destination types are undefined.

AH = A2h   NON-RECOVERABLE

An attempt was made to wrap around the 1M-byte address

space of conventional memory during the move.  The

combination of source/destination starting address and

length of the region to be moved exceeds 1M byte.  No data

was moved.

EXAMPLE

move_source_dest                   move_source_dest_struct (?)

MOV  AX, SEG move_source_dest

MOV  DS, AX

LEA  SI, move_source_dest          ; DS:SI points to

; move_source_dest

MOV  AX, 5700h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

90

FUNCTION 24   MOVE/EXCHANGE MEMORY REGION

EXCHANGE MEMORY REGION SUBFUNCTION

PURPOSE

This subfunction exchanges (using a string move) a region of memory in

any of the following memory source/destination combinations.

o   conventional memory to conventional memory

o   conventional memory to expanded memory

o   expanded memory to conventional memory

o   expanded memory to expanded memory

The term expanded memory region refers only to the area of memory

above 640K bytes (9FFFFh).  If a system provides mappable conventional

memory, this function treats the mappable conventional memory regions

as ordinary conventional memory.  The contents of the source region

and the destination region are exchanged.

The exchange operation can be performed without having to save and

restore the expanded memory mapping context.  The current mapping

context is maintained throughout this operation.  The length of the

region is limited to the amount of expanded memory allocated to the

specified EMM handles.  A length of zero is not an error; however, no

exchange will be performed.  A region length which exceeds 16K bytes

is not an error.  In this case the function assumes that a group of

logical pages is the target for the exchange.  The logical page

specified represents the first logical page in which the exchange will

take place.  If the region length exceeds 16K bytes, or if the region

is less than 16K bytes but spans logical pages, there must be

sufficient logical pages remaining after the first logical page for

the entire region to fit.

If your application needs to exchange a region of conventional memory

with expanded memory, you can simply exchange it with the region of

interest without having to perform a save or restore of the current

mapping context.

An exchange of up to 1M byte may be performed, although practical

lengths are obviously below that value.  Checking is done before

starting the exchange to prevent the possibility of overlap during the

exchange operation.  Overlapping source and destination regions for a

exchange are invalid, and the exchange will not take place.

CALLING PARAMETERS

AX = 5701h

Contains the Exchange Memory Region function.

xchg_source_dest_struct      STRUC

region_length             DD ?

source_memory_type        DB ?

source_handle             DW ?

91

source_initial_offset     DW ?

source_initial_seg_page   DW ?

dest_memory_type          DB ?

dest_handle               DW ?

dest_initial_offset       DW ?

dest_initial_seg_page     DW ?

xchg_source_dest_struct      ENDS

DS:SI = pointer to move_source_dest structure

Contains a pointer to the data structure which contains the

source & destination information for the exchange.  The

structure members are described here:

.region_length

The first member is a double word which specifies the

length of the memory region to be exchanged.

.source_memory_type

The second member is a byte which specifies the type of

memory where the source region resides.  A value of zero

indicates that the source region resides in conventional

memory.  A value of one indicates that the source region

resides in expanded memory.

.source_handle

If the source region resides in expanded memory, the third

member is a word which specifies the handle number

associated with the source memory region.  If the source

region resides in conventional memory, this variable has no

meaning and should be set to zero for future compatibility.

.source_initial_offset

The fourth member is a word which specifies the offset

within the source region from which to begin the exchange.

If the source region resides in expanded memory, the

source_initial_offset is relative to the beginning of the

16K-byte logical page.  Because the offset is relative to

the beginning of a 16K-byte expanded memory page, it may

only take on values between 0000h and 3FFFh.

If the source region resides in conventional memory, the

source_initial_offset is a word which specifies the offset,

relative to the beginning of the source segment, to begin

the exchange at.  Because the offset is relative to the

beginning of a 64K-byte conventional memory segment, it may

only take on values between 0000h and FFFFh.

.source_initial_seg_page

The fifth member is a word which specifies the initial

segment or logical page number within the source region

from which to begin the exchange.

If the source region resides in expanded memory then the

value specifies the logical page within the source region

from which to begin the exchange.

92

If the source region resides in conventional memory, the

source_initial_seg_page specifies the initial segment

address within conventional memory from which to begin the

exchange.

.dest_memory_type

The sixth member is a byte which specifies the type of

memory where the destination region resides.  A value of

zero indicates that the destination region resides in

conventional memory (excluding the page frame segment).  A

value of one indicates that the destination region resides

in expanded memory.

.dest_handle

If the destination region resides in expanded memory, the

seventh member is a word which specifies the handle number

associated with the destination memory region.  If the

destination region resides in conventional memory , then

this variable has no meaning and should be set to zero for

future compatibility.

.dest_initial_offset

The eighth member is a word which specifies the offset

within the destination region from which to begin the

exchange.

If the destination region resides in expanded memory, the

dest_initial_offset is relative to the beginning of the

16K-byte logical page.  Because the offset is relative to

the beginning of a 16K-byte expanded memory page, it may

only take on values between 0000h and 3FFFh.

If the destination region resides in conventional memory,

the dest_initial_offset is a word which specifies the

offset, relative to the beginning of the destination

segment, to begin the exchange at.  Because the offset is

relative to the beginning of a 64K-byte conventional memory

segment, it may only take on values between 0000h and

FFFFh.

.dest_initial_seg_page

The ninth member is a word which specifies the initial

segment or logical page number within the destination

region from which to begin the exchange.

If the destination region resides in expanded memory, the

value specifies the logical page within the destination

region from which to begin the exchange.

If the destination region resides in conventional memory,

the dest_initial_seg_page specifies the initial segment

address within conventional memory from which to begin the

exchange.

REGISTERS MODIFIED

AX

93

STATUS

AH = 0     SUCCESSFUL

The memory regions have been exchanged.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 83h   NON-RECOVERABLE

The manager could not find either the source or destination

EMM handles.  The memory manager does not currently have

any information pertaining to the handles specified.  The

program may have corrupted its EMM handles.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Ah   NON-RECOVERABLE

One or more of the logical pages is out of the range of

logical pages allocated to the source/destination handle.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 93h   CONDITIONALLY RECOVERABLE

The length of the source or destination expanded memory

region specified, exceeds the length of the expanded memory

region allocated to the source or destination specified EMM

handle.  There are insufficient pages allocated to this

handle to exchange a region of the size specified.  The

program can recover from this condition by attempting to

allocate additional pages to the destination or source

handle and attempting to execute the function again.

However, if the application program was allocated as much

expanded memory as it thought it needed, this may be a

program error and is therefore not recoverable.

AH = 94h   NON-RECOVERABLE

The conventional memory region and expanded memory region

overlap.  This is invalid, the conventional memory region

cannot overlap the expanded memory region.

AH = 95h   NON-RECOVERABLE

The offset within the logical page exceeds the length of

the logical page.  The initial source or destination

offsets within an expanded memory region must be between

0000h and 3FFFh (16383 or (length of a logical page - 1)).

AH = 96h   NON-RECOVERABLE

Region length exceeds 1M Byte limit.

94

AH = 97h   NON-RECOVERABLE

The source and destination expanded memory regions have the

same handle and overlap.  This is invalid, the source and

destination expanded memory regions cannot have the same

handle and overlap when they are being exchanged. Note that

the source and destination expanded memory regions which

have different handles will never physically overlap

because the different handles specify totally different

regions of expanded memory.

AH = 98h   NON-RECOVERABLE

The memory source and destination types are undefined.

AH = A2h   NON-RECOVERABLE

An attempt was made to wrap around the 1M-byte address

space of conventional memory during the exchange.  The

source starting address together with the length of the

region to be exchanged exceeds 1M byte.  No data was

exchanged.

EXAMPLE

xchg_source_dest                   xchg_source_dest_struct (?)

MOV  AX, SEG xchg_source_dest

MOV  DS, AX

LEA  SI, xchg_source_dest          ; DS:SI points to

; xchg_source_dest

MOV  AX, 5701h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

95

FUNCTION 25   GET MAPPABLE PHYSICAL ADDRESS ARRAY

GET MAPPABLE PHYSICAL ADDRESS ARRAY SUBFUNCTION

PURPOSE

This subfunction returns an array containing the segment address and

physical page number for each mappable physical page in a system.  The

contents of this array provide a cross reference between physical page

numbers and the actual segment addresses for each mappable page in the

system.  The array is sorted in ascending segment order.  This does

not mean that the physical page numbers associated with the segment

addresses are also in ascending order.

CALLING PARAMETERS

AX = 5800h

Contains the Get Mappable Physical Address Array

subfunction.

mappable_phys_page_struct   STRUC

phys_page_segment        DW ?

phys_page_number         DW ?

mappable_phys_page_struct   ENDS

ES:DI = mappable_phys_page

Contains a pointer to an application-supplied memory area

where the memory manager will copy the physical address

array.  Each entry in the array is a structure containing

two members:

.phys_page_segment

The first member is a word which contains the segment

address of the mappable physical page associated with the

physical page number following it.  The array entries are

sorted in ascending segment address order.

.phys_page_number

The second member is a word which contains the physical

page number which corresponds to the previous segment

address.  The physical page numbers are not necessarily in

ascending order.

EXAMPLE 1

An expanded memory board has its page frame starting at address C0000h

and has no mappable conventional memory.  For this configuration,

physical page 0 corresponds to segment address C000h, physical page 1

corresponds to segment address C400h, etc.  The array would contain

the following data (in this order):

C000h, 00h

C400h, 01h

C800h, 02h

CC00h, 03h

EXAMPLE 2

96

An expanded memory board has a large page frame starting at address

C0000h and has mappable conventional memory from 90000h through

9FFFFh.  For this configuration, physical page 0 corresponds to

segment address C000h, physical page 1 corresponds to segment address

C400h, etc.  The array would contain the following data in the order

specified.  Note that the expanded memory region always has the lowest

numerically valued physical page numbers.

9000h, 0Ch

9400h, 0Dh

9800h, 0Eh

9C00h, 0Fh

C000h, 00h

C400h, 01h

C800h, 02h

CC00h, 03h

D000h, 04h

D400h, 05h

D800h, 06h

DC00h, 07h

E000h, 08h

E400h, 09h

E800h, 0Ah

EC00h, 0Bh

RESULTS

These results are valid only if the status returned is zero.

CX = number of entries in the mappable_phys_page

Multiply this number by (SIZE mappable_phys_page_struct) to

determine the number of bytes the physical page address

array requires.

REGISTERS MODIFIED

AX, CX

STATUS

AH = 0     SUCCESSFUL

The hardware configuration array has been returned.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

97

EXAMPLE

mappable_phys_page                 mappable_phys_page_struct (?)

MOV  AX, SEG mappable_phys_page

MOV  ES, AX

LEA  DI, mappable_phys_page        ; ES:DI points to

; mappable_phys_page

MOV  AX, 5800h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  mappable_page_entry_count, CX ; save mappable page entry count

98

FUNCTION 25   GET MAPPABLE PHYSICAL ADDRESS ARRAY

GET MAPPABLE PHYSICAL ADDRESS ARRAY ENTRIES SUBFUNCTION

PURPOSE

This subfunction gets the number of entries which will be required for

the array the first subfunction returns.

CALLING PARAMETERS

AX = 5801h

Contains the Get Physical Page Address Array Entry Count

subfunction.  This subfunction returns a word which

represents the number of entries in the array returned by

the previous subfunction.  This number also represents the

number of mappable physical pages in a system.

RESULTS

These results are valid only if the status returned is zero.

CX = number of entries in the mappable_phys_page

Multiply this number by (SIZE mappable_phys_page_struct) to

determine the number of bytes the physical page address

array will require.

REGISTERS MODIFIED

AX, CX

STATUS

AH = 0     SUCCESSFUL

The physical page address array has been returned.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

mappable_page_entry_count          DW ?

MOV  AX, 5801h                     ; load function code

INT  67h                           ; call memory manager

OR   AH, AH                        ; check EMM status

99

JNZ  emm_err_handler               ; jump to error handler on error

MOV  mappable_page_entry_count, CX ; save mappable page entry count

100

FUNCTION 26   GET EXPANDED MEMORY HARDWARE INFORMATION

GET HARDWARE CONFIGURATION ARRAY SUBFUNCTION

Note..............................................................

This function is for use by operating systems only.  This function can

be disabled at any time by the operating system.  Refer to Function 30

for a description of how an operating system does this.

PURPOSE

This subfunction returns an array containing expanded memory hardware

configuration information for use by an operating system/environment.

CALLING PARAMETERS

AX = 5900h

Contains the Get Hardware Configuration Array subfunction.

hardware_info_struct         STRUC

raw_page_size             DW ?

alternate_register_sets   DW ?

context_save_area_size    DW ?

DMA_register_sets         DW ?

DMA_channel_operation     DW ?

hardware_info_struct         ENDS

ES:DI = hardware_info

Contains a pointer to a memory area that the operating

system supplies where the memory manager will copy expanded

memory hardware information.  The structure contains these

five members:

.raw_page_size

The first member is a word which contains the size of a raw

mappable physical page in paragraphs (16 bytes).  LIM

standard pages are always 16K bytes.  However, other

implementations of expanded memory boards do not

necessarily comply with this standard and can emulate a

16K-byte page by mapping in multiple smaller pages.  This

member specifies the size of a mappable physical page

viewed from the hardware implementation level.

.alternate_register_sets

The second member is a word which specifies the number of

alternate mapping register sets.  The additional mapping

register sets are termed alternate mapping register sets in

this document.

All expanded memory boards have at least one set of

hardware registers to perform the logical to physical page

mapping.  Some expanded memory boards have more than one

set of these mapping registers.  This member specifies how

many of these alternate mapping register sets exist (beyond

the one set that all expanded memory boards have) on the

expanded memory boards in the system.  If an expanded

101

memory card has only one set of mapping registers (that is,

no alternate mapping register sets) this member has a value

of zero.

.context_save_area_size

The third member is a word which contains the storage

requirements for the array required to save a mapping

context.  The value returned in this member is exactly the

same as that returned by Function 15 (Get Size of Page Map

Save Array subfunction).

.DMA_channels

The fourth member is a word which contains the number of

register sets that can be assigned to DMA channels.  These

DMA register sets, although similar in use to alternate

register sets, are for DMA mapping and not task mapping.

If the expanded memory hardware does not support DMA

register sets, care must be taken when DMA is taking place.

In a multitasking operating system, when one task is

waiting for DMA to complete, it is useful to be able to

switch to another task.  However, if the DMA is taking

place in memory that the second task will need to remap,

remapping would be disastrous.

If the expanded memory hardware can detect when DMA is

occurring, the OS/E should allow task switches and

remapping during DMA.  If no special support for DMA is

available, no remapping should be done when DMA is in

progress.

.DMA_channel_operation

The fifth member is a word which specifies a special case

for the DMA register sets.  A value of zero specifies that

the DMA register sets behave as described in Function 28.

A value of one specifies that the expanded memory hardware

has only one DMA register set.  In addition, if any channel

is mapped through this register set, then all channels are

mapped through it.  For LIM standard boards, this value is

zero.

RESULTS

These results are valid only if the status returned is zero.

hardware_info

Contains the expanded memory hardware specific information

described above.

REGISTERS MODIFIED

AX

STATUS

102

AH = 0     SUCCESSFUL

The hardware configuration array has been returned.

AH = 80h   NON-RECOVERABLE

The manager has detected a malfunction in the memory

manager software.

AH = 81h   NON-RECOVERABLE

The manager has detected a malfunction in the expanded

memory hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the manager is not defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = A4h   NON-RECOVERABLE

Access to this function has been denied by the operating

system.  The function cannot be used at this time.

EXAMPLE

hardware_info                      hardware_info_struct (?)

MOV  AX, SEG hardware_info

MOV  ES, AX

LEA  DI, hardware_info             ; ES:DI points to hardware_info

MOV  AX, 5900h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

103

FUNCTION 26   GET EXPANDED MEMORY HARDWARE INFORMATION

GET UNALLOCATED RAW PAGE COUNT SUBFUNCTION

PURPOSE

The Get Unallocated Raw Page Count subfunction returns the number of

unallocated non-standard length mappable pages as well as the total

number of non-standard length mappable pages in expanded memory to the

operating system.

One variety of expanded memory board has a page size which is a sub-

multiple of 16K bytes.  An expanded memory page which is a sub-

multiple of 16K is termed a raw page.  An operating system may deal

with mappable physical page sizes which are sub-multiples of 16K

bytes.

If the expanded memory board supplies pages in exact multiples of 16K

bytes, the number of pages this function returns is identical to the

number Function 3 (Get Unallocated Page Count) returns.  In this case,

there is no difference between a page and a raw page.

CALLING PARAMETERS

AX = 5901h

Contains the Get Unallocated Raw Page Count subfunction.

RESULTS

These results are valid only if the status returned is zero.

BX = unallocated raw pages

The number of raw pages that are currently available for

use.

DX = total raw pages

The total number of raw pages in expanded memory.

REGISTERS MODIFIED

AX, BX, DX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the number unallocated raw pages

and the number of total raw pages in expanded memory.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

104

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

unalloc_raw_pages                  DW ?

total_raw_pages                    DW ?

MOV  AX, 5901h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  unalloc_raw_pages, BX         ; save unallocated raw page

; count

MOV  total_raw_pages, DX           ; save total raw page count

105

FUNCTION 27   ALLOCATE STANDARD/RAW PAGES

ALLOCATE STANDARD PAGES SUBFUNCTION

PURPOSE

The Allocate Standard Pages subfunction allocates the number of

standard size (16K bytes) pages that the operating system requests and

assigns a unique EMM handle to these pages.  The EMM handle owns these

pages until the operating system deallocates them.  This subfunction

allows you to allocate zero pages to a handle, unlike Function 4

(Allocate Pages).

Note..............................................................

This note affects expanded memory manager implementers and operating

system developers only.  Applications should not use the following

characteristic of the memory manager.  An application violating this

rule will be incompatible with future versions of Microsoft's

operating systems and environments.

To be compatible with this specification, an expanded memory manager

will provide a special handle which is available to the operating

system only.  This handle will have a value of 0000h and will have a

set of pages allocated to it when the expanded memory manager driver

installs.  The pages that the memory manager will automatically

allocate to handle 0000h are those that backfill conventional memory.

Typically, this backfill occurs between addresses 40000h (256K) and

9FFFFh (640K).  However, the range can extend below and above this

limit if the hardware and memory manager have the capability.

An operating system won't have to invoke Function 27 to obtain this

handle because it can assume the handle already exists and is

available for use immediately after the expanded memory device driver

installs.  When an operating system wants to use this handle, it uses

the special handle value of 0000h. The operating system will be able

to invoke any EMM function using this special handle value.  To

allocate pages to this handle, the operating system need only invoke

Function 18 (Reallocate pages).

There are two special cases for this handle:

1.  Function 27 (Allocate Standard Pages).  This function must never

return zero as a handle value.  Applications must always invoke

Function 27 to allocate pages and obtain a handle which identifies

the pages which belong to it.  Since Function 27 never returns a

handle value of zero, an application will never gain access to

this special handle.

2.  Function 6 (Deallocate Pages). If the operating system uses it to

deallocate the pages which are allocated to this handle, the pages

the handle owns will be returned to the manager for use.  But the

handle will not be available for reassignment.  The manager should

treat a deallocate pages function request for this handle the same

as a reallocate pages function request, where the number of pages

to reallocate to this handle is zero.

106

CALLING PARAMETERS

AX = 5A00h

Contains the Allocate Standard Pages subfunction.

BX = num_of_standard_pages_to_alloc

Contains the number of standard pages the operating system

wants to allocate.

RESULTS

These results are valid only if the status returned is zero.

DX = handle

Contains a unique EMM handle.  The operating system must

use this EMM handle as a parameter in any function that

requires it.  Up to 255 handles may be obtained.  (Both

Function 27 and Function 4 must share the same 255

handles).

For all functions using this handle, the length of the

physical and logical pages allocated to it are standard

length (that is, 16K bytes).

REGISTERS MODIFIED

AX, DX

STATUS

AH = 0     SUCCESSFUL

The manager has allocated the pages to an assigned EMM raw

handle.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 85h   RECOVERABLE

All EMM handles are being used.

AH = 87h   RECOVERABLE

There aren't enough expanded memory pages present in the

system to satisfy the operating system's request.

AH = 88h   RECOVERABLE

There aren't enough unallocated pages to satisfy the

operating system's request.

107

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

num_of_standard_pages_to_alloc     DW ?

emm_handle                         DW ?

MOV  BX, num_of_standard_pages_to_alloc

MOV  AX, 5A00h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  emm_handle, DX                ; save handle

108

FUNCTION 27   ALLOCATE STANDARD/RAW PAGES

ALLOCATE RAW PAGES SUBFUNCTION

PURPOSE

The Allocate Raw Pages subfunction allocates the number of non-

standard size pages that the operating system requests and assigns a

unique EMM handle to these pages.  The EMM handle owns these pages

until the operating system deallocates them.  This function allows you

to allocate zero pages to a handle, unlike Function 4 (Allocate

Pages).

A hardware vendor may design an expanded memory board that has a page

size which is a sub-multiple of 16K bytes.  A physical page which is a

sub-multiple of 16K is termed a raw page.  The operating system may

deal with page sizes which are sub-multiples of 16K bytes.  The memory

manager must treat any function using a handle with raw pages

allocated to it by Allocate Raw Pages differently than it does a

handle that has normal 16K-byte pages allocated to it.

Handles which are assigned using Function 4 (Allocate pages) or

Function 27 (Allocate Standard Pages subfunction) must have pages

which are 16K bytes -- this is the length of a standard expanded

memory page.  If the expanded memory board hardware is not able to

supply 16K-byte pages, the memory manager must emulate pages which are

16K bytes combining multiple non-standard size pages to form a single

16K-byte page.

Handles which are assigned using Function 27 (Allocate Raw Pages) are

called raw handles.  All logical pages allocated to a raw handle may

have a non-standard length (that is, not 16K bytes).  However, once

the operating system has allocated a number of raw pages to a handle,

it is the responsibility of the memory manager to recognize that raw

handle as one that has non-standard size pages allocated to it.  The

memory manager must identify these handles and treat all functions

which use handles which have non-standard page lengths differently.

The logical page length becomes the length of the non-standard size

page for any raw handle that Function 27 assigns.

Note..............................................................

This note affects expanded memory manager implementers and operating

system developers only.  Applications should not use the following

characteristic of the memory manager.  An application violating this

rule will be incompatible with future versions of Microsoft's

operating systems and environments.

To be compatible with this specification, an expanded memory manager

will provide a special handle which is available to the operating

system only.  This handle will have a value of 0000h and will have a

set of pages allocated to it when the expanded memory manager driver

installs.  The pages that the memory manager will automatically

allocate to handle 0000h are those that backfill conventional memory.

Typically, this backfill occurs between addresses 40000h (256K) and

9FFFFh (640K).  However, the range can extend below and above this

limit if the hardware and memory manager have the capability.

109

An operating system won't have to invoke Function 27 to obtain this

handle because it can assume the handle already exists and is

available for use immediately after the expanded memory device driver

installs.  When an operating system wants to use this handle, it uses

the special handle value of 0000h. The operating system will be able

to invoke any EMM function using this special handle value.  To

allocate pages to this handle, the operating system need only invoke

Function 18 (Reallocate pages).

There are two special cases for this handle:

1   Function 27 (Allocate Raw Pages).  This function must never return

zero as a handle value.  Applications must always invoke Function

27 to allocate pages and obtain a handle which identifies the

pages which belong to it.  Since Function 27 never returns a

handle value of zero, an application will never gain access to

this special handle.

2   Function 6 (Deallocate Pages). If the operating system uses it to

deallocate the pages which are allocated to this special handle,

the pages the handle owns will be returned to the manager for use.

But the handle will not be available for reassignment.  The

manager should treat a deallocate pages function request for this

handle the same as a reallocate pages function request, where the

number of pages to reallocate to this handle is zero.

CALLING PARAMETERS

AX = 5A01h

Contains the Allocate Raw Pages subfunction.

BX = num_of_raw_pages_to_alloc

Contains the number of raw pages the operating system wants

to allocate.

RESULTS

These results are valid only if the status returned is zero.

DX = raw handle

Contains a unique EMM raw handle.  The operating system

must use this EMM raw handle as a parameter in any function

that requires it.  Up to 255 handles may be obtained.

(Both Function 4 and Function 27 must share the same 255

handles).

For all functions using this raw handle, the length of the

physical and logical pages allocated to it may be non-

standard (that is, not 16K bytes).

REGISTERS MODIFIED

AH = 0     SUCCESSFUL

The manager has allocated the raw pages to an assigned EMM

raw handle.

110

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 85h   RECOVERABLE

All EMM handles are being used.

AH = 87h   RECOVERABLE

There aren't enough expanded memory raw pages present in

the system to satisfy the operating system's request.

AH = 88h   RECOVERABLE

There aren't enough unallocated raw pages to satisfy the

operating system's request.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

EXAMPLE

num_of_raw_pages_to_alloc          DW ?

emm_raw_handle                     DW ?

MOV  BX, num_of_raw_pages_to_alloc

MOV  AX, 5A01h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  emm_raw_handle, DX            ; save raw handle

111

FUNCTION 28   ALTERNATE MAP REGISTER SET

Note...............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.  Refer to Function 30

for a description of how an operating system can enable or disable

this function.

DESIGN CONSIDERATIONS

The hardware support for the entire set of subfunctions described is

generally not present on every expanded memory board from every vendor

of expanded memory board products.  For some of the subfunctions,

software emulation is provided.  For other subfunctions, a certain

protocol in their use must be observed.  The subfunctions for which

this is most crucial are those which address system DMA capabilities.

System DMA Capabilities & Expanded Memory Support of DMA

In a multitasking operating system, when one task is waiting for DMA

to complete, it is useful to be able to switch to another task.  This

specification describes a capability which may be designed into

expanded memory boards to provide DMA into memory regions which may be

mapped out while the DMA is occurring.  For expanded memory boards

that do not provide this, it is crucial to understand that while DMA

is in progress into a region of mappable memory, the memory mapping

context cannot be changed.  That is, all DMA action must be complete

before any remapping of pages can be done.

Expanded Memory Support of DMA Register Sets

Expanded memory boards which have DMA registers sets could support DMA

into a region of mappable memory while the memory mapping context is

being switched.  It is important to realize that these DMA register

sets are separate from the alternate map register sets.  An example of

how an OS/E might use DMA registers sets follows:

EXAMPLE 1

1.  Allocate a DMA register set.

2.  Get current register set.

3.  Set the DMA register set.

4.  Map in the memory desired.

5.  Get the DMA register set.

6.  Set the original register set.

7.  Assign the desired DMA channel to the DMA register set.

The preceding set of calls makes all DMA accesses for the desired DMA

channel get mapped through the current DMA register set regardless of

the current register set.  In other words, the DMA register set

112

overrides the current mapping register set for DMA operations on the

DMA channel specified.  A DMA channel that is not assigned to a DMA

register set has all its DMA operations mapped through the current

mapping register set.

113

FUNCTION 28   ALTERNATE MAP REGISTER SET

GET ALTERNATE MAP REGISTER SET SUBFUNCTION

Note..ss...........................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.  Refer to Function 30

for a description of how an operating system can enable or disable

this function.

PURPOSE

The subfunction does one of two things depending on the map register

set which is active at the time this function is invoked:

1.  If the preceding Set Alternate Map Register Set call was done with

the alternate map register set equal to zero (BL = 0), these

points apply:

a. The context save area pointer saved within EMM by the Set

Alternate Map Register Set subfunction) is returned by

this call.  This pointer is always returned for boards

which do not supply alternate mapping register sets.

b. If the context save area pointer returned is not equal to

zero, this subfunction copies the contents of the mapping

registers on each expanded memory board in the system into

the save area specified by the pointer.  The format of

this save area is the same as that returned by Function 15

(Get Page Map subfunction).  This is intended to simulate

getting an alternate map register set.  Note that the

memory manager does not allocate the space for the

context: the operating system must do so.

c. If the context save area pointer returned is equal to

zero, this subfunction does not copy the contents of the

mapping registers on each expanded memory board in the

system into the save area specified by the pointer.

d. The context save area pointer must have been initialized

by a previous Set Alternate Map Register Set call.  Note

that the value of the context save area pointer saved

within EMM is zero immediately after installation.

e. The context save area must be initialized by a previous

Get Page Map call (Function 15).

2.  If the preceding Set Alternate Map Register Set call was done with

the alternate map register set  greater than zero (BL > 0), then

the number of the alternate map register set which is in use at

the time that this function is invoked is returned.  The context

save area pointer is not returned in this case.

CALLING PARAMETERS

114

AX = 5B00h

Contains the Get Alternate Map Register Set subfunction.

RESULTS

These results are valid only if the status returned is zero.

IF (BL <> 0) current active alternate map register set number

Contains the alternate map register set which was active at

the time that this function was invoked.

ES:DI

Unaffected.

IF (BL = 0)

Indicates that a pointer to an area which contains the

state of all the map registers on all boards in the system,

and any additional information necessary to restore the

boards to their original state, has been returned.

ES:DI = pointer to a map register context save area

Contains a pointer to an operating system supplied context

save area.  The pointer is in standard segment:offset

format.  This pointer is always returned if the expanded

memory hardware does not supply alternate mapping register

sets.

The operating system first passes this pointer to the memory manager

whenever it invokes a Set Alternate Map Register Set subfunction (the

description follows).  If the OS/E invokes this function before

invoking a Set Alternate Map Register Set subfunction, this function

returns a pointer value of zero.  The OS/E must have allocated the

space for the save area.  However, the OS must request that the memory

manager initialize the contents of this save area before it contains

any useful information.

The OS/E must initialize the save area it has allocated by invoking

Function 15 (Get Page Map subfunction).  After the OS/E has done this,

the save area will contain the state of all the map registers on all

boards in the system.  The save area will also contain any additional

information necessary to restore the boards to their original state

when the operating system invokes a Set Alternate Map Register Set

subfunction.

REGISTERS MODIFIED

AX, BX, ES:DI

STATUS

AH = 0     SUCCESSFUL

The manager got the alternate map register set.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

115

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = A4h   NON-RECOVERABLE

The operating system denied access to this function.  The

function cannot be used at this time.

EXAMPLE

alt_map_reg_set                    DB ?

context_save_area_ptr_seg          DW ?

context_save_area_ptr_offset       DW ?

MOV  AX, 5B00h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  alt_map_reg_set, BL

TEST BL, BL

JNZ  no_ptr_returned

MOV  context_save_area_ptr_seg, ES ; save pointer values

MOV  context_save_area_ptr_offset,  DI

no_ptr_returned:

116

FUNCTION 28   ALTERNATE MAP REGISTER SET

SET ALTERNATE MAP REGISTER SET SUBFUNCTION

Notes..............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.  Refer to Function 30

for a description of how an operating system can enable or disable

this function.

PURPOSE

The subfunction does one of two things, depending on the map register

set specified:

1   If the alternate map register set specified is zero, map register

set zero is activated.  If the map register context restore area

pointer is not equal to zero, the contents of the restore area

pointed to by ES:DI are copied into register set zero on each

expanded memory board in the system.  If the pointer is equal to

zero, the contents are not copied.

Regardless of its value, the map register context restore area

pointer is saved within the memory manager.  It will be used

during the Get Alternate Map Register Set subfunction.

The operating system must supply the pointer to the area.  This

subfunction is intended to simulate setting an alternate map

register set.  Note that the operating system must allocate the

space for the context.  The memory manager saves the context save

area pointer internally.

2   If the alternate map register set specified is not zero, the

alternate map register set specified is activated.  The restore

area, which the operating system is pointing to, is not used.

CALLING PARAMETERS

AX = 5B01h

Contains the Set Alternate Map subfunction.

BL = new alternate map register set number

Contains the number of the alternate map register set which

is to be activated.

If BL <> 0

A pointer to a map register context restore area is not

required and the contents of ES:DI are unaffected and

ignored.  The alternate map register set specified in BL is

activated if the board supports it.

If BL = 0

A pointer to an area which contains the state of all the

map registers on all boards in the system, and any

additional information necessary to restore the boards to

their original state, has been passed in ES:DI.

117

ES:DI = pointer to a map register context restore area

Contains a pointer to an OS/E supplied map register context

restore area.  The pointer is in standard segment:offset

format.  This pointer must always be passed if the expanded

memory hardware does not supply alternate mapping register

sets.

The memory manager must save this pointer whenever the OS/E

invokes this function.  The OS/E must have allocated the

space for the restore area.  Additionally, the contents of

this restore area must have been initialized by the memory

manager before it will contain any useful information.  The

OS/E initializes the restore area it has allocated by

invoking Function 15 (Get Page Map subfunction).  After the

OS/E has done this, the restore area will contain the state

of the map registers on all boards in the system, and any

additional information necessary to restore the boards to

their original state when the operating system invokes a

Set Alternate Map Register Set subfunction.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager set the alternate map register set.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 9Ah   NON-RECOVERABLE

Alternate map register sets are supported, but the

alternate map register set specified is not supported.

AH = 9Ch   NON-RECOVERABLE

Alternate map register sets are not supported, and the

alternate map register set specified is not zero.

AH = 9Dh   NON-RECOVERABLE

Alternate map register sets are supported, but the

alternate map register set specified is either not defined

or not allocated.

118

AH = A3h   NON-RECOVERABLE

The contents of the source array have been corrupted, or

the pointer passed to the subfunction is invalid.

AH = A4h   NON-RECOVERABLE

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

alt_map_reg_set                    DB ?

context_restore_area_ptr_seg       DW ?

context_restore_area_ptr_offset    DW ?

MOV  AX, 5B01h                     ; load function code

MOV  BL, alt_map_reg_set

TEST BL, BL

JZ   no_ptr_passed

MOV  ES, context_restore_area_ptr_seg

MOV  DI, context_restore_area_ptr_offset

no_ptr_passed:

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

119

FUNCTION 28   ALTERNATE MAP REGISTER SET

GET ALTERNATE MAP SAVE ARRAY SIZE SUBFUNCTION

Note...............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.  Refer to Function 30

for a description of how an operating system can enable or disable

this function.

PURPOSE

This subfunction returns the storage requirements for the map register

context save area referenced by the other subfunctions.

CALLING PARAMETERS

AX = 5B02h

Contains the Get Alternate Map Save Array Size subfunction.

RESULTS

These results are valid only if the status returned is zero.

DX = size_of_array

Contains the number of bytes that will be transferred to

the memory area supplied by an operating system whenever an

operating system requests the Get, Set, or Get and Set

subfunction.

REGISTERS MODIFIED

AX, DX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the array size.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

120

AH = A4h   NON-RECOVERABLE

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

size_of_array                      DW ?

MOV  AX, 5B02h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  size_of_array, DX             ; save size of array

121

FUNCTION 28   ALTERNATE MAP REGISTER SET

ALLOCATE ALTERNATE MAP REGISTER SET SUBFUNCTION

Note...............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.   Refer to Function 30

for a description of how an operating system can enable or disable

this function.

PURPOSE

The Allocate Alternate Map Register Set subfunction gets the number of

an alternate map register set for an operating system if an alternate

map register set is currently available for use.  If the hardware does

not support alternate map register sets, an alternate map register set

number of zero will be returned.

The alternate map register set allocated may be referred to by this

number when using the Get or Set Alternate Map Register Set

subfunctions.  The operating system can use these subfunctions to

switch map contexts very rapidly on expanded memory boards with

alternate map register sets.

This subfunction copies the currently active alternate map register

sets contents into the newly allocated alternate map register set's

mapping registers.  This is done so that when the OS/E performs a Set

Alternate Map Register Set subfunction the memory mapped before the

allocation of the new alternate map will be available for reading and

writing.  This function does not actually change the alternate map

register set in use, but in addition to allocating a new alternate map

register set, it prepares the new alternate map register set for a

subsequent Set Alternate Map Register Set subfunction.

CALLING PARAMETERS

AX = 5B03h

Contains the Allocate Alternate Map Register Set

subfunction.

RESULTS

These results are valid only if the status returned is zero.

BL = alternate map register set number

Contains the number of an alternate map register set.  If

there are no alternate map register sets supported by the

hardware, a zero will be returned.  In this case, the Get

Alternate Map function (Function 28) should be invoked in

order to obtain a pointer to a map register context save

area.  The OS/E must supply this save area.  The save area

is necessary because the hardware doesn't support alternate

map register sets.

REGISTERS MODIFIED

122

AX, BX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the array size.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 9Bh   NON-RECOVERABLE

Alternate map register sets are supported.  However, all

alternate map register sets are currently allocated.

AH = A4h   NON-RECOVERABLE

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

alt_map_reg_num                    DB ?

MOV  AX, 5B03h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  alt_map_reg_num, BL           ; save # of alternate map reg

; set

123

FUNCTION 28   ALTERNATE MAP REGISTER SET

DEALLOCATE ALTERNATE MAP REGISTER SET SUBFUNCTION

Note...............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.  Refer to Function 30

for a description of how an operating system can enable or disable

this function.

PURPOSE

The Deallocate Alternate Map Register Set subfunction returns the

alternate map register set to the memory manager for future use.  The

memory manager may reallocate the alternate map register set when

needed.

This subfunction also makes the mapping context of the alternate map

register specified unavailable for reading or writing (unmapping).

This protects the pages previously mapped in an alternate map register

set by making them inaccessible.  Note that the current alternate map

register set cannot be deallocated.  This makes all memory which was

currently mapped into conventional and expanded memory inaccessible.

CALLING PARAMETERS

AX = 5B04h

Contains the Deallocate Alternate Map Register Set

subfunction.

BL = alternate register set number

Contains the number of the alternate map register set to

deallocate.  Map register set zero cannot be allocated or

deallocated.  However, if alternate map register set zero

is specified and this subfunction is invoked, no error will

be returned.  The function invocation is ignored in this

case.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the array size.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

124

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 9Ch   NON-RECOVERABLE

Alternate map register sets are not supported and the

alternate map register set specified is not zero.

AH = 9Dh   NON-RECOVERABLE

Alternate map register sets are supported, but the

alternate map register set specified is either not defined

or not allocated.

AH = A4h   NON-RECOVERABLE

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

alternate_map_reg_set              DB ?

MOV  AL, alternate_map_reg_set     ; specify alternate map reg set

MOV  AX, 5B04h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

125

FUNCTION 28   ALTERNATE MAP REGISTER SET

ALLOCATE DMA REGISTER SET SUBFUNCTION

Note...............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.  Refer to Function 30

for a description of how an operating system can enable or disable

this function.

PURPOSE

The Allocate DMA Register Set subfunction gets the number of a DMA

register set for an OS/E, if a DMA register set is currently available

for use.  If the hardware does not support DMA register sets, a DMA

register set number of zero will be returned.

In a multitasking operating system, when one task is waiting for DMA

to complete, it is useful to be able to switch to another task.

However, if the DMA is being mapped through the current register set,

the switching cannot occur.  That is, all DMA action must be complete

before any remapping of pages can be done.

The operating system would initiate a DMA operation on a specific DMA

channel using a specific alternate map register set.  This alternate

map register set would not be used again, by the operating system or

an application, until after the DMA operation is complete.  The

operating system guarantees this by not changing the contents of the

alternate map register set, or allowing an application to change the

contents of the alternate map register set, for the duration of the

DMA operation.

CALLING PARAMETERS

AX = 5B05h

Contains the Allocate DMA Register Set subfunction.

RESULTS

These results are valid only if the status returned is zero.

BL = DMA register set number

Contains the number of a DMA register set.  If there are no

DMA register sets supported by the hardware, a zero will be

returned.

REGISTERS MODIFIED

AX, BX

STATUS

AH = 0     SUCCESSFUL

The manager has allocated the DMA register set.

126

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 9Bh   NON-RECOVERABLE

DMA register sets are supported.  However, all DMA register

sets are currently allocated.

AH = A4h   NON-RECOVERABLE

Access to this function has been denied by the operating

system.  The function cannot be used at this time.

EXAMPLE

DMA_reg_set_number                 DB ?

MOV  AX, 5B05h                     ; load function code

INT  67h                           ; call memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  DMA_reg_set_number, BL        ; save number of DMA register

; set

127

FUNCTION 28   ALTERNATE MAP REGISTER SET

ENABLE DMA ON ALTERNATE MAP REGISTER SET SUBFUNCTION

Note...............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.  Refer to Function 30

for a description of how an operating system can enable or disable

this function.

PURPOSE

This subfunction allows DMA accesses on a specific DMA channel to be

associated with a specific alternate map register set.  In a

multitasking operating system, when a task is waiting for the

completion of DMA, it is useful to be able to switch to another task

until the DMA operation completes.

Any DMA on the specified channel will go through the specified DMA

register set regardless of the current register set.  If a DMA channel

is not assigned to a DMA register set, DMA for that channel will be

mapped through the current register set.

CALLING PARAMETERS

AX = 5B06h

Contains the Enable DMA on Alternate Map Register Set

subfunction.

BL = DMA register set number

Contains the number of the alternate map register set to be

used for DMA operations on the DMA channel specified by DL.

If the alternate map register set specified is zero, no

special action will be taken on DMA accesses for the DMA

channel specified.

DL = DMA channel number

Contains the DMA channel which is to be associated with the

DMA map register set specified in BL.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has enabled DMA on the DMA register set and the

DMA channel specified.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

128

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 9Ah   NON-RECOVERABLE

Alternate DMA register sets are supported, but the

alternate DMA register set specified is not supported.

AH = 9Ch   NON-RECOVERABLE

Alternate DMA register sets are not supported, and the DMA

register set specified is not zero.

AH = 9Dh   NON-RECOVERABLE

DMA register sets are supported, but the DMA register set

specified is either not defined or not allocated.

AH = 9Eh   NON-RECOVERABLE

Dedicated DMA channels are not supported.

AH = 9Fh   NON-RECOVERABLE

Dedicated DMA channels are supported.  But the DMA channel

specified is not supported.

AH = A4h   NON-RECOVERABLE

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

alt_map_reg_set                    DB ?

DMA_channel_num                    DB ?

MOV  BL, alt_map_reg_set

MOV  DL, DMA_channel_num

MOV  AX, 5B06h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

129

FUNCTION 28   ALTERNATE MAP REGISTER SET

DISABLE DMA ON ALTERNATE MAP REGISTER SET SUBFUNCTION

Note...............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.  Refer to Function 30

for a description of how an operating system can enable or disable

this function.

PURPOSE

This subfunction disables DMA accesses for all DMA channels which were

associated with a specific alternate map register set.

CALLING PARAMETERS

AX = 5B07h

Contains the Disable DMA on Alternate Map Register Set

subfunction.

BL = alternate register set number

Contains the number of the DMA register set for which all

operations are to be disabled.  If the alternate map

register set specified is zero, no action will be taken.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has returned the array size.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 9Ah   NON-RECOVERABLE

Alternate DMA register sets are supported, but the

alternate DMA register set specified is not supported.

130

AH = 9Ch   NON-RECOVERABLE

Alternate DMA register sets are not supported, and the DMA

register set specified is not zero.

AH = 9Dh   NON-RECOVERABLE

DMA register sets are supported, but the DMA register set

specified is either not defined or not allocated.

AH = 9Eh   NON-RECOVERABLE

Dedicated DMA channels are not supported.

AH = 9Fh   NON-RECOVERABLE

Dedicated DMA channels are supported, but the DMA channel

specified is not supported.

AH = A4h   NON-RECOVERABLE

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

DMA_reg_set                        DB ?

MOV  BL, DMA_reg_set

MOV  AX, 5B07h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

131

FUNCTION 28   ALTERNATE MAP REGISTER SET

DEALLOCATE DMA REGISTER SET SUBFUNCTION

Note...............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.  Refer to Function 30

for a description of how an operating system can enable or disable

this function.

PURPOSE

The Deallocate DMA Register Set subfunction deallocates the specified

DMA register set.

CALLING PARAMETERS

AX = 5B08h

Contains the Deallocate DMA Register Set subfunction.

BL = DMA register set number

Contains the number of the DMA register set which should

not be used for DMA operations any longer.  The DMA

register set would have been previously allocated and

enabled for DMA operations on a specific DMA channel.  If

the DMA register set specified is zero, no action will be

taken.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has deallocated the DMA register set.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = 9Ch   NON-RECOVERABLE

DMA register sets are not supported, and the DMA register

set specified is not zero.

132

AH = 9Dh   NON-RECOVERABLE

DMA register sets are supported, but the DMA register set

specified is either not defined or not allocated.

AH = A4h   NON-RECOVERABLE

The operating system has denied access to this function.

The function cannot be used at this time.

EXAMPLE

DMA_reg_set_num                    DB  ?

MOV  DMA_reg_set_num, BL

MOV  AX, 5B08h                     ; load function code

INT  67h                           ; call memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

133

FUNCTION 29   PREPARE EXPANDED MEMORY HARDWARE FOR WARM BOOT

PURPOSE

This function prepares the expanded memory hardware for an impending

warm boot.  This function assumes that the next operation that the

operating system performs is a warm boot of the system.  In general,

this function will effect the current mapping context, the alternate

register set in use, and any other expanded memory hardware

dependencies which need to be initialized at boot time.  If an

application decides to map memory below 640K, the application must

trap all possible conditions leading to a warm boot and invoke this

function before performing the warm boot itself.

CALLING PARAMETERS

AH = 5Ch

Contains the Prepare for Warmboot function.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The manager has prepared the expanded memory hardware for a

warm boot.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

EXAMPLE

MOV  AH, 5Ch                       ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

134

FUNCTION 30   ENABLE/DISABLE OS/E FUNCTION SET FUNCTIONS

ENABLE OS/E FUNCTION SET SUBFUNCTION

Note...............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.

PURPOSE

This subfunction provides an OS/E with the ability to enable all

programs or device drivers to use the OS/E specific functions.  The

capability is provided only for an OS/E which manages regions of

mappable conventional memory and cannot permit programs to use any of

the functions which affect mappable conventional memory regions, but

must be able to use these functions itself.  When an OS/E disables

these functions and a program attempts to use them, the memory manager

returns a status to the program indicating that the OS/E has denied

the program access to the function.  In other words, the functions

will not work when disabled.  However, all programs may use them when

enabled.  The OS/E (Operating System/Environment) functions this

subfunction enables are:

Function 26.  Get Expanded Memory Hardware Information.

Function 28.  Alternate Map Register Sets.

Function 30.  Enable/Disable Operating System Functions.

It appears contradictory that the OS/E can re-enable these functions

when the function which re-enables them is itself disabled.  An

overview of the process follows.

The memory manager enables all the OS/E specific functions, including

this one, when it is loaded.  The OS/E gets exclusive access to these

functions by invoking either of the Enable/Disable OS/E Function Set

subfunctions before any other software does.

On the first invocation of either of these subfunctions, the memory

manager returns an access_key which the OS/E must use in all future

invocations of either of these subfunctions.  The memory manager does

not require the access_key on the first invocation of the

Enable/Disable OS/E Function Set subfunctions.

ENABLE OS/E FUNCTION SET SUBFUNCTION

On all subsequent invocations, the access_key is required for either

the Enable or Disable OS/E Function Set subfunctions.  Since the

access_key is returned only on the first invocation of the

Enable/Disable OS/E Function Set subfunctions, and presumably the OS/E

is the first software to invoke this function, only the OS/E obtains a

copy of this key.  The memory manager must return an access key with a

random value, a fixed value key defeats the purpose of providing this

level of security for an OS/E.

CALLING PARAMETERS

135

AX = 5D00h

Contains the Enable OS/E Function Set subfunction.

BX,CX = access_key

Required on all function invocations after the first.  The

access_key value returned by the first function invocation

is required.

RESULTS

These results are valid only if the status returned is zero.

BX,CX = access_key

Returned only on the first function invocation, the memory

manager returns a random valued key which will be required

thereafter for the execution of this function.  On all

invocations after the first, this key is not returned.

Neither BX or CX is affected after the first time this

function is invoked.

REGISTERS MODIFIED

AX, BX, CX

STATUS

AH = 0     SUCCESSFUL

The operating system function set has been enabled.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = A4h   NON-RECOVERABLE

The operating system has denied access to this function.

The function cannot be used at this time.  The value of the

key which was passed to this function does not entitle the

program to execute this function.

EXAMPLE

First invocation

access_key                         DW 2 DUP (?)

MOV  AX, 5D00h                     ; load function code

136

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  access_key[0], BX

MOV  access_key[2], CX

All invocations after the first

access_key                         DW 2 DUP (?)

MOV  BX, access_key[0]

MOV  CX, access_key[2]

MOV  AX, 5D00h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to test handler on error

137

FUNCTION 30   ENABLE/DISABLE OS/E FUNCTION SET FUNCTIONS

DISABLE OS/E FUNCTION SET SUBFUNCTION

Note...............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.

PURPOSE

This subfunction provides an OS/E with the ability to disable all

programs or device drivers from using the OS/E specific functions.

The capability is provided only for an OS/E which manages regions of

mappable conventional memory and cannot permit programs to use any of

the functions which would affect mappable conventional memory regions.

When an OS/E disables these functions and a program attempts to use

them, the memory manager returns a status to the program indicating

that the OS/E has denied the program access to the function.  In other

words, the functions will not work when disabled.

The OS/E (Operating System) functions which are disabled by this

subfunction are:

Function 26.  Get Expanded Memory Hardware Information.

Function 28.  Alternate Map Register Sets.

Function 30.  Enable/Disable Operating System Functions.

CALLING PARAMETERS

AX = 5D01h

Contains the Disable OS/E Function Set subfunction.

BX,CX = access_key

Required on all function invocations after the first.  The

access_key value returned by the first function invocation

is required.

RESULTS

These results are valid only if the status returned is zero.

BX,CX = access_key

Returned only on the first function invocation, the memory

manager returns a random valued key which will be required

thereafter for the execution of this function.  On all

invocations after the first, this key is not returned.

Neither BX nor CX is affected after the first time this

function is invoked.

REGISTERS MODIFIED

AX, BX, CX

STATUS

138

AH = 0     SUCCESSFUL

The operating system function set has been disabled.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = A4h   NON-RECOVERABLE

The operating system has denied access to this function.

The function cannot be used at this time.  The value of the

key which was passed to this function does not entitle the

program to execute this function.

EXAMPLE

First Function invocation:

access_key                         DW 2 DUP (?)

MOV  AX, 5D01h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

MOV  access_key[0], BX

MOV  access_key[2], CX

All invocations after the first:

access_key                         DW 2 DUP (?)

MOV  BX, access_key[0]

MOV  CX, access_key[2]

MOV  AX, 5D01h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

139

FUNCTION 30   ENABLE/DISABLE OS/E FUNCTION SET FUNCTIONS

RETURN ACCESS KEY SUBFUNCTION

Note.............................................................

This function is for use by operating systems only.  The operating

system can disable this function at any time.

PURPOSE

This subfunction provides an OS/E with the ability to return the

access key to the memory manager.  Returning the access key to the

memory manager places the memory manager in the state it is in at

installation time (regarding the use of the OS/E function set and the

access key).  That is, access to the OS/E function set is enabled.

Upon execution of the next enable/disable OS/E function set

subfunction, the access key will once again be returned.

CALLING PARAMETERS

AX = 5D02h

Contains the Return Access Key subfunction.

BX,CX = access_key

Required on all function invocations.  The access_key value

returned by the first function invocation of the enable or

disable subfunctions is required.

REGISTERS MODIFIED

AX

STATUS

AH = 0     SUCCESSFUL

The access key has been returned to the memory manager.

AH = 80h   NON-RECOVERABLE

The manager detected a malfunction in the memory manager

software.

AH = 81h   NON-RECOVERABLE

The manager detected a malfunction in the expanded memory

hardware.

AH = 84h   NON-RECOVERABLE

The function code passed to the memory manager is not

defined.

AH = 8Fh   NON-RECOVERABLE

The subfunction parameter is invalid.

AH = A4h   NON-RECOVERABLE

Access to this function has been denied by the operating

system.  The function cannot be used at this time.  The

140

value of the key which was passed to this function does not

entitle the program to execute this function.

EXAMPLE

access_key                         DW 2 DUP (?)

MOV  BX, access_key[0]

MOV  CX, access_key[2]

MOV  AX, 5D02h                     ; load function code

INT  67h                           ; call the memory manager

OR   AH, AH                        ; check EMM status

JNZ  emm_err_handler               ; jump to error handler on error

141

APPENDIX A    FUNCTION AND STATUS CODE CROSS REFERENCE TABLES

This appendix contains two cross reference tables: one lists this

function codes and the status codes they return; the other lists the

status codes and the functions that return them.

TABLE A-1.  FUNCTION AND STATUS CODE CROSS REFERENCE

----------------------------------------------------------------------

Function   Status       Description

----------------------------------------------------------------------

40h   00h 80h 81h 84h   Get memory manager status

41h   00h 80h 81h 84h   Get Page Frame Segment Address

42h   00h 80h 81h 84h   Get Unallocated Page Count

43h   00h 80h 81h 84h   Allocate Pages

85h 87h 88h

44h   00h 80h 81h 83h   Map/Unmap Handle Page

84h 8Ah 8Bh

45h   00h 80h 81h 83h   Deallocate Pages

84h 86h

46h   00h 80h 81h 84h   Get EMM Version

47h   00h 80h 81h 83h   Save Page Map

84h 8Ch 8Dh

48h   00h 80h 81h 83h   Restore Page Map

84h 8Eh

49h                     Reserved

4Ah                     Reserved

4Bh   00h 80h 81h 84h   Get EMM Handle Count

4Ch   00h 80h 81h 83h   Get EMM Handle Pages

84h

4Dh   00h 80h 81h 84h   Get All EMM Handle Pages

4E00h 00h 80h 81h 84h   Get Page Map

8Fh

4E01h 00h 80h 81h 84h   Set Page Map

8Fh A3h

4E02h 00h 80h 81h 84h   Get & Set Page Map

8Fh A3h

4E03h 00h 80h 81h 84h   Get Size of Page Map Save Array

8Fh

142

4F00h 00h 80h 81h 84h   Get Partial Page Map

8Bh 8Fh A3h

4F01h 00h 80h 81h 84h   Set Partial Page Map

8Fh A3h

4F02h 00h 80h 81h 84h   Get Size of Partial Page Map Array

8Bh 8Fh

5000h 00h 80h 81h 83h   Map/Unmap Multiple Handle Pages

84h 8Ah 8Bh 8Fh   (Physical page mode)

5001h 00h 80h 81h 83h   Map/Unmap Multiple Handle Pages

84h 8Ah 8Bh 8Fh   (Segment address mode)

51h   00h 80h 81h 83h   Reallocate Pages

84h 87h 88h

5200h 00h 80h 81h 83h   Get Handle Attribute

84h 8Fh 91h

5201h 00h 80h 81h 83h   Set Handle Attribute

84h 8Fh 90h 91h

5202h 00h 80h 81h 84h   Get Handle Attribute Capability

8Fh

5300h 00h 80h 81h 83h   Get Handle Name

84h 8Fh

5301h 00h 80h 81h 83h   Set Handle Name

84h 8FH A1h

5400h 00h 80h 81h 84h   Get Handle Directory

8Fh

5401h 00h 80h 81h 84h   Search for Named Handle

8Fh A0h A1h

5402h 00h 80h 81h 84h   Get Total Handles

8Fh

5500h 00h 80h 81h 83h   Alter Page Map & Jump

84h 8Ah 8Bh 8Fh   (Physical page mode)

5501h 00h 80h 81h 83h   Alter Page Map & Jump

84h 8Ah 8Bh 8Fh   (Segment address mode)

5600h 00h 80h 81h 83h   Alter Page Map & Call

84h 8Ah 8Bh 8Fh   (Physical page mode)

5601h 00h 80h 81h 83h   Alter Page Map & Call

84h 8Ah 8Bh 8Fh   (Segment address mode)

5602h 00h 80h 81h 84h   Get Alter Page Map & Call Stack Space Size

8Fh

143

5700h 00h 80h 81h 83h   Move Memory Region

84h 8Ah 8Fh 92h

93h 94h 95h 96h

98h A2h

5701h 00h 80h 81h 83h   Exchange Memory Region

84h 8Ah 8Fh 93h

94h 95h 96h 97h

98h A2h

5800h 00h 80h 81h 84h   Get Mappable Physical Address Array

8Fh

5801h 00h 80h 81h 84h   Get Mappable Physical Address Array Entries

8Fh

5900h 00h 80h 81h 84h   Get Expanded Memory Hardware Information

8Fh A4h

5901h 00h 80h 81h 84h   Get Unallocated Raw Page Count

8Fh

5A00h 00h 80h 81h 84h   Allocate Standard Pages

85h 87h 88h 8Fh

5A01h 00h 80h 81h 84h   Allocate Raw Pages

85h 87h 88h 8Fh

5B00h 00h 80h 81h 84h   Get Alternate Map Register Set

8Fh A4h

5B01h 00h 80h 81h 84h   Set Alternate Map Register Set

8Fh 9Ah 9Ch 9Dh

A3h A4h

5B02h 00h 80h 81h 84h   Get Alternate Map Save Array Size

8Fh A4h

5B03h 00h 80h 81h 84h   Allocate Alternate Map Register Set

8Fh 9Bh A4h

5B04h 00h 80h 81h 84h   Deallocate Alternate Map Register Set

8Fh 9Ch 9Dh A4h

5B05h 00h 80h 81h 84h   Allocate DMA Register Set

8Fh 9Bh A4h

5B06h 00h 80h 81h 84h   Enable DMA on Alternate Map Register Set

8Fh 9Ah 9Ch 9Dh

9Eh 9Fh A4h

5B07h 00h 80h 81h 84h   Disable DMA on Alternate Map Register Set

8Fh 9Ah 9Ch 9Dh

9Eh 9Fh A4h

5B08h 00h 80h 81h 84h   Deallocate DMA Register Set

8Fh 9Ch 9Dh A4h

144

5Ch   00h 80h 81h 84h   Prepare Expanded Memory Hardware for

Warmboot

5D00h 00h 80h 81h 84h   Enable Operating System Function Set

8Fh A4h

5D01h 00h 80h 81h 84h   Disable Operating System Function Set

8Fh A4h

5D02h 00h 80h 81h 84h   Return Operating System Access Key

8Fh A4h

145

TABLE A-2.  STATUS AND FUNCTION CODE CROSS REFERENCE

----------------------------------------------------------------------

Status     Functions            Description

----------------------------------------------------------------------

00h   All                       The function completed normally.

80h   All                       The memory manager has detected a

malfunction in the expanded memory

software.

81h   All                       The memory manager has detected a

malfunction in the expanded memory

hardware.

82h   None                      This error code is not returned in

version 3.2 of the memory manager

or above.  In earlier versions of

the memory manager this code meant

a "busy" status.  This status

indicated that the memory manager

was already processing an expanded

memory request when the current

request was made and is unable to

process another request.  In

versions 3.2 of the memory manager

and above, the memory manager is

never "busy" and can always honor

requests.

83h   44h   45h   47h   48h     The memory manager can not find

4Ch   5000h 5001h 51h     the handle specified.

5200h 5201h 5300h 5301h

5500h 5501h 5600h 5601h

5700h 5701h

84h   All                       The function code passed to the

manager is not currently defined.

85h   43h   5A00h 5A01h         No handles are currently available.

All assignable handles are

currently in use.

86h   45h                       A mapping context restoration error

has been detected.  This error

occurs when a program attempts to

return a handle and there is still

a "mapping context" on the context

stack for the indicated handle.

87h   43h   51h   5A00h 5A01h   The number of total pages that are

available in the system is

insufficient to honor the request.

146

88h   43h   51h   5A00h 5A01h   The number of unallocated pages

currently available is insufficient

to honor the allocation request.

89h   43h                       Zero pages could not be assigned to

a handle.

8Ah   44h   5000h 5001h 5500h   The logical page to map into memory

5501h 5600h 5601h 5700h   is out of the range of logical

5701h                     pages which are allocated to the

handle.

8Bh   44h   4F00h 4F02h 5000h   One or more of the physical pages

5001h 5600h 5601h 5500h   is out of the range of allowable

5501h                     physical pages.

8Ch   47h                       The mapping register context save

area is full.

8Dh   47h                       The mapping register context stack

already has a context associated

with the handle.  The program has

attempted to save the mapping

register context when there was

already a context for the handle on

the stack.

8Eh   48h                       The mapping register context stack

does not have a context associated

with the handle.  The program has

attempted to restore the mapping

register context when there was no

context for the handle on the

stack.

8Fh   All                       The subfunction parameter passed to

the function requiring a

subfunction code is not defined.

90h   5201h                     The attribute type is undefined.

91h   5200h 5201h               The system configuration does not

support non-volatility.

92h    5700h                    The source and destination expanded

memory regions have the same handle

and overlap.  This is valid for a

move.  The move has been completed

and the destination region has a

full copy of the source region.

However, at least a portion of the

source region has been overwritten

by the move.

93h   5700h 5701h               The length of the specified source

or destination expanded memory

region exceeds the length of the

147

expanded memory region allocated to

the specified source or destination

handle.  There are insufficient

pages allocated to this handle to

move/exchange a region of the size

specified.

94h   5700h 5701h               The conventional memory region and

expanded memory region overlap.

This is invalid, the conventional

memory region cannot overlap the

expanded memory region.

95h   5700h 5701h               The offset within the logical page

exceeds the length of the logical

page.  The initial source or

destination offsets within an

expanded memory region must be

between 0 and the (length of a

logical page - 1) or 16383 (3FFFh).

96h   5700h 5701h               Region length exceeds 1M-byte

limit.

97h   5701h                     The source and destination expanded

memory regions have the SAME handle

AND overlap.  This is invalid; the

source and destination expanded

memory regions cannot have the same

handle and overlap when they are

being exchanged.

98h   5700h 5701h               The memory source and destination

types are undefined/not supported.

9Ah   5B01h 5B06h 5B07h         Alternate map register sets are

supported, but the alternate map

register set specified is not

supported.

9Bh   5B03h 5B05h               Alternate map/DMA register sets are

supported.  However, all alternate

map/DMA register sets are currently

allocated.

9Ch   5B01h 5B04h 5B06h 5B07h   Alternate map/DMA register sets are

5B08h                     not supported, and the alternate

map/DMA register set specified is

not zero.

9Dh   5B01h 5B04h 5B06h 5B07h   Alternate map/DMA register sets are

5B08h                     supported, but the alternate map

register set specified is not

defined, not allocated, or is the

currently allocated map register

set.

148

9Eh   5B06h 5B07h               Dedicated DMA channels are not

supported.

9Fh   5B06h 5B07h               Dedicated DMA channels are

supported.  But the DMA channel

specified is not supported.

A0h   5401h                     No corresponding handle value could

be found for the handle name

specified.

A1h   5301h 5401h               A handle with this name already

exists.  The specified handle was

not assigned a name.

A2h   5700h 5701h               An attempt was made to "wrap

around" the 1M-byte address space

during the move/exchange.  The

source starting address together

with the length of the region to be

moved/exchanged exceeds 1M-byte.  No

data was moved/exchanged.

A3h   4E01h 4E02h 4F00h 4F01h   The contents of the data  structure

5B01h                     passed to the function have either

been corrupted or are meaningless.

A4h   5900h 5B00h 5B01h 5B02h   The operating system has denied

5B03h 5B04h 5B05h 5B06h   access to this function.  The

5B07h 5B08h 5D00h 5D01h   function cannot be used at this

5D02h                     time.

149

APPENDIX B

TESTING FOR THE PRESENCE OF THE EXPANDED MEMORY MANAGER

Before an application program can use the Expanded Memory Manager, it

must determine whether DOS has loaded the manager.  This appendix

describes two methods your program can use to test for the presence of

the memory manager and how to choose the correct one for your

situation.

The first method uses the DOS "open handle" technique; the second

method uses the DOS "get interrupt vector" technique.

WHICH METHOD SHOULD YOUR PROGRAM USE?

The majority of application programs can use either the "open handle"

or the "get interrupt vector" method.  However, if your program is a

device driver or if it interrupts DOS during file system operations,

you must use only the "get interrupt vector" method.

Device drivers execute from within DOS and can't access the DOS file

system; programs that interrupt DOS during file system operations have

a similar restriction.  During their interrupt processing procedures,

they can't access the DOS file system because another program may be

using the system.

Since the "get interrupt vector" method doesn't require the DOS file

system, you must use it for these types of programs.

THE "OPEN HANDLE" TECHNIQUE

Most application programs can use the DOS "open handle" technique to

test for the presence of the memory manager.  This section describes

how to use the technique and gives an example.

Caution..........................................................

Don't use this technique if your program is a device driver or if it

interrupts DOS during file system operations.  Use the "get interrupt

vector" technique described later in this chapter.

USING THE "OPEN HANDLE" TECHNIQUE

This section describes how to use the DOS "open handle" technique to

test for the presence of the memory manager. Follow these steps in

order:

1.  Issue an "open handle" command (DOS function 3Dh) in "read only"

access mode (register AL = 0).  This function requires your

program to point to an ASCII string which contains the path name

of the file or device in which you're interested (register set

DS:DX contains the pointer).  In this case the file is actually

the name of the memory manager.

You should format the ASCII string as follows:

ASCII_device_name   DB   "EMMXXXX0", 0

150

The ASCII codes for the capital letters EMMXXXX0 are terminated by

a byte containing a value of zero.

2.  If DOS returns no error status code, skip Steps 3 and 4 and go to

Step 5.  If DOS returns a "Too many open files" error status code,

go to Step 3. If DOS returns a "File/Path not found" error status

code, skip Step 3 and go to Step 4.

3.  If DOS returns a "Too many open files" (not enough handles),

status code, your program should invoke the "open file" command

before it opens any other files.  This will guarantee that at

least one file handle will be available to perform the function

without causing this error.

After the program performs the "open file" command, it should

perform the test described in Step 6 and close the "file handle"

(DOS function 3Eh).  Don't keep the manager "open" after this

status test is performed since "manager" functions are not

available thru DOS.  Go to Step 6.

4.  If DOS returns a "File/Path not found", the memory manager is not

installed.  If your application requires the memory manager, the

user will have to reboot the system with a disk containing the

memory manager and the appropriate CONFIG.SYS file before

proceeding.

5.  If DOS doesn't return an error status code you can assume that

either a device with the name EMMXXXX0 is resident in the system,

or a file with this name is on disk in the current disk drive.  Go

to Step 6.

6.  Issue an "I/O Control for Devices" command (DOS function 44h) with

a "get device information" command (register AL = 0h).  DOS

function 44h determines whether EMMXXXX0 is a device or a file.

You must use the file handle (register BX) which you obtained in

Step 1 to access the "EMM" device.  This function returns the

"device information" in a word (register DX).  Go to step 7.

7.  If DOS returns any error status code, you should assume that the

memory manager device driver is not installed.  If your

application requires the memory manager, the user will have to

reboot the system with a disk containing the memory manager and

the appropriate CONFIG.SYS file before proceeding.

8.  If DOS didn't return an error status, test the contents of bit 7

(counting from 0) of the "device information" word (register DX)

the function returned.  Go to Step 9.

9.  If bit 7 of the "device information" word contains a zero, then

EMMXXXX0 is a file, and the memory manager device driver is not

present.  If your application requires its presence, the user will

have to reboot the system with a disk containing the memory

manager and the appropriate CONFIG.SYS file before proceeding.  If

bit 7 contains a one, then EMMXXXX0 is a device. Go to Step 10.

151

10. Issue an "I/O Control for Devices" command (DOS function 44h) with

a "get output status" command (register AL = 7h).

You must use the file handle you obtained in Step 1 to access the

"EMM" device (register BX).  Go to Step 11.

11. If the expanded memory device driver is "ready," the memory

manager passes a status value of "0FFh" in register AL.  The

status value is "00h" if the device driver is "not ready."

If the memory manager device driver is "not ready" and your

application requires its presence, the user will have to reboot

the system with a disk containing the memory manager and the

appropriate CONFIG.SYS file before proceeding.

If the memory manager device driver is "ready", go to Step 12.

12. Issue a "Close File Handle" command (DOS function 3Eh) to close

the expanded memory device driver.  You must use the file handle

you obtained in Step 1 to close the "EMM" device (register BX)

AN EXAMPLE OF THE "OPEN HANDLE" TECHNIQUE

The following procedure is an example of the "open handle" technique

outlined in the previous section.

;---------------------------------------------------------------;

;   The following procedure tests for the presence of the EMM   ;

;   in the system.  It returns the CARRY FLAG SET if the EMM is ;

;   present.  If the EMM is not present, this procedure returns ;

;   the CARRY FLAG CLEAR.                                       ;

;---------------------------------------------------------------;

first_test_for_EMM  PROC  NEAR

PUSH DS

PUSH CS

POP  DS

MOV  AX, 3D00h                     ; issue device open in

LEA  DX, ASCII_device_name         ; read only mode

INT  21h

JC   first_test_for_EMM_error_exit ; test for error during open

MOV  BX, AX                        ; get file handle returned by DOS

MOV  AX, 4400h                     ; issue IOCTL get device info

INT  21h

JC   first_test_for_EMM_err_exit   ; test for get device info error

TEST DX, 0080h                     ; test to determine

JZ   first_test_for_EMM_err_exit   ; if ASCII_device_name

; is a device or a file

MOV  AX, 4407h                     ; issue "IOCTL"

INT  21h

JC   first_test_for_EMM_error_exit ; test for IOCTL error

PUSH AX                            ; save "IOCTL" status

MOV  AH, 3Eh                       ; issue "close

INT  21h                           ; file handle"

152

POP  AX                            ; restore "IOCTL" status

CMP  AL, 0FFh                      ; test for "device ready" status

JNE  first_test_for_EMM_error_exit ; returned by the driver

first_test_for_EMM_exit:

POP  DS                            ; EMM is present in the system

STC

RET

first_test_for_EMM_error_exit:

POP  DS                            ; EMM is NOT present in the system

CLC

RET

ASCII_device_name                  DB  "EMMXXXX0", 0

first_test_for_EMM ENDP

153

THE "GET INTERRUPT VECTOR" TECHNIQUE

Any type of program can use the DOS "get interrupt vector" technique

to test for the presence of the memory manager.  This section

describes how to use the technique and gives an example.

Caution..........................................................

Be sure to use this technique (and not the "open handle technique) if

your program is a device driver or if it interrupts DOS during file

system operations.

USING THE "GET INTERRUPT VECTOR" TECHNIQUE

This section describes how to use the DOS "get interrupt vector"

technique to test for the presence of the memory manager.  Follow

these steps in order:

1.  Issue a "get vector" command (DOS function 35h) to obtain the

contents of interrupt vector array entry number 67h (addresses

0000:019C thru 0000:019F).

The memory manager uses this interrupt vector to perform all

manager functions.  The Offset portion of this interrupt service

routine address is stored in the word located at address

0000:019Ch;  the Segment portion is stored in the word located at

address 0000:019Eh.

2.  Compare the "device name field" with the contents of the ASCII

string which starts at the address specified by the segment

portion of the contents of interrupt vector address 67h and a

fixed offset of 000Ah.  If DOS loaded the memory manager at boot

time this name field will have the name of the device in it.

Since the memory manager is implemented as a character device

driver, its program origin is 0000h.  Device drivers are required

to have a "device header" located at the program origin.  Within

the "device header" is an 8 byte "device name field."  For a

character mode device driver this name field is always located at

offset 000Ah within the device header.  The device name field

contains the name of the device which DOS uses when it references

the device.

If the result of the "string compare" in this technique is

positive, the memory manager driver is present.

AN EXAMPLE OF THE "GET INTERRUPT VECTOR" TECHNIQUE

The following procedure is an example of the "get interrupt vector"

technique outlined in the previous section.

;---------------------------------------------------------------;

;   The following procedure tests for the presence of the EMM   ;

;   in the system.  It returns the CARRY FLAG SET if the EMM is ;

;   present.  If the EMM is not present, this procedure returns ;

;    the CARRY FLAG CLEAR.                                      ;

;---------------------------------------------------------------;

154

second_test_for_EMM                PROC    NEAR

PUSH DS

PUSH CS

POP  DS

MOV  AX, 3567h                     ; issue "get interrupt vector"

INT  21h

MOV  DI, 000Ah                     ; use the SEGMENT in ES returned by

; DOS, place the "device name

; field" OFFSET in DI

LEA  SI, ASCII_device_name         ; place the OFFSET of the device

; name string in SI, the SEGMENT

; is already in DS

MOV  CX, 8                         ; compare the name strings

CLD

REPE CMPSB

JNE  second_test_for_EMM_error_exit

second_test_for_EMM_exit:

POP  DS                            ; EMM is present in the system

STC

RET

second_test_for_EMM_error_exit:

POP  DS                            ; EMM is NOT present in the system

CLC

RET

ASCII_device_name                  DB "EMMXXXX0"

second_test_for_EMM                ENDP

155

APPENDIX C

EXPANDED MEMORY MANAGER IMPLEMENTATION GUIDELINES

In addition to the functional specification, the expanded memory

manager should provide certain resources.  The following guidelines

are provided so required resources are present in expanded memory

managers which comply with this version of the LIM specification.

o   The amount of expanded memory supported:

Up to a maximum of 32M bytes of expanded memory should be

supported.

o   The number of handles supported:

The maximum number expanded memory handles provided should be 255,

the minimum should be 64.

o   Handle Numbering:

Although a handle is a word quantity, there is a maximum of 255

handles, including the operating system handle.  This

specification defines the handle word as follows: the low byte of

the word is the actual handle value, the high byte of the handle

is set to 00 by the memory manager.  Previous versions of this

specification did not specify the value of handles.

o   New handle type: Handles versus Raw Handles:

The difference between a raw handle and a regular handle is

slight. If you use Function 27 to "Allocate raw pages to a

handle", what is returned in DX is termed a raw handle. The raw

handle does not necessarily refer to 16 Kbyte pages. Instead it

refers to the "raw" page size, which depends on the expanded

memory hardware.

An application program can use Function 26 to find the raw page

size, and by using the raw handle Function 27 returns, it can

access them with the finer resolution that a particular expanded

memory board may allow.

On the other hand, applications which use Function 4 to "allocate

pages to handle" receive a handle which always refers to 16K-byte

pages. On expanded memory boards with smaller raw pages, the EMM

driver will allocate and maintain the number of raw pages it takes

to create a single composite 16K-byte page.  The difference

between the expanded memory boards raw page size and the 16K-byte

LIM page size is transparent to the application when it is using a

handle obtained with Function 4.

The memory manager must differentiate between pages allocated to

handles and pages allocated to raw handles.  The meaning of a call

to the driver changes depending on whether a handle or a raw

handle is passed to the memory manager. If, for example, a handle

is passed to Function 18 (Reallocate), the memory manager will

156

increase or decrease the number of 16K-byte pages allocated to the

handle.  If Function 18 is passed a raw handle, the memory manager

will increase or decrease the number of raw (non-16K-byte) pages

allocated to the raw handle.  For LIM standard boards, there is no

difference between pages and raw pages.

o   The system Raw Handle (Raw Handle = 0000):

For expanded memory boards that can remap the memory in the lower

640K-byte address space, managing the pages of memory which are

used to fill in the lower 640K can be a problem.  To solve this

problem, the memory manager will create a raw handle with a value

of 0000 when DOS loads the manager.  This raw handle is called the

system handle.

At power up, the memory manager will allocate all of the pages

that are mapped into the lower 640K bytes to the system handle.

These pages should be mapped in their logical order.  For example,

if the system board supplies 256K bytes of RAM, and the 384K bytes

above it is mappable, the system handle should have its logical

page zero mapped into the first physical page at 256K, its logical

page one mapped into the next physical page, and so on.

The system handle should deal with raw pages.  To release some of

these pages so application programs can use them, an operating

system could decrease the number of pages allocated to the system

handle with the "Reallocate" function.  Invoking the "Deallocate"

function would decrease the system handle to zero size, but it

must not deallocate the raw handle itself.  The "Deallocate"

function treats the system handle differently than it treats other

raw handles.  If the operating system can ever be "exited" (for

example, the way Windows can be exited), it must increase the size

of the system handle back to what is needed to fill 640K and map

these logical pages back into physical memory before returning to

DOS.

157

There are two functional special cases for this handle:

-  The first special case deals with Function 4 (Allocate

Pages).  This function must never return zero as a handle

value.  Applications must always invoke Function 4 to

allocate pages and obtain a handle which identifies its

pages.  Since Function 4 will never return a handle value

of zero, an application will never gain access to this

special handle.

-  The second special case deals with Function 6 (Deallocate

Pages). If the operating system uses Function 6 to

deallocate the pages which are allocated to the system

handle, the pages will be returned to the manager for use,

but the handle will not be available for reassignment.

The manager should treat a "deallocate pages" function

request for this handle the same as a "reallocate pages"

function request, where the number of pages to reallocate

to this handle is zero.

o   Terminate and Stay Resident (TSR) Program Cooperation In order for

TSR's to cooperate with each other and with other applications,

TSR's must follow this rule:  a program may only remap the DOS

partition it lives in.

This rule applies at all times, even when no expanded memory is

present.

o   Accelerator Cards: To support generic accelerator cards, the

support of Function 34, as defined by AST, is encouraged.

158

APPENDIX D

OPERATING SYSTEM/ENVIRONMENT USE OF FUNCTION 28

All expanded memory boards have a set of registers that "remember" the

logical to physical page mappings.  Some boards have extra (or

alternate) sets of these mapping registers.  Because no expanded

memory board can supply an infinite number of alternate map register

sets, this specification provides a way to simulate them using

Function 28 (Alternate Map Register Set).

EXAMPLES

For the examples in this section, assume the hardware supports

alternate map register sets.  First Windows is brought up, then

"Reversi" is started.  Then control is switched back to the MS-DOS

Executive.  For this procedure, here are the calls to the expanded

memory manager:

EXAMPLE 1

Allocate alt reg set               ; Start up the MS-DOS

(for the MS-DOS executive)         ; Executive

Set alt reg set

(for MS-DOS Executive)

Allocate alt reg set               ; Start up Reversi

(for Reversi)

Set alt reg set

(for Reversi)

Map pages

(for Reversi)

Set alt reg set                    ; Switch back to MS-DOS

(for MS-DOS Executive)             ; Executive

Notice this procedure needed no "get" calls because the register set

contained all the information needed to save a context.  However,

using "Get" calls would have no ill effects.

159

EXAMPLE 2

Allocate alt reg set               ; Start up MS-DOS

(for MS-DOS Executive)             ; Executive

Set alt reg set

(for MS-DOS Executive)

Get alt reg set

(for MS-DOS Executive)

Allocate alt reg set               ; Start up Reversi

(for Reversi)

Set alt reg set

(for Reversi)

Map pages

(for Reversi)

Get alt reg set

(for Reversi)

Set alt reg set                    ; Switch back to MS-DOS

(for MS-DOS Executive)             ; Executive

The important point to follow is that a Set must always precede a Get.

The model of Set then Get is the inverse of what interrupt handlers

use, which is Get then Set (Get the old map context and Set the new

one).  Another crucial point is that an alternate map register set

must have the current mapping when allocated; otherwise, the Set will

create chaos.

What happens if this is simulated in software?  The same Set and Get

model applies.  The main difference is where the context is saved.

Since the allocate call is dynamic and there is no limit on the number

of sets allocated, the OS/E must supply the space required.  Device

drivers cannot allocate space dynamically, since the request would

fail.  If the Allocate register set call returns a status indicating

the alternate map register sets aren't supported, the OS/E must

allocate space for the context.  It must also initialize the context

using Function 15.  At that point it can do the Set, passing a pointer

to the map context space.  On the Get call, the EMM driver is to

return a pointer to the same context space.

EXAMPLE 3

Allocate alt reg set               ; Start up MS-DOS

(for the MS-DOS executive)         ;Executive

Get Page Map

(for the MS-DOS executive)

Set alt reg set

(for MS-DOS Executive)

160

Allocate alt reg set               ; Start up Reversi

(for Reversi)

Set alt reg set

(for Reversi)

Map pages

(for Reversi)

Get Page Map

(for Reversi)

Set alt reg set                    ; Switch back to MS-DOS

(for MS-DOS Executive)             ; Executive

161

GLOSSARY

The following terms are used frequently in this specification:

Allocate

To reserve a specified amount of expanded memory pages.

Application Program

An application program is the program you write and your customer

uses.  Some categories of application software are word

processors, database managers, spreadsheet managers, and project

managers.

Conventional Memory

The memory between 0 and 640K bytes, address range 00000h thru

9FFFFh.

Deallocate

To return previously allocated expanded memory to the memory

manager.

EMM

See Expanded Memory Manager.

Expanded Memory

Expanded memory is memory outside DOS's 640K-byte limit (usually

in the range of C0000h through EFFFFH).

Expanded Memory Manager (EMM)

A device driver that controls the interface between DOS

application programs and expanded memory.

Extended Memory

The 15M-byte address range between 100000h thru FFFFFFh available

on an 80286 processor when it is operating in protected virtual

address mode.

Handle

A value that the EMM assigns and uses to identify a block of

memory requested by an application program.  All allocated

logical pages are associated with a particular handle.

Logical Page

The EMM allocates expanded memory in units (typically 16K-bytes)

called logical pages.

Mappable Segment

A 16K-byte region of memory which can have a logical page mapped

at it.

Map Registers

The set of registers containing the current mapping context of

the EMM hardware.

Mapping

The process of making a logical page of memory appear at a

physical page.

162

Mapping Context

The contents of the mapping registers at a specific instant.

This context represents a map state.

Page Frame

A collection of 16K-byte contiguous physical pages from which an

application program accesses expanded memory.

Page Frame Base Address

A page frame base address is the location (in segment format) of

the first byte of the page frame.

Physical Page

A physical page is the range of memory addresses occupied by a

single 16K-byte page.

Raw Page

The smallest unit of mappable memory that an expanded memory

board can supply.

Resident Application Program

A resident application program is loaded by DOS, executes, and

remains resident in the system after it returns control to DOS.

This type of program occupies memory and is usually invoked by

the operating system, an application program, or the hardware.

Some examples of resident application programs are RAM disks,

print spoolers, and "pop-up" desktop programs.

Status Code

A code that an EMM function returns which indicates something

about the result of running the function.  Some status codes

indicate whether the function worked correctly and others may

tell you something about he expanded memory hardware or software.

Transient Application

A transient application program is loaded Program by DOS,

executes, and doesn't remain in the system after it returns

control to DOS.  After a transient application program returns

control to DOS, the memory it used is available for other

programs.

Unmap

To make a logical page inaccessible for reading or writing.

163