ADDPORT.DOC

This program was written to address a few very specific problems.

1. Certain makes of BIOS do not support beyond COM2. This means

that at the power-up equipment check they do not add COM3 and

COM4 to the low-memory equipment list and/or do not add the

appropriate base addresses to the BIOS data area even though

you may have a modem or I/O card configured as COM3 or COM4.

2. This same problem seems to occur occasionally as a result of

conflicts between various boards.

3. The equipment list may be incorrect if an internal modem is

installed as COM2 (or COM3 or COM4) with no COM1 serial port

in the computer.

The result of all these problems is that any serial I/O involving DOS

redirection will not work properly. For instance, one may use COM3

successfully with a terminal program until "shelling out of Host

Mode," which often involves DOS CTTY redirection. Another symptom

would be a "WRITE FAULT ERROR WRITING DEVICE COMx" following simple

redirection such as "DIR >COMx".

ADDPORT will write information about your serial ports into the

appropriate low-memory areas. It may be used to make your system

aware of your specific serial hardware.

Syntax: ADDPORT <COMx> <BASE_ADDRESS_IN_HEX>

example: addport com3 3e8

If the COM port is already listed in the low-memory equipment list,

ADDPORT will simply change the base address to what you specify in

the second argument to the command. Typing ADDPORT alone will show

what ports are currently in the equipment list along with their base

addresses.

A typical use of ADDPORT would be to put the line ADDPORT COM3 3E8 in

the autoexec.bat file for BIOS versions that do not support COM3.

COMMON SERIAL PORT BASE ADDRESSES

COM1 - 3F8        the addresses for COM1 and COM2

COM2 - 2F8        are pretty much a standard

COM3 - 3E8        these addresses for COM3 and COM4 are

COM4 - 2E8        are common, though it's sometimes reversed

PS/2 SERIAL PORT BASE ADDRESSES

COM1 - 3F8

COM2 - 2F8

COM3 - 3220

COM4 - 3228

Currently ADDPORT does not support beyond COM4.