Refer to the User's Guide for more details about installation.

Prior to installation, make a backup copy of this Software Disk using the

DISKCOPY command.  Store the master disk in a safe place and use the copy

for installation.

Do not write-protect the backup copy of the Software Disk used for the

installation.

Before installation, confirm the type of your computer, whether it is

IBM PC/AT compatible or PC/XT compatible.

1. After booting, insert the Software Disk into drive A.

2. Type a: ENTER to change to drive A.

3. Type INSTALL and press ENTER at the DOS prompt.

The installation utility starts. Press any key until the following

messages appear on the screen.

Type the following and press ENTER at the DOS prompt.

CH1 AT [Sub_Dir]           ; Install to the hard disk drive C:\[Sub_Dir]

on an IBM PC/AT compatible computer.

CH1 XT [Sub_Dir]           ; Install to the hard disk drive C:\[Sub_Dir]

on an IBM PC/XT compatible computer.

[Sub_Dir] is optional. If you omit this, the installation program installs the

device driver and extension program in the root directory of your hard disk.

To install the files in a subdirectory on the hard drive, specify the name of

the subdirectory.  For example: CH1 AT CDROM

The installation does the following:

1. Makes a subdirectory CDROM on the hard disk.

2. Makes backup files of AUTOEXEC.BAT and CONFIG.SYS with extension "BAK"

in the subdirectory.

3. Adds the executable path to the C:\CDROM in AUTOEXEC.BAT file.

path=C:\CDROM;%path%

MSCDEX /D:MSCD001 /M:6 /V

4. Appends \CDROM\ before the file names in CONFIG.SYS file.

LASTDRIVE=Z

DEVICE = C:\CDROM\TANDYCDU.SYS /D:MSCD001 /U:1 /B:340 /M:H /Q:* /T:*

CUSTOMIZATION

MSCDEX has some parameters in the AUTOEXEC.BAT and CONFIG.SYS files.

After installation, you can customize them if necessary. See "Manually

Editing Config.sys" and "Manually Editing Autoexec.bat" in your User's Guide.