Appendix   A

Jumper and Connector Summary

CPU Type

| CPU Type | JP5 | JP13 |
| --- | --- | --- |
| P54C | 1-2, 3-4, 5-6 | 1-2, 3-4, 5-6 |

CPU Voltage

| CPU Voltage | JP18 | JP19 | JP20 |
| --- | --- | --- | --- |
| VRE Type | Closed | Open | Open |
| STD Type | Open | Closed | Open |

CPU Frequency

| CPU Frequency | JP3 | JP4 | JP14 | JP15 |
| --- | --- | --- | --- | --- |
| 75 MHz | 2-3 | 2-3 | 2-3 | 2-3 |
| 90 MHz | 2-3 | 1-2 | 2-3 | 2-3 |
| 100 MHz | 1-2 | 2-3 | 2-3 | 2-3 |
| 120 MHz | 2-3 | 1-2 | 2-3 | 1-2 |
| 133 MHz | 1-2 | 2-3 | 2-3 | 1-2 |
| 150 MHz | 2-3 | 1-2 | 1-2 | 1-2 |
| 166 MHz | 1-2 | 2-3 | 1-2 | 1-2 |
| 200 MHz | 1-2 | 2-3 | 1-2 | 2-3 |

L2 Cache Mode

| Cache Mode | JP1 |
| --- | --- |
| Linear mode (Cyrix) | 1-2 |
| Interleave mode (Intel) | 2-3 |

Flash ROM Type

| Flash ROM | JP8 |
| --- | --- |
| 12V | 1-2 |
| 5V (default) | 2-3 |

Super I/O Controller

| Function | JP16 |
| --- | --- |
| Enabled | 1-2 |
| Disabled | 2-3 |

ECP DMA Channel

| ECP DMA Channnel | JP6 | JP7 |
| --- | --- | --- |
| 3 | 1-2 | 1-2 |
| 1 | 2-3 | 2-3 |

CMOS

| Function | JP12 |
| --- | --- |
| Normal | 1-2 |
| Clear CMOS | 2-3 |

PS/2 Mouse

| Function | JP10 |
| --- | --- |
| Enabled | 1-2 |
| Disabled | 2-3 |

Onboard Connectors

| Connector | Function |
| --- | --- |
| CON2 | AT-keyboard |
| CON3 | Power |
| CON4 | PS/2 mouse header |
| CON5 | COM1 |
| CON6 | Floppy disk |
| CON7 | IDE1 |
| CON8 | COM2 |
| CON9 | Parallel |
| CON10 | IDE2 |
| CON11 | Multifunction connector |
| JP2 | Reserved |
| JP9 | Reserved |
| JP11 | Reserved |
| JP17 | Fan |
| JP21 | IR |
| JP22 | IDE LED |
| JP23 | Reserved |
| JP24 | Reserved |
| JP25 | Reserved |
| USB | USB interface |