INSTALL.DOC

Kingston Technology Company

INSTALLATION GUIDE

Novell Netware Ver.4.0x Server LAN Driver

for

EtheRx PCI 10/100 Fast Ethernet Adapter (KNE110TX)

Copyright (C) 1996 Kingston Technology Company.

All rights reserved.

Introduction:

This installation guide provides instructions for installing the

Novell Netware Ver. 4.0x Server LAN driver.

Driver Installation Procedure:

I. Extracting the driver files from the compressed file

When you use the Generic or Custom install options in QStart, the

files associated with the network operating system environment you

choose will be displayed on the screen and copied to the default

destination directory (C:\NWSERVER) which is also the default

directory for the Netware 4.0x system files. If you changed the

default directory when you loaded Netware 4.0x, please make sure when

you run QStart, you specify this same directory.

1.

Make sure the EtheRx PCI adapter is properly installed in

your computer.

2.

Insert the Kingston Drivers diskette into your floppy

drive A:.

3.

Type A:QSTART[Enter] to run QStart.

4.

Select 'Generic Install', after the adapter test completed,

select [Continue].

5.

Select 'Novell Netware 386 Ver.4.0x Server' in the Driver

screen.

6.

Change the default destination directory if desired then click

'Copy'.

7.

After QStart copied the files to the destination directory,

click 'Exit' to get back to DOS.

II. Installing Novell Server Driver

1.

Make sure the EtheRx PCI adapter is properly installed in

your computer.

2.

Start your Novell file server as you usually do.

3.

At the system console screen, type the following command to load

modules needed.

: Load C:\NWSERVER\KTC110 FRAME=<frame type> SLOT=1

Note:

SLOT=1 for the 1st adapter, 2 for 2nd and so on

4.

Bind the driver to the IPX protocol by typing:

: Bind ipx to KTC110 NET=<network number>

5.

To load the driver automatically, put command LOAD and BIND

described above into your AUTOEXEC.NCF file.

Type 'LOAD INSTALL' at the server prompt to access the install

utility. Open the "NCF FILES OPTIONS" menu to access "EDIT

AUTOEXEC.NCF file"

: Load C:\NWSERVER\KTC110 FRAME=<frame type> SLOT=1

: Bind ipx to KTC110 NET=<network number>

where C:\NWSERVER is the default directory where QStart

loaded the driver files. If you specified some other

directory name when you ran QStart, then you should use the

directory you specified.

6.

The installation is now complete.

Kingston Technology, EtheRx, and QStart are registered

trademarks of Kingston Technology Company.  All others are

trademarks or registered trademarks of their respective holders.