Introduction to PCTIME

----------------------

PCTIME is an easy-to-use software program that sets your computer clock to

agree with precise time from the National Institute of Standards and

Technology (NIST).  Since 1988, NIST has offered an Automated Computer Time

Service (ACTS) so users can keep their computer clocks on time.  PCTIME makes

a phone call to ACTS and automatically sets your computer's clock.

To use PCTIME, you must have a modem (either internal or external) installed

on your computer.

Installing PCTIME

-----------------

To use PCTIME, copy the PCTIME.EXE file to the disk or directory where you

want it to be stored.  Then, type PCTIME and press <ENTER> to start the

software.

Configuring PCTIME

------------------

The first thing to do when you use PCTIME is to make sure that it is set up

properly to run on your computer.  To do so, select "Change Configuration"

from the menu.  The setup screen will appear on your monitor.

Complete the following steps to configure PCTIME.

1)   SELECT THE PHONE NUMBER

The top line of the PCTIME configuration screen lists the

phone number for ACTS.  This number is:

1-303-494-4774

This number will work for most users in the United States.

However, some users will have to edit the phone number to

make it work.  For example, if you are within the 303 area

code (in the Denver, Colorado area), you can remove the 1

and the area code.  Change the number to look like this:

494-4774

If you are outside the Denver area but need an access code

to dial out of your building, you also need to change the

number.  For example, if you need to dial 9 to get an

outside line, change the number to look like this:

9-1-303-494-4774

2)   MODEM COMMANDS

This field allows you to add modem commands to the modem

initialization string.  PCTIME initializes the modem using

the standard "ATZ" sequence.

This field is blank by default.  You should try using PCTIME

without adding anything to this field.  It it works, you have

nothing to worry about.  If it doesn't work, you may need to

consult the manual that came with your modem and add commands

to this field in order to get your modem to connect properly.

3)   SELECT TONE OR PULSE DIALING

After you have selected the correct phone number, you

need to select either tone or pulse dialing.  Most phone

systems support tone dialing, so this is the default setting.

If you have to use pulse dialing, enter a P for pulse.

4)   SELECT THE COMMUNICATIONS PORT

PCTIME needs to know what communications port your

modem is using.  You can use a modem connected to ports

1 through 4.  The default is 1.  If you using a different

port, enter the appropriate number.

5)   SELECT THE TRANSMISSION SPEED

PCTIME can call ACTS using either 300, 1200, or 9600 baud

(the highest speed supported by ACTS).  The default speed is

9600 baud.  If you want to call ACTS at 300 baud, enter a

L (for low speed).  To call ACTS at 1200 baud, enter M for

medium speed.  Many users will have modems capable

of speed higher than 9600 baud (28.8 modems, for

example).  These modems will automatically switch down in

speed when they call ACTS.

6)   WRITE TIME DIFFERENCES TO FILE

Each time PCTIME sets your clock, it measures how far off

the clock was when it was set.  If you wish, you can save

this information to a file.  To do so, enter a Y.  A file

named PCTIME.DIF will be created in the same directory

as PCTIME when you call ACTS.  This file will contain

information about how far your clock was off each time it

was set.

PCTIME has a menu item called "VIEW CLOCK

PERFORMANCE" that displays the PCTIME.DIF file if it

exists and shows you how well your clock is performing.

7)   SELECT YOUR LOCAL TIME ZONE

Select your local time zone by entering a number from 0 to

9.  PCTIME will set your clock to agree with the time in

that zone.  For example, if you are in New York City,

select 1 so that your clock is set to Eastern Time.

PCTIME automatically adjusts for Daylight and Standard

Time.

When you have everything entered the way you want it, press <END> to save the

configuration to disk.

Running PCTIME from a batch file

--------------------------------

After PCTIME has been configured, you can add this command to your

AUTOEXEC.BAT file to make it run automatically and exit after it sets the

clock:

PCTIME /AUTO

Using PCTIME

------------

PCTIME has a number of interesting features that are briefly described below:

DIAL ACTS AND SET CLOCK

-----------------------

This option calls ACTS and sets your computer clock.  If

you have a AT type computer (286, 386, or 486), it sets

both the CMOS clock and the software clock.  On PC or

XT type computers, only the software clock is set.

During the phone call, a window will appear on screen

showing you the timecode being transmitted by ACTS.

TIMELINE DISPLAY

----------------

This option displays a timeline that produces a short audio

tone each second and a longer audio tone at the start of

each minute.  Coordinated Universal Time (UTC) and your

local time are displayed below the timeline.

You can use this option to set your watch or another clock

after you call ACTS.

Please note that the PCTIME software does not know whether

or not Daylight or Standard time is in effect until after

you call ACTS.  If ACTS has not been called, PCTIME

assumes that we are using Standard Time.  If Daylight Saving

Time is in effect and you have not yet called ACTS, the

Timeline Display will show the wrong offset between your

local time and UTC.  For example, if ACTS has not been called,

this display will show a 7 hour offset between Denver time

and UTC, even though the correct offset is 6 hours.  After

calling ACTS the offset will be displayed correctly.

Because of this (and to insure that the time is accurate),

you should call ACTS each time you run PCTIME, before

using the Timeline Display.

UNITED STATES TIME DISPLAY

--------------------------

This option displays the time in all United States time

zones, and in many major United States cities.

Please note that the PCTIME software does not know whether

or not Daylight or Standard time is in effect until after

you call ACTS.  If ACTS has not been called, PCTIME

assumes that we are using Standard Time.  If Daylight Saving

Time is in effect and you have not yet called ACTS, the United

States Time Display will show the wrong offset between your

local time and UTC.  For example, if ACTS has not been called,

this display will show a 7 hour offset between Denver time

and UTC, even though the correct offset is 6 hours.  After

calling ACTS the offset will be displayed correctly.

Because of this (and to insure that the time is accurate),

you should call ACTS each time you run PCTIME, before

using the United States Time Display.

VIEW CLOCK PERFORMANCE

----------------------

If you save your time differences to disk, this option will

display the performance of your computer's clock.  On AT

type computers, the CMOS clock performance is displayed.

On PC or XT type computers, the software clock

performance is displayed.

This display shows the date and time of each call to ACTS

(up to 200 calls can be displayed), and the time error (in

seconds) between your clock and NIST.  Based on this

number, PCTIME estimates the number of seconds per day

that your clock is gaining or losing time, and also the

frequency error of your clock.

HELP SCREEN

-----------

This option displays a screen that summarizes the features

of PCTIME.

EXIT TO DOS

-----------

This option exits the PCTIME software and returns you to

DOS.