MAPX.WK1, the Worksheet v2.0

A tool to find latitude and longitude

from a known reference on a map.  A

handy GPS companion.

Copyright 1995, John Ceccherelli

************************  WARNING  **************************

* This program has been tested to the best of my ability,   *

* but ...  If you plan to risk life, limb, or property on   *

* this program, it's at YOUR OWN RISK. You are responsible  *

* for its application and consequences.  The author is not  *

* liable for damages arising from the use of this product.  *

************************  WARNING  **************************

Quid Pro Quo:

You get what you pay for and MAPX is free.  Free for personal

and commercial use!  So feel free to copy, distribute, post, or

purge as you wish.  Just make sure that this document accompanies

the worksheet.  I would appreciate a note with your comments

and a brief description of what you do with MAPX.

Introduction:

I've spent many late nights trying to figure out latitude and

longitude of points on USGS Quadrangles.  It's a chore.  I've

spent many dollars for rulers, overlays, and similar contraptions

that never work all that well.  Conversely, I've been hiking

deep in the backwoods with $500 of GPS satellite technology and

have yet to find an easy way to translate positional coordinates

back to the map.  This is why MAPX was written.  With a simple

(and cheap) metric ruler, MAPX solves both problems.

MAPX was developed to solve these problems with USGS 7.5'

quadrangles.  It works well with scales down to 1/500,000 used

for Sectional Aeronautical Charts.  Smaller than this and the

results get dicey.  By the way, the bigger the denominator,

the smaller the scale of the map.  For example, a USGS 7.5'

quad has a scale of 1/24,000.  This is a larger number than

1/10,000,000 - a scale you might find on a wall map of the USA.

Remember: large scale-great detail, small scale-no detail.

Rules:

A few rules apply here.  First, North and West are positive (+),

East and South are negative (-).  I live and travel north

of the Equator and west of Greenwich so that's the way it is.

(Ceccherelli's Convention).  This applies to latitude, longitude

and the distances you will measure with your metric ruler.

Notice I said METRIC ruler, no English distance units allowed

because I'm a serious engineer (translation - the math is easier).

Measurements from the metric ruler are entered as millimeters.

Do not use MAPX for spans greater than 2 degrees from the

reference, map scale errors will begin to be noticeable.  MAPX

does not handle polar regions greater than +/- 88 degrees latitude.

A Real Life Example:

I recently went camping in the Catskills with some buddies of

mine.  Due to scheduling constraints and minor commitments

(like, my job), my friend Tom and I wouldn't be able to hit the

trailhead until dark.  It would be a 3 mile hike, just me, Tom,

and Coleman the lantern.  With a little luck and some expert

navigation, we would meet the others on the south shore of Tunis

Pond.  There was no room for error in this mission.  Rumor had

it that beverages made from fermented grains were being served

at the camp site.  The supply was sure to dwindle rapidly with

each passing minute.  Time was critical, we had to execute

flawlessly.

There's a hitch though.  Tunis Pond is not on the trail.  It's

a 300 meter bushwack north from a non-descript portion of the

trail.  Fortunately, I'm equipped with a GPS receiver.  All we

have to do is enter the lat and long of the point on the trail

(waypoint) where we depart due north.  Dead reckoning 300 meters

to the south shore of Tunis Pond should get us within shouting

distance of the others.

Tunis Pond is on Map 43 published by the New York-New Jersey

Trail Conference.  These maps are OK except they have precious

little positional information on them.  If you're lucky, there

will be one latitude and longitude intersection labeled.  I'm

lucky and this point is designated "the reference".

Here's where MAPX comes in (finally).  First we have to enter

the scale of the map.  This map is 1:63,360.

So enter 63360 in cell B11

The reference I'm using is 42� 00' 00"  north latitude and

74� 30' 00" west longitude. Enter the lat �in B15, ' in C15 and

" in D15.  Enter long �in B16, ' in C16 and " in D16.

Now get out the metric ruler and measure the vertical distance

from 42�00'00' latitude.  Note it's vertical, the vertical component

of the destination from the reference.  This will be a rectangular

deal where we measure Y then X from the reference:

Waypoint--> *            -

|

|<---vertical mm from x to * (27.5mm)

|

-            x <--Reference, 42�00'N

74�30'W

Enter 27.5 in B21 (it's positive because it's north of the reference)

Next measure the horizontal distance:

Waypoint--> *

|--horz distance = 80.5mm--|

x <--REF

Enter 80.5 in B22 (it's positive because it's west of the reference)

The worksheet should look like:

A           B      C     D     E      F          G

1

2                 MAPX, the Worksheet v2.0

3                 ?1995  J. Ceccherelli

4

5  Note; Lat/long north of the Equator and west of Greenwich are (+

6  south of the Equator and east of Greenwich are (-).  Distances

7  are measured_FROM_the reference_TO_the target or waypoint.  West

8  and north are (+), east and south are (-).

9  ----------------------------------------------------------------

10 Enter reference information in this section:

11  Map Scale  1:   63360             Magnetic Dev:          14 W

12

13                    Deg   Min   Sec

14                      �    '     "                 Ellipsoid

15   REF Latitude      42     0   0.0             a:  6378204.4

16  REF Longitude      74    30   0.0           e^2: 0.00676865

17 ----------------------------------------------------------------

18 To find the Lat/Long of a point on the map (target), enter the

19 X,Y distance to the target from the reference here:

20 Distance, Ref to Target:

21    VERTICAL mm    27.5

22  HORIZONTAL mm    80.5

23

24                    Deg   Min   Sec

25        Target:       �    '     "        UTM Zone         18

26       Latitude      42     0  56.5       Northing: 4651396.2

27      longitude      74    33  41.7       Easting :    536301

28 ----------------------------------------------------------------

The waypoint latitude and longitude appear in row 26 and 27.

Cool eh?  I plugged that data into my GPS receiver.  You also get

the Universal Transverse Mercator coordinates; more on that later.

We're hiking along wondering how much progress we've made

toward that waypoint.  So I whip out the map and the GPS

receiver.  A couple of minutes later, GPS tells me we're at

41�59'59"N lat, 74�33'32"W long.  Great!?  Where's that on the

map?  No big deal with MAPX, just enter 41�59'59" and 74�33'32"

on lines 33 and 34.  Cells B37 and B38 give you the Y and X

distance from the reference to the point on the map (where we

are now).

ref

x---

I'm here *                   ---  -0.5mm

|------ 77mm ------|

I pencil in that point and see there should be a spring on

my left... aaahhh, there it is.  Several gulps later, we continue

on the trail knowing we're more than half way there.

Well, we hit the waypoint with the aid of the GPS receiver.

Switched over to geomagnetic mode (used a compass, GPS sucks

for short distances), and dead reckoned due north 300 meters.

We ended up at the south shore of Tunis Pond about 50 meters west

of our friends!  The last drop of the aforementioned beverage had

been consumed about 30 minutes prior to our arrival :-(.

Polar Coordinates (Range/Bearing):

The polar (azimuth and distance) coordinates from the reference

to the position are also available in cells D37 and D38.  The

azimuth is referenced to true north so north = 0�, east = 90�,

south = 180�, and west = 270� (0� and 360� are both north).

Chuck Hawley of West Marine, has described a very useful method

to find your position on the map.  His "range/bearing" method

is simple and all it requires is a protractor or plotter.

Using a protractor with the example above, the position is 269.6

degrees and 77 millimeters from the reference.  Some people find

this measurement system easier than the X,Y rectangular system.

Enter the map corner (or other convenient) latitude and longitude

as a waypoint.  You then use the GPS range and bearing information

to plot your position.  I find it is easiest to plot from the

reference so you need to calculate the back azimuth (bearing)

by adding or subtracting 180 degrees which ever is appropriate.

Chuck suggests jotting down the distance conversion factor

in the map margin.  The math is easy to do by hand and you

don't need to bring a palmtop computer or similar device with

you.  I find this method beautifully simple and elegant.

If you are not comfortable with calculating in your head, I've

included the Range/Bearing method in MAPX.

The bearing from the reference to the waypoint is automatically

displayed in cell D37.  The range in millimeters is in cell D38

and the true ground distance is in cell D39.

If you want to determine the range and bearing FROM the

reference given the range and bearing TO the reference,

use cells B46 and B47 for input.  The back azimuth from the

reference is displayed in cells E46 and E47.  The X/Y

coordinates are also displayed in cell G46 and G47.

True to Magnetic Conversion:

MAPX will also adjust for magnetic deviation.  Magnetic

deviation for the local area is entered in cell G11.

Following the rules, west deviation is (+) and east is (-).

Cell H11 will automatically display E for east or W for west.

If you forget the rules, cell H11 will remind you.  Input

the true azimuth in cell B46 (range is not required).  Cell

C46 will display the magnetic azimuth.

Ellipsoid:

The default ellipsoid supplied with the worksheet is Clarke

1866.  This is the ellipsoid used for NAD27.  You can change

it to WGS84 by entering a, 6,378,137 in cell G15 and e^2,

0.00669438 in cell G16.  The Clarke 1866 values are a=

6,378,204.4, e^2=0.006768658.  You can use any other reference

ellipsoid to correspond with the datum of your map providing

you can find the appropriate values for equatorial radius (a)

and eccentricity squared (e^2).  See Appendix.

Universal Transverse Mercator (UTM) Conversion

MAPX converts lat/long to UTM!  You may have already noticed the

UTM output for the target and waypoint.  If you want to convert

any ol' latitude and longitude to UTM, use the last two sections

of the worksheet.  Be certain you are using the right ellipsoid.

I like using lat/long a whole lot more than UTM.  You need to keep

track of which hemisphere you're in (northern or southern).

Where lat/long can place you uniquely anywhere on the Earth with

just two pieces of data, with UTM you need four:

Hemisphere (north or south), zone (1 through 60), northing, and

easting.  I've combined hemisphere and zone to boil it down to

three pieces of data.  If zone is positive (+), you're in the

northern hemisphere.  If zone is negative (-), you're in the land

down under (southern hemisphere).

UTM is great for aiming artillery but I don't find it all that

useful for navigation.  Many people like it because they can draw

in the UTM grid on USGS quads.  I find it to be a major pain in the

ass.  The grid is usually skewed from true north (unless the central

meridian for the zone is the edge of the map).  I prefer to use the

lat/long fiducials that are spaced every 2.5' on a 7.5' quad.  You

draw in four lines and you end up with 12 additional reference

points on the quadrangle.

Accuracy:

How accurate is MAPX?  It depends on the map scale, map

projection, printing errors, and humidity.  Humidity?  Yup,

paper can expand and contract +/- 1% due to humidity.  And,

it's not necessarily uniform across the sheet.  This error

is twice that incurred by using a spherical model for the

Earth Vs. an ellipsoid.

You can compensate for humidity and printing error by calculating

the map scale yourself.  I use the bar scale on the bottom

margin of USGS quadrangles to do this along with digital

calipers.  For example, spanning 4km of bar scale, I measure

166mm or .166 meters. 4000 meters / .166 meters = a scale of

24,096.  The published scale is 24,000.  So this map has

0.4% error.  Enter 24096 instead of 24000 for scale (B11).

The smaller the map scale, the more distortion you get from

placing the curved Earth on a flat sheet.  This will introduce

errors.  For large scale USGS Quads like 24,000 and 63,360 scales,

you won't be able to measure the effect (well... maybe on

63,360 scales).  Minimize the error by using the closest

reference available.  This is usually the corners of the map,

use the closest corner for the reference.  Note that there are

"crosshairs" every 2.5' on 7.5' quads.  They are often very hard

to pick out through the clutter but they are handy for drawing

in supplimental grid lines (only 4!).  This is much easier than

drawing in the UTM grid.  These crosshairs appear every 7.5' on

USGS 1:100,000 scale maps also.  You can punch in the lat/long

of the crosshairs and MAPX will tell you where to find them!

After you adjust for all the above, you still have to deal with

the fact that the mapped feature may be wrong.  National Map

Accuracy Standards used by the USGS say that no more than 10%

of well defined features will be off by more than 1/50 inch.

What about not so well defined features?  What about the 10%

allowed to be off by more than 1/50 inch (.5mm)?

MAPX displays distances to .1mm precision from the reference.

That translates to 2.4 meters on the ground.  A .3mm pencil point

is a 7.2 meter circle on the ground (1:24,000 scale).  GPS

with SA on has CEP of 100 meters.  That's a 8.4 mm circle on

the map.  Don't get wrapped around the precision axle.

Thanks:

Thanks to Ron Harris for his suggestions and critique of MAPX

computations.  Thanks to Chuck Hawley for the "Range/Bearing"

method.  Special thanks to Tom Cowell for prodding me into

camping/hiking/canoeing situations where navigation turned

out to be a matter of life and death rather than an interesting

intellectual exercise.

References:

Snyder, John P., "Map Projections - A Working Manual", USGS

Professional Paper 1395, second printing, 1989.

Bowditch,N., "American Practical Navigator", U.S. Navy Hydrographic

Office, Washington D.C., 1966.

Duffet-Smith,P., "Practical Astronomy with Your Calculator",

third edition, Cambridge University Press, 1988.

Clarke,B., "Aviator's Guide to GPS", Tab Books, McGraw-Hill, 1994.

Moulton,F.,"An Introduction to Celestial Mechanics", Dover

Publications, 1970, originally Macmillan Company, 1914.

Greenhood,D., "Mapping", The University of Chicago Press, 1964.

Robinson et al,"Elements of Cartography", fifth edition,

John Wiley & Sons, 1978.

Thompson, M.,"Maps for America", third edition, U.S. Department

of the Interior, U.S. Printing Office, 1988.

Revision History:

Version 1.23 - Release  12/94

Version 1.24 - Documentation typos corrected  12/94

Version 1.30 - Fixed latitude display error occuring from 30' 1/95

to 59.99' south.

- Fixed X/Y sign error when crossing reference

and waypoint over the prime meridian.

- Fixed longitude modulus error when crossing

reference and waypoint over International

Date Line.

- Added user configurable ellipsiod.

- Added polar (range/bearing) input and output.

- Added true to magnetic heading conversion.

Version 2.00 - Added UTM conversion 2/95

Appendix:

Some Official Ellipsoids -

Name:                  a           e^2          Use:

GRS80              6,378,137     .006694380     WGS84

Clarke 1866        6,378,204.4   .006768658     NAD27, North America

Clarke 1880        6,378,249.1   .006803627     Africa, France

International 1924 6,378,388     .00672267      rest of world

Austrailian 1965   6,378,160     .00669454      Austrailia

Krasovsky 1940     6,378,245     .00669342      Soviet Union

Airy 1830          6,377,563.4   .00667065      Great Britain

Bessel 1841        6,377,397.2   .00667443      Central Europe

Everest 1830       6,377,276.3   .00663788      India, So. Asia

******************************************************************

*  Questions or comments?  Send them to me, John Ceccherelli on  *

*  CompuServe, 71011.3424  Internet: 71011.3424@CompuServe.com   *

*  February 1995 - Wappingers Falls, NY                          *

******************************************************************