How to connect QV10 to LX with cables and wiring

JCG00070@niftyserve.or.jp

Masaki "Brahma" Tsumori

95/12/12

To connect QV-10 and LX, the most simple and secure method is

to use cables and adapters.

Casio's cable and adapter supplied with QV-10 Link Software

(LK-1) are intended to meet the serial port of PC AT.  You can

connect this to a male DE9 adapter (thin black connector

supplied in HP F1023A adapter kit) and then HP F1015A(serial

cable for PC) and finally to LX.

________  ________

QV-10 ==========::________::________::=========== HP100/200LX

Casio      Casio      DE9 male    Serial

cable      25 to 9    Adapter     Cable

Adapter    in F1023A   F1015A

If you use a gender converter with DB25 connectors, you will

have smaller cable assembly.

________  ________

QV-10 ==========::________::________::=========== HP100/200LX

Casio      Gender     Modem       Serial

cable      converter  Adapter     Cable

DB25       male DB25   F1015A

female     in F1023A

However with this method, the cables and adapters together

make a mass as large as LX itself.  If you pursue compactness,

a small modification of the cable assembly from Casio would be

the best choice, as I did.

I was told (but not confirmed yet) that the serial cable F1016A

for Macintosh has unwired pins, which will make connection

unstable or even impossible.  You should use adapter cable with

all the necessary wires in place, or you should conduct

following modification of Casio's cable.

Modification consists of adding wires and making direct

connection.

Following is what you should do.  I have heard that there are

several version of cable assemblies attached to the Link

Software.  Please confirm the wiring first by yourself.

You can find a small simple circuit board if you open the D-sub

connector of the cable assembly supplied with Link software.

You should find six wires connecting this board to the

connector. They are

2 : TD

3 : RD

4 : RTS

5 : CTS

7 : SG

20 : DTR (other side of the board)

You can simply add wires which directly connect these signals

to the serial connector of LX.

Pin assignments of the serial connector of LX is as follows,

viewed from LX side.

/                 /

/     /   /       /

+------'~~~'------+  /

|  5  4  3  2  1  | /

| 10  9  8  7  6  |/

+-----------------+

@@D-Sub          LX

2    <->    3

3    <->    2

4    <->    7

5    <->    8

7    <->    5

20    <->    4

Placement of the added cable needs attention.  Cable cover of

the connector of Link Software has extra space, stuffed with

foam rubber.  Removing it gives you just enough space for

another cable, if its surface is slightly removed.

Do not shove a cable by force, otherwise it would make the

housing unable to mate, or would destroy the thin wires to

QV-10.  If you are not confident about your skill, please

request to those who have.

Please refer to the photograph (file CABLE.PCX) to see an

example of this connection.

[  Translated from Japanese by:       ]

[  Toshiki Sasabe (Janbe) 73560,524   ]