ENCOLSURE FOR THE SECOND DISK DRIVE

The second disk drive should be enclosed in some sort of box.  The type is not

critical sice the drive generates very little heat.  However, a metal box

which can dissipate heat is preferred to a close fitting wooden or plastic box

which will act as an insulator.

A very nice box which just fits a half-height 5.25 inch disk drive is

available from:

IB Computers

1519 S.W. Marlow

Portland, OR 97225

(503) 297-8425

The part number is P/N 8000.  The price in May 1988 was 14.95 plus shipping

and handling.  There is a volume discount for 12 or more units to $11.25.

The box is IBM beige with a color and texture that matches the PCjr case

almost exactly.

The only modification required for this box is to cut the back down

sufficiently to allow the power and ribbon cables to be connected and

disconnected without opening the box.  All the necessary screw holes and

screws are provided.

A second alternative is to use a Radio Shack box which requires major

modifications.  The part number is 270-272A.  The price in early 1988 was

$6.99.

This box reqires folding back the metal lips at one end which will be the

front.  The back must be modified to allow the cables to be connected and

disconntected.  There are no holes, so holes must be drilled in both sides to

support the drive and in the bottom plate to attach it to the drive.  The

black bottom plate must be cut down in front so it just reaches the bottom of

the drive face plate.  The back must be cut down to allow passage of the

cables.  If you want the bottom flush with the sides of the box, you will have

to trim about 3/8 in. off one side.

The final assembly is functional and attractive if care is taken, but it

requires much more work that the IB Computers box.

John M. King

Oct. 16, 1988

The third possibility is to find a used disk drive box from an Apple, Tandy or

other computer and use that.

#########################################################################################################