MCS-51 Microcontroller Family Bootstrap Program

BBBBBB    OOOOO    OOOOO  TTTTTTTT       5555555   11

BB   BB  OO   OO  OO   OO    TT          55       111

BB   BB  OO   OO  OO   OO    TT          55        11

BBBBBB   OO   OO  OO   OO    TT    ====  555555    11

BB   BB  OO   OO  OO   OO    TT               55   11

BB   BB  OO   OO  OO   OO    TT               55   11

BBBBBB    OOOOO    OOOOO     TT          555555   1111

U S E R ' S    M A N U A L

Version  1.0

January 20, 1996

copyright (c) 1996  by  W.W. Heinz

TABLE OF CONTENTS

-----------------

I.    Introduction

II.   BOOT-51 Installation

II.1   Files

II.2   Customizing BOOT-51

II.3   Tayloring the Cables

III.  BOOT-51 Operation

III.1  Interactive Operation

III.2  Batch File Operation

III.3  The BLINK Program

IV.   Troubleshooting

Appendix A:     CUSTOMIZ Error Messages

A.1 Parameter Errors

A.2 Baudrate Errors

A.3 Runtime Errors

Appendix B:     RESET Error Messages

Appendix C:     BOOT-51 Error Messages

Appendix D:     Trademarks

Appendix E:     Tables of Baudrates

I. Introduction

---------------

Usually an 8051 application program is not running correctly at once, and

it is not quite easy to debug it without suitable test equipment.

It is very unconvenient, to blow the program into an EPROM after every

change. Instead it would be nice to load the program into the external

RAM of the target board and then start it. This would dramatically speed

up the turn-around cycles. However, three problems must be solved first:

1. There must be a suitable interface to transmit the application

program from the host (PC) to the target (8051) system.

2. The target board needs a suitable firmware that can receive a

program from the host computer, store it in the external RAM,

and finally execute it.

3. Once loaded, the application program must be executable, although

it is stored in the external RAM, which is usually mapped into the

XDATA address space.

(Programs can only be executed in the CODE address space!)

The corresponding three solutions are:

1. an RS-232 interface between the PC and the 8051 board

2. a bootstrap program

3. external RAM that is mapped into both the CODE and XDATA address space

Nearly all evaluation boards for 8051 derivatives are meeting requirements

1 and 3. In most cases it is easy to establish a memory map according to

requirement 3, even on target boards that do not originally support it:

The -OE input of the (usually static) RAM must be driven by a logic AND of

the -RD and -PSEN signals of the MCU, rather than by -RD alone. That's all!

Requirement 2 can be met by blowing a customized version of BOOT-51 into

the EPROM of the target board. After system reset, it can receive an

Intel-HEX file over the serial interface, store it in the external RAM,

and finally jump to the program start address.

BOOT-51 itself doesn't need any external RAM, and it is by no means

restricted to a PC-based host system. BOOT-51 requires only 1 kB of EPROM.

Sure, a bootstrap program cannot replace a target debugger, but it's a start.

II. BOOT-51 Installation

------------------------

Before blowing BOOT-51 into an EPROM, it must be customized for the target

board, and one or two cables must be taylored for local requirements.

II.1 Files

----------

The BOOT-51 package consists of the following files:

BOOT51.DOC            BOOT-51 user's manual    (this file)

BOOT51.A51            BOOT-51 assembler source (requires ASEM-51 V1.2)

CUSTOMIZ.EXE          BOOT-51 customization utility

BOOT.BAT              batch file for application program upload

LOAD.BAT              called by BOOT.BAT only

RESET.EXE             program to reset target system via PC printer port

BLINK.A51             sample test program for BOOT-51

II.2 Customizing BOOT-51

------------------------

BOOT-51 must be customized for the target board, before it is ready for

assembly. This is performed with an include file BOOT51.INC, containing

the required configuration data. BOOT51.INC is generated with the utility

program CUSTOMIZ.EXE provided.

There are two major groups of configuration parameters:

-  baudrate generation

-  memory addresses

CUSTOMIZ is invoked as shown below:

CUSTOMIZ <baudrate> [<timer> [<accuracy> [<clock> [<start> [<user>]]]]]

The first four parameters are for baudrate generation, and the last two

parameters are memory addresses. Parameters in brackets are optional.

<baudrate> is the desired nominal baudrate (in Baud), BOOT-51 should use

for communication with the host system.

<timer>    is the on-chip timer or baudrate generator that is used for

baudrate generation. Legal values are:

T1 = timer 1, available on nearly all 8051 derivatives (default)

T2 = timer 2, available on 8052/32 and all true compatibles

BG = internal baudrate generator on 80C515/35 and 80C517/37

BA = internal baudrate generator on 80C515A and 80C517A

D1 = timer 1 with clock/12 or clock/4 prescaler on 80C320

<accuracy> is the required relative precision of the baudrate in %.

The default precision is 1.0 %.

<clock>    is the system clock frequency in MHz. (11.0592 MHz default)

<start>    is the start address of BOOT-51 on the target system in hex

representation. (default is 0000H)

<user>     is the usual hex start address of the application programs

in the external memory. (default is 8000H)

Baudrate:

---------

For the serial interface, the standard UART is used that is available on

almost every 8051 derivative. BOOT-51 is always running the UART in mode

1, that is asynchronuous mode, 8 data bits, 1 stop bit, no parity.

However, depending on the derivative, there may be various timers or

prescalers that can be used for baudrate generation.

If the desired baudrate cannot be generated or the precision is too bad,

CUSTOMIZ will display a corresponding error message. In this case, try

another baudrate generator that is also available on your derivative.

When this fails, too, try another baudrate that may also do, or think of

using another oscillator crystal that suits better for the task.

You should play around with CUSTOMIZ, to explore how to get best results.

The tables in Appendix E show the precisions of the most frequently used

baudrates that can be generated with the supported timers and baudrate

generators, for some standard clock frequencies.

In most cases baudrate deviations of 2 to 3 % are no problem for the

communication with a PC. Whenever in doubt, try it!

BOOT-51 can receive data at very high baudrates without the need for a

handshake protocol. Output is done more slowly with short pauses between

the characters, to avoid a data overrun of the PC UART. The maximum

effective output speed of BOOT-51 is corresponding to 9600 Baud.

In general, speed is rather a problem for the PC than for the micro.

Example 1:   CUSTOMIZ 9600 BG 0.1 12

----------

will generate an include file for BOOT-51 that configures it

for 9600 Baud, generated with the internal baudrate generator

of a SAB80C535 or SAB80C537 that is clocked with 12.0 MHz.

The required precision is 0.1 %.

The bootstrap and application program start addresses are

set to their default values.

Addresses:

----------

<start>:

An 8051 starts program execution at CODE address 0000H after reset.

Thus there must always be ROM at CODE address 0000H to execute the first

instructions. On most 8051 boards there is EPROM in the range 0000H-7FFFH,

and RAM in the range 8000H-FFFFH. However, the final application programs

will usually start at address 0000H, and the 8051 interrupt addresses start

at address 0003H. When application programs can be loaded into RAM at 8000H,

this is normally not quite realistic for testing.

Hence it would be nice to have CODE/XDATA-RAM at address 0000H to load

and test application programs. That is why some evaluation boards remap the

EPROM from address 0000H to 8000H, and the RAM from 8000H to 0000H, when the

first memory access to a CODE address > 7FFFH is made.

To support this, BOOT-51 starts with a long jump to its entry point, to

eventually remap the memory. If you own a corresponding evaluation board,

specify the <start> address of BOOT-51 to the location, where it resides

after memory remapping!

<user>:

In principle, BOOT-51 can load an application program at any location in

the external RAM and start it there. However, the interrupt addresses

should be redirected to the location, where application programs usually

start, e.g. the start address of the external RAM.

Hence the <user> address should point to the location, where the CODE/XDATA-

RAM starts. If it starts at 8000H, interrupt 0003H is redirected to 8003H,

interrupt 000BH to 800BH, and so on.

If the CODE/XDATA-RAM starts at 0000H (after remapping), the <user> address

has no practical meaning and can be left default.

Example 2:   CUSTOMIZ 19200 T2 1 11.0592 0 C000

----------

will generate an include file for BOOT-51 that configures it

for 19200 Baud, generated with timer 2 of an 80C52 or 80C32

that is clocked with 11.0592 MHz. The required precision is 1 %.

BOOT-51 is located at address 0000H in the EPROM, and the

interrupt addresses are redirected to location 0C000H.

If CUSTOMIZ aborts with an error message, it returns ERRORLEVEL 1 on para-

meter- and baudrate-errors, and ERRORLEVEL 2 on fatal runtime errors.

When CUSTOMIZ terminates without error message, it returns ERRORLEVEL 0, and

a customization header file BOOT51.INC should be present in the default

directory. With this file, BOOT-51 can be assembled as usual with

ASEM BOOT51

Remember that BOOT-51 requires ASEM-51 V1.2 or later!

Now there should be an Intel-Hex file BOOT51.HEX, which may serve directly

as input for the EPROM blower. We may also convert it to a binary image file

with the HEXBIN utility:

HEXBIN BOOT51/LENGTH:4000

would generate a binary image file BOOT51.BIN for a 16 kB EPROM (27C128).

(Note: Only 1 kB of EPROM is required for the BOOT-51 program code.)

Once in an EPROM, BOOT-51 should be ready to run on the target system.

II.3 Tayloring the Cables

-------------------------

Application programs are uploaded to the target system over a serial

interface. The serial ports of the PC may have 9-pin or 25-pin male

D-Shell connectors. Suitable RS-232 cables for the MCS-51 board can

be taylored as follows:

RS-232 cable with 25-pin female D-Shell connector:

--------------------------------------------------

PC serial port:                                  MCS-51 target system:

pin

Tx   2  o----------------------------------------->  RXD

Rx   3  o----------------------------------------->  TXD

RTS   4  o-----+

|

CTS   5  o-----+

DSR   6  o-----+

|

DCD   8  o-----+

|

DTR  20  o-----+

ground   7  o----------------------------------------->  ground

Shortening the modem control signals is absolutely required for batch

operation! If the CTS-, DSR-, or DCD-input is not set to logic low (+12V),

the PC will hang during character output to the serial port.

For operation with most terminal emulation programs, only the Tx-, Rx-,

and ground lines are required, and the handshake lines may be left open.

For further information refer to the documentation of your terminal

emulation program.

RS-232 cable with 9-pin female D-Shell connector:

-------------------------------------------------

PC serial port:                                  MCS-51 target system:

pin

Rx   2  o----------------------------------------->  TXD

Tx   3  o----------------------------------------->  RXD

RTS   7  o-----+

|

CTS   8  o-----+

DCD   1  o-----+

|

DTR   4  o-----+

|

DSR   6  o-----+

ground   5  o----------------------------------------->  ground

If you own a 9/25-pin mouse adaptor with all pins connected, the 9-pin

version of the cable should be preferred!

If your MCS-51 board has a reset input (or you can establish one), the

target system can be reset in batch files over a printer port of the PC.

For this, a suitable reset cable must be taylored as shown below:

Reset cable with 25-pin male D-Shell connector:

-----------------------------------------------

PC printer port:                                 MCS-51 target system:

pin

D0   2  o----------------------------------------->  Reset

_____

D1   3  o----------------------------------------->  Reset

ground  25  o----------------------------------------->  ground

In practice, only one of the two reset lines will be required.

When the program RESET.EXE is invoked, a positive reset pulse appears

at D0 (8031, 8032), and a negative pulse at D1 (80C535, 80C537).

For detailed information on the RESET program, refer to chapter III.2,

"Batch File Operation".

III. BOOT-51 Operation

----------------------

When BOOT-51 is customized correctly, and the target board is connected to

the host PC with the cable(s) described above, we can start to upload and

run application programs. There are two possible modes of operation:

- interactive operation

- batch file operation

III.1 Interactive Operation

---------------------------

BOOT-51 implements a simple ASCII user interface for serial communication.

The bootstrap program can be operated interactively with a terminal emulation

program running on the PC (e.g. TELIX).

To perform this, start your terminal emulation program, and configure it for

the correct baudrate, 8 data bits, 1 stop bit, no parity, and no handshake.

Then reset the MCS-51 target system.

Now the BOOT-51 sign-on message should appear on your screen:

BOOT-51  V1.0           Copyright (c) 1996 by W.W. Heinz

>

The '>' character indicates that the BOOT-51 command line interpreter is

ready for input. It implements exactly two commands:

1. The UPLOAD command:

First press the 'U', and then the return key.

Now the upload prompt ':' should be displayed. BOOT-51 is now ready to

receive an application program in Intel-Hex format. Send an Intel-Hex

file with the ASCII upload function of your terminal emulation program.

Now the received Intel-Hex records are loaded to their start addresses

specified in the record headers. If an EOF record is received, upload

is terminated and the command prompt '>' is displayed again, and the

command line interpreter is ready for the next command.

When a format error is detected in the Intel-Hex file, BOOT-51 waits

until no more characters are received for about two seconds, and then

aborts with a corresponding error message.  (see Appendix C)

2. The GO TO command:

To start an application program at address 8000H, enter "G 8000" and

press return. BOOT-51 sets now all MCU registers to reset conditions

(as far as possible), and jumps to the start address specified.

Now the previously loaded application program should be running!

Any desired number of Intel-Hex files may be loaded, before entering the

'G' command. The commands may be entered in upper or lower case letters.

Interactive operation should be preferred, when the application program

performs serial I/O. This can easily be supervised with the terminal

emulation program. In interactive mode, a reset cable is of little use!

III.2 Batch File Operation

--------------------------

If serial I/O is only used for program upload, batch file operation may be

the most convenient way of testing with the shortest turn-around cycles.

The sample batch job below, shows the principles of operating BOOT-51 from

a PC with MS-DOS batch commands:

MODE COM2:9600,N,8,1,P

RESET LPT1

ASEM %1

ECHO U >COM2

COPY %1.HEX COM2

ECHO G 8000 >COM2

In this example, serial I/O is done over serial port COM2, and the target

system can be reset over printer port LPT1.

First of all, the serial port COM2 is initialized to the desired baudrate

of 9600 bps, 8 data bits, 1 stop bit, and no parity with the MODE utility.

Then the RESET program provided is executed to reset the target system via

printer port LPT1.

To give the MCS-51 board some time to recover from reset (and to send its

sign-on message), ASEM-51 is now invoked to assemble the application program.

When finished, the BOOT-51 upload command 'U' is ECHOed to COM2, and the

Intel-Hex file is also simply copied to the serial port.

Finally the application program is started at address 8000H with the

command "ECHO G 8000 >COM2".

To test an application program MYPROG.A51 with the above batch job (stored

in a file TEST.BAT), simply type

TEST MYPROG

at the DOS prompt, and see what you get.

Of course this was only a minimized example! In general you will not only

have to modify serial port, baudrate, printer port, and start address for

your local requirements, but also to do something for plausibility checking

and error handling.

For this, the more sophisticated batch file BOOT.BAT has been provided.

It allows an easy change of all configuration parameters with any ASCII

editor. For further information see the commentary inside BOOT.BAT.

In principle, BOOT.BAT contains only that configuration data!

(The actual work is done by another batch file LOAD.BAT provided,

which is invoked by BOOT.BAT.)

A program MYPROG.A51 can now be assembled, uploaded and started with

BOOT MYPROG

Finally, the RESET program provided, may be worth a detailed explanation:

When invoked, RESET forms a reset pulse of the duration <pulsewidth> at

the D0 and D1 outputs of the PC printer port <port>:

RESET <port> [<pulsewidth>]

Legal <port> values are:  PRN, LPT1, LPT2, and LPT3.

PRN and LPT1 are equivalent.

The optional parameter <pulsewidth> is the duration of the reset pulse in ms.

Legal values are:  1 <= <pulsewidth> <= 65535.  (default is 50)

When invoked without parameters, a help screen is displayed.

In case of error, RESET returns the ERRORLEVEL 1, 0 otherwise.

When executed, RESET forms a positive pulse at D0 (pin 2), and a negative

pulse at D1 (pin 3) of the PC printer port. The default pulse width is 50 ms.

If it is too short to reset the MCS-51 board, specify a greater pulse width.

For batch file operation, a reset cable is highly recommended! If you don't

have it, you will always have to press the reset button of the target system,

before invoking BOOT.BAT. Since this may easily be forgotten, it is better

to automate this step.

III.3 The BLINK Program

-----------------------

When BOOT-51 is customized and blown into an EPROM, and the target board

is connected to the host PC, it would be nice to verify, whether the whole

configuration is working together correctly. This can be done with a short

test program that performs a well-defined action when started.

For this purpose the tiny assembly program BLINK.A51 has been provided.

It simply toggles the logic level of a port pin (roughly) once a second

(great when connected to a LED) to verify, whether all the components in

the chain

assembly -> serial interface -> target system <-> bootstrap program

are working together correctly.

It can be adapted to your requirements with little efforts:

1. If your target system carries a LED that can be switched with a

(bit-addressable) port pin, please change the BIT symbol LEDPIN

accordingly. Then the program will make the LED blink.

2. Change the program start address START to the location, where

user programs are usually loaded on your target system.

The program code itself is position-independent!

If there is no LED on your target system, connect the port pin LEDPIN to

a volt-meter. This may also do. Aside of P0 and P2 you may use every

bit-addressable port with LEDs or spare outputs. Now the DOS command

BOOT BLINK

should reset the MCS-51 board, assemble the test program, upload it to the

target system, and finally start it. If the LED is blinking, everything is

now perfectly installed and ready for daily work.

If not, you should read the next chapter!

IV. Troubleshooting

--------------------

1. When initializing the serial port, the DOS MODE utility aborts with an

error message like:  "function not supported on this computer", or

"illegal parameter", or something like that.

On most PCs, the BIOS doesn't support baudrates greater than 9600 Baud.

However, most terminal emulation programs can operate at baudrates of up

to 115200 Baud.

2. The RESET program aborts with "printer port not found".

The PC-BIOS didn't recognize the printer port specified.

Try another printer port or check your PC hardware.

3. The RESET program aborts with "hardware failure".

The PC-BIOS has recognized the printer port, but the data latch doesn't

work. Try another printer port, or check the defective port.

4. RESET terminates without any error, but the target system is not reset.

The reset cable may be plugged into the wrong printer port.

First of all try the other ports.

There may also be something wrong with the cable itself.

Employ the BOOT-51 manual and the circuit diagram of the target board,

and check the cable.

Furthermore, the reset pulse may be too short.

Specify a greater pulse width. You may also decrease the reset capacitor

of the MCS-51 system.

5. The target system is reset all the time.                            _____

When tayloring the reset cable, you may have mixed up the RESET and RESET

lines. Simply interchange them.

6. The PC hangs when data are sent over the serial interface in batch mode.

The RS-232 cable may be plugged into the wrong COM-port.

First of all, try the other ports.

There may also something be wrong with the modem control signals.

Employ the BOOT-51 manual and check the RS-232 cable on the PC side.

7. It works in interactive mode, but the PC hangs, if data are sent in batch

mode. There is something wrong with the modem control lines!  (see 6.)

==

8. In batch mode everything seems to work fine on the PC, but on the target

system you get absolutely nothing.

Try the interactive mode and reset the target system. If nothing happens,

you may have mixed up the receive and transmit lines of the target board.

If you get some garbage, the baudrates and/or serial data formats of the

target system and the PC are probably different.

Configure your terminal emulation program correctly, and change BOOT.BAT

accordingly. (correct data format: 8 data bits, 1 stop bit, no parity)

9. When invoked, the MS-DOS MODE command hangs, or fails to change the

previous baudrate, although it terminates without error.

Remove all networking and communication drivers and TSR programs from

your CONFIG.SYS and AUTOEXEC.BAT files, reboot, and try it again.

There is plenty of quick'n dirty software, doing something strange

with the BIOS interrupts!

Appendix A:     CUSTOMIZ Error Messages

===========

A.1 Parameter Errors:

---------------------

Parameter errors apply to the correctness and consistency of the command

line parameters. If one of these errors is detected, it is flagged on the

console, and CUSTOMIZ is aborting with ERRORLEVEL 1:

accuracy out of range                 Baudrate accuracy < 0, or > 5 %.

address out of range                  Address is no unsigned 16-bit number.

baudrate out of range                 Specified baudrate is < 0.

clock frequency out of range          Specified clock frequency is < 0.

illegal real number                   Parameter is no valid real number.

invalid hex number                    Parameter is no valid hex number.

too many parameters                   More than 6 parameters specified.

unknown baudrate generator            Baudrate generator not implemented.

A.2 Baudrate Errors:

--------------------

Baudrate errors apply to the results that CUSTOMIZ has derived from the

given program parameters. If one of these errors is detected, it is flagged

on the console, and CUSTOMIZ is aborting with ERRORLEVEL 1:

baudrate cannot be generated          Baudrate error is greater than 10 %.

insufficient accuracy                 Baudrate error is greater than the

required accuracy.

A.3 Runtime Errors:

-------------------

In general, runtime errors are file I/O errors.

If one of these errors is detected, it is flagged on the console,

and CUSTOMIZ is aborting with ERRORLEVEL 2:

access denied                 No priviledge for attempted operation.

disk full                     No more free disk space.

disk write protected          Attempt to write to a write-protected disk.

drive not ready               Disk drive is off, or no media mounted.

fatal I/O error               General (unknown) disk or device I/O error.

Appendix B:     RESET Error Messages

===========

too many parameters           More than two parameters specified.

illegal device name           No printer port.

invalid pulse-width           Pulse width is 0, or no unsigned 16-bit number.

printer port not found        The BIOS did not recognize the printer port.

hardware failure              The printer port data latch cannot be read back.

Appendix C:     BOOT-51 Error Messages

===========

checksum error                Intel-HEX record checksum is not correct.

illegal command               A command other than U or G has been entered.

illegal hex digit             Hex digit expected. Another character received.

illegal record ID             Intel-HEX record type is greater than 1.

unexpected character          The received Intel-HEX record doesn't start

with ':', or doesn't end with CR/LF.

Appendix D:     Trademarks

===========

ASEM-51 is a trademark of W.W. Heinz.

MCS-51 is a trademark of Intel Corporation.

TELIX is a trademark of deltaComm Development.

IBM-PC, IBM-XT, and IBM-AT are trademarks of IBM Corporation.

MS-DOS is a trademark of Microsoft Corporation.

All device codes of 8051 derivatives are trademarks of the manufacturers.

Other brand and product names are trademarks of their respective holders.

Appendix E:     Tables of Baudrates

===========

The following tables show the most frequently used baudrates that can

be generated with a particular timer or baudrate generator, for some

standard clock frequencies.

Baudrates which can be generated with a relative accuracy of better

than 4.0 %, are marked with that accuracy (in %) or with a '+' character,

if they can be generated exactly (= better than 0.01 %).

Baudrates that can be generated with Timer 1:

---------------------------------------------

bps\MHz |  11.0592  12.0000  14.7456  16.0000  18.0000  20.0000  24.0000

--------+---------------------------------------------------------------

115200 |

76800 |                       +

64000 |             2.34                                         2.34

57600 |     +

38400 |                       +

31250 |              +                          +                 +

19200 |     +                 +                2.34

9600 |     +                 +       3.55     2.34     1.36     0.16

7200 |     +       3.55     3.03     3.55     0.16     3.34     2.12

4800 |     +       0.16      +       2.12     2.34     1.36     0.16

3600 |     +       2.12     1.59     0.64     0.16     0.22     0.79

2400 |     +       0.16      +       0.79     0.16     0.94     0.16

1800 |     +       0.79     0.78     0.64     0.16     0.22     0.64

1200 |     +       0.16      +       0.64     0.16     0.22     0.16

600 |     +       0.16      +       0.08     0.16     0.22     0.16

300 |     +       0.16      +       0.08     0.16     0.22     0.16

200 |     +       0.16      +       0.16     0.16     1.73

150 |     +       0.16      +

134 |    0.03     0.09

110 |    2.27

75 |

50 |

Baudrates that can be generated with Timer 2 of the 8052:

---------------------------------------------------------

bps\MHz |  11.0592  12.0000  14.7456  16.0000  18.0000  20.0000  24.0000

--------+---------------------------------------------------------------

115200 |     +                 +                2.34

76800 |             2.34      +                         1.73     2.34

64000 |             2.34     2.86     2.34     2.34     2.34     2.34

57600 |     +                 +       3.55     2.34     1.36     0.16

38400 |     +       2.34      +       0.16     2.34     1.73     2.34

31250 |    0.54      +       1.70      +        +        +        +

19200 |     +       2.34      +       0.16     1.02     1.36     0.16

9600 |     +       0.16      +       0.16     0.69     0.16     0.16

7200 |     +       0.16      +       0.64     0.16     0.22     0.16

4800 |     +       0.16      +       0.16     0.16     0.16     0.16

3600 |     +       0.16      +       0.08     0.16     0.22     0.16

2400 |     +       0.16      +       0.16     0.16     0.16     0.16

1800 |     +       0.16      +       0.08     0.16     0.06     0.08

1200 |     +       0.16      +       0.08     0.05     0.03      +

600 |     +        +        +       0.04     0.05     0.03      +

300 |     +        +        +       0.02      +       0.02      +

200 |     +        +        +        +       0.02      +        +

150 |     +        +        +       0.01      +        +        +

134 |     +       0.02      +        +        +        +        +

110 |     +        +        +       0.01      +        +        +

75 |     +        +        +        +        +        +        +

50 |     +        +        +        +        +        +        +

Baudrates that can be generated with the 80535 Internal Baudrate Generator:

---------------------------------------------------------------------------

bps\MHz |  11.0592  12.0000  14.7456  16.0000  18.0000  20.0000  24.0000

--------+---------------------------------------------------------------

115200 |

76800 |

64000 |

57600 |

38400 |

31250 |

19200 |                                                           +

9600 |              +                                            +

7200 |                                         +

4800 |              +

3600 |

2400 |

1800 |

1200 |

600 |

300 |

200 |

150 |

134 |

110 |

75 |

50 |

Baudrates that can be generated with the 80C515A Internal Baudrate Generator:

-----------------------------------------------------------------------------

bps\MHz |  11.0592  12.0000  14.7456  16.0000  18.0000  20.0000  24.0000

--------+---------------------------------------------------------------

115200 |     +                 +                2.34

76800 |             2.34      +                         1.73     2.34

64000 |             2.34     2.86     2.34     2.34     2.34     2.34

57600 |     +                 +       3.55     2.34     1.36     0.16

38400 |     +       2.34      +       0.16     2.34     1.73     2.34

31250 |    0.54      +       1.70      +        +        +        +

19200 |     +       2.34      +       0.16     1.02     1.36     0.16

9600 |     +       0.16      +       0.16     0.69     0.16     0.16

7200 |     +       0.16      +       0.64     0.16     0.22     0.16

4800 |     +       0.16      +       0.16     0.16     0.16     0.16

3600 |     +       0.16      +       0.08     0.16     0.22     0.16

2400 |     +       0.16      +       0.16     0.16     0.16     0.16

1800 |     +       0.16      +       0.08     0.16     0.06     0.08

1200 |     +       0.16      +       0.08     0.05     0.03      +

600 |     +        +        +       0.04     0.05     0.03      +

300 |     +        +        +       0.04     0.05     1.73

200 |     +       0.05

150 |

134 |

110 |

75 |

50 |

Baudrates that can be generated with Timer 1 of the DALLAS 80C320:

------------------------------------------------------------------

bps\MHz |  11.0592  12.0000  14.7456  16.0000  18.0000  20.0000  24.0000

--------+---------------------------------------------------------------

115200 |                       +

76800 |                       +                         1.73     2.34

64000 |             2.34              2.34              2.34     2.34

57600 |     +                 +                2.34

38400 |             2.34      +                         1.73     2.34

31250 |              +                 +        +        +        +

19200 |     +       2.34      +       0.16     2.34     1.73     2.34

9600 |     +       2.34      +       0.16     1.02     1.36     0.16

7200 |     +       0.16      +       0.79     0.16     0.94     0.16

4800 |     +       0.16      +       0.16     0.69     0.16     0.16

3600 |     +       0.16      +       0.64     0.16     0.22     0.16

2400 |     +       0.16      +       0.16     0.16     0.16     0.16

1800 |     +       0.16      +       0.08     0.16     0.22     0.16

1200 |     +       0.16      +       0.16     0.16     0.16     0.16

600 |     +       0.16      +       0.08     0.16     0.22     0.16

300 |     +       0.16      +       0.08     0.16     0.22     0.16

200 |     +       0.16      +       0.16     0.16     1.73

150 |     +       0.16      +

134 |    0.03     0.09

110 |    2.27

75 |

50 |