----------------------------------------------------------------------------

6)                     SPECIFIC TASKS

a)  Using the directory tree for locating a file

b)  Averaging,  and saving the result.

c)  Fitting using DELP.

d)  Fast laser plotting

e)  Setting up display windows.

f)  Filtering.

g)  Integrating.

h)  Leak and transient subtraction.

i)  Response from control subtraction.

j)  Mean,  maximum and minimum.

k)  Creating and running a macro.

l)  Editing a data file.

a) USING THE DIRECTORY TREE FOR LOCATING A FILE

The file name is divided loosely into a path,  a file name and a file

extension.  First enter the File menu with <Alt>+F.  Next type "c"  for

"Change  dir"  and  enter  a path:  for example "C:\view\data"<Return>.

Next type "d" for "Directory", and (if necessary re-enter the path and)

enter a file mask, for example "*.*" or "*.dat" <Return>.

A box with the first 30 file names found corresponding to the  search

parameters  will  appear.   To  go  back  down  the  tree,  locate  the

highlighted bar on the  top  Left  corner  "..\".  If  untoward  things

happen,  such  as  no file names appear,  then <Esc> and check that the

path and file name mask are entered correctly.

Note that it is often quickest to start with no path entered  (except

maybe C:) and a file mask of *.*, and work through the menu tree to the

required  file.  You  only have to do this once,  as the search path is

retained even on exiting the program.

To re-load the same file to the displayed buffer,  just type <Alt>+L,

<Return>.

To  swap  between  the two most recently loaded files,  type <Alt>+f,

"p", <Return>.

---------------------------------------------------------------------------

b) AVERAGING AND SAVING THE RESULT

First load a data file by typing <Alt>+F, L,  [file name],  <Return>.

Get to the display mode by typing <Alt>+G.  NB check the following:

1)  that  the data sweep is displayed correctly,  if necessary,  type

<Home> and re-load the trace (or <spacebar>,  <tab> for  a  multi-sweep

file).  Then  re-size  and  position  the  trace  with  the arrow keys,

<Ctrl>, <insert), <delete>, <pg up>, <pg dwn>.

2) that "rotation" is zero for all sweeps.  The rotated status of the

displayed trace is given in the status line at the top  of  the  screen

(for  example  rot:  10  means  that  the  displayed  data is 10 points

displaced to the right).  Typing lower case  "z"  zeros  the  displayed

trace rotation, upper case "Z" zeros all rotations.

3)  that  the  average  buffer  buffer  is  empty (buf:0),  by typing

<Alt>+"z".

The displayed trace is averaged and the result  stored  in  the  not-

displayed  buffer when "+" is typed.  First a red trace show the result

of the averaging procedure,  then the  next  sweep  from  the  file  is

automatically loaded.

To  see  the result of averaging,  press "x" to exchange the buffers.

The word "average" will appear at the end of the status line at the top

of the screen,  and the scale will appear in red.  Note that as far  as

the   program   is   concerned   the   two   buffers   are   completely

interchangeable.

To save the result of averaging, first ensure that the averaged sweep

is NOT being displayed,  ie  that  the  displayed  buffer  is  the  one

containing  the  raw  data.  Next type <Esc> to get to the "Graph" menu

branch.  Type "a" to reach the "Average" menu, and select the option to

enter an "average file name".  Note that  the  program  forces  you  to

enter  a valid file name.  To save the average do one of the following,

type "t" to tag the trace to the end of the DATA FILE (not  recommended

on  principle),  "o"  to  delete the current average file and write the

contents of the buffer to this file,  "a" to append the buffer contents

to the end of the average file.

---------------------------------------------------------------------------

c) FITTING USING DELP (Differential equation, Legendre Polynomial)

There  are essentially three parts to finding an exponential fit to a

data trace.  First it is necessary to define which part of the trace is

to be fitted, next the parameters used in the fitting procedure must be

specified, last the fitting procedure can be run.

So load up a data file (<Alt>+F, L, filename, <Enter>), display it on

the screen (<Alt>+G) (if the trace looks odd,  then try pressing <Home>

to reset the scaling).

1)  DEFINING  THE  LIMITS OF THE REGION TO BE FITTED:  the two points

"Startfit1" and "Endfit1" are used to define the limits.  They  may  be

set in one of two ways, visually from the trace display, or numerically

from  the  Graph-Set  cursors- menu.  So from the display screen,  just

press <Enter> and two cursors with flags will  appear  on  screen.  The

arrow  keys  and  <Ctrl>  move  the  flags around the screen.  <Return>

toggles between the two flags,  and <Esc> leaves this routine with  the

cursors set.

Suppose you choose the numerical method (you want to start the fit at

point  number  300 exactly,  and terminate at 1000 for example).  Press

<Esc> to return to the menu tree (assuming you had got to  the  display

mode  by pressing <Alt>+G) and the Graph menu should appear.  Now press

"S" (for Set cursors),  1 (for Startfit1) and then type in  the  number

300  <Enter>.  Do the same for Endfit1.  Note that the program does not

allow you to enter out-of-range-values,  so that at no time can Endfit1

be less than Startfit1 for example.

2) SET UP THE FITTING PARAMETERS by going to the Delp menu (<Alt>+D).

Suppose  you  wish  to  fit  a  double  exponential,  then  select  the

"Int.level" option, and enter the number 2.

The number of polynomials used in the intermediate step  should  also

be  set.  It  should  be  large  enough  that  the  trace is adequately

described,  not so  large  that  time  is  wasted  calculating  surplus

precision,  and  at any rate greater than the number of free parameters

2*L+1  where  L  is  the  integration  level  (number  of   exponential

components).  A reasonable number for a double exponential fit would be

around 10.

Check  for  the following:  that the Baseline option is set to "Auto"

(pressing "B" toggles between  "Auto"  and  "Set"-  if  you  have  been

inquisitive  and pressed "B" when Baseline was already "Auto" then just

press <Esc>),  check that the Skip points option is set to 1 (calculate

every  point),  check  that  the  Mode  for  output  option  is  set to

"Info+rates" by pressing "M" repeatedly.  Lastly press "R" to  run  the

fit.  Now that the parameters have been set up, you could return to the

display  mode  (<Alt>+G) and fiddle around with the scaling,  alter the

cursors,  change sweeps or files,  and then run fit  just  by  pressing

<Alt>+R.

3)  SAVING  THE  RESULT.  Suppose you need to save the results of the

fit in an ASCII text file:  first you have to specify the name  of  the

file  to  which  to write the results.  So from the Delp- menu type "D"

for Delp file name,  and type in a file name.  You will not be  allowed

to  enter  an  invalid  file  name.  Press  <Esc>  to leave the default

unchanged,  <Enter> to leave your new name.  Presuming  that  you  have

just run a fit,  press either "O" for Overwrite or "A" for Append,  and

the fit parameters will be written to the text file.  The way in  which

parameters appear is affected by the choice of "Mode for output" in the

following  ways:  "Info" means the file path,  name,  file time,  sweep

number,  baseline value are included;  "rates" means  that  exponential

rate  constants  are  written,  "taus"  means  that  time constants are

written, and "LDE coeffs" means that the coefficients of the polynomial

whose roots are the exponential rate constants is written.

NB the residual from this fit is automatically saved in  the  current

directory  as "RESIDUAL.DLP".  It may be loaded quickly from the "Pick"

function,  by typing <Alt>+"F",  "P",  and then move the highlight  bar

over "RESIDUAL.DLP".  This trace will be loaded as though it were data,

so  do not opt for re-setting the scaling factors.  Similarly the trace

generated by the fitted parameters is saved to "SOLUTION.DLP".

USE OF THE BASELINE SET OPTION:

The accuracy of any fitting procedure depends on the number  of  free

parameters being fitted.  By setting a value for the baseline, one more

of  the  polynomial  coefficients  is  freed for use in calculating the

exponential rate constants.  This has no effect on the  calculation  of

the  amplitudes,  and  the  calculated amplitude can be used as a cross

check that your guess at the baseline was accurate:  they should  agree

to 3 to 4 significant figures.

From  the Delp- menu,  type "B" to toggle this option on and off.  If

the option is being turned on,  then a horizontal cursor will appear on

screen.  It  can be moved either by using the arrow keys,  or by typing

the value of the baseline that is required.  Press <Esc>  to  continue.

It  is  often  possible  to  make  a better estimate of the baseline by

fitting the later part of the trace only,  and observing the calculated

offset.  To  pick  up  this  offset value by typing answering "y" after

pressing "B" to set the baseline.

USE OF THE SKIP POINTS OPTION:

Frequently data  is  over-sampled.  To  save  computer  time,  it  is

possible  to  use  for  example every tenth point only by setting "Skip

points" to 10.  If too many points  are  to  be  skipped,  the  program

ignores  this  value  and calculates every point.  Note that if you are

going to use this function,  then you should filter the data at  1/5'th

to 1/10'th of the decimated digitization frequency.

USE OF THE WEIGHTING OPTION:

In  situations  where  there  is  a small amplitude,  relatively fast

component present,  and the noise level is high,  the fast component is

missed entirely in the fit,  because its amplitude is comparable to the

noise present.  By weighting the early part of the data (where the fast

component is most  significant)  a  fit  can  often  be  achieved.  The

weighting  method  used here is to multiply the data by an exponential.

The weighting exponential rate is then subtracted at a later point from

the fitted values.

Since the data  is  multiplied  through  by  an  exponential,  it  is

advisable  to use the Baseline SET option,  so that there will not be a

constant offset multiplied by the weighting factor.

Fit, no offset:    A1.exp(-L1.t)+A2.exp(-L2.t)

after weighting:   A1.exp(-[L1+W].t)+A2.exp(-[L2+W].t)

Fit, with offset:  A1.exp(-L1.t)+A2.exp(-L2.t)+A3

after weighting:   A1.exp(-[L1+W].t)+A2.exp(-[L2+W].t)+A3.exp(-W.t)

The presence of the A3.exp(-W.t) component is a nuisance,  and detracts

from  the accuracy of the fit.  It is avoided by using the Baseline SET

option.

To use exponential weighting,  first set a  "Weight  rate:".  If  the

weight  rate  is  set  to -10 (Note the negative sign),  then the first

point in the section of data being fitted is weighted more heavily by a

factor exp(10), compared to the last data point.

Next, toggle "Exp.  weighting" ON, and run the fit "R".  The apparent

deviation  of  the  polynomial fit (red) from the data is quite normal,

and arises because it is not meant to be a fit to the data,  but  to  a

trace that is exp(-W.t)* the data.  It is usually necessary to use more

polynomials to get good convergence at the start of the fitting region.

Experiment first with relatively small values of Weight rate, such as -

2 to -8, and look for consistency.

HOW MANY EXPONENTIALS ARE THERE?

It  is  inappropriate  to  ask  this  question  unless  the  data  is

sufficiently well defined to give an answer.  It obvious that the  more

free parameters that are fit to a data set,  the closer the fit will be

(unless the procedure just blows up).

A number of indications help in deciding what is the "correct" number

of exponentials.

1) Well defined data,  containing 2 exponentials for example,  may be

fit with a single exponential, but the fit is clearly not good, and the

residual  shows systematic deviations.  A double exponential fit should

lead to a visually good fit,  and a residual  that  has  no  systematic

deviations.  A treble exponential should do the same, but the amplitude

of the third component is small, and the rate constant may be positive.

2)  Consistency  from  one  data  set  to another:  the two "correct"

components should remain  similar  across  data  sets,  but  the  third

component will vary randomly.

3)  Does  the  offset,  calculated with the Baseline AUTO option set,

correspond to the presumed signal baseline?  If it differs,  there  may

be a slow, un-fitted component present.

ERROR ESTIMATION

In   principle,   it  is  straightforwards  to  calculate  the  error

associated with a  given  fit,  since  representing  the  data  with  a

polynomial  series  and fitting a differential equation are both linear

processes.   Allowance  needs  to  be  made  for  the  point  to  point

correlation,  but  in  principle  that is also possible.  The only non-

linear part is  the  calculation  of  the  roots  of  the  differential

equation.

In  practice,  an  empirical  estimate can be made by using test data

with the same noise spectrum and signal as the data  of  interest,  and

fitting  multiple  synthesized  data  sets,  or  just  by repeating the

experiment.

_____________________________________________________________________________

(d) FAST LASER PLOTTING

Two utilities are provided for quickly getting hard copy of  multiple

trace or sweep data sets, or multiple files.  They are both found under

the  Copy-menu,  and  are called Plot all sweeps- and Contents of dir-.

They have some features in common:  they depend on the  settings  under

the  Laser  menu.  For quick output,  it is best to use the Laser menu-

Plot style = Fast dots setting.

1) PLOTTING SEVERAL MULTI-TRACE FILES.  Suppose you have  six  files,

each containing several sweeps of data,  and you would like to show all

the sweeps of each file overlaid in a single  box,  one  box  for  each

file, all the boxes on one sheet of paper.

First you need to set up six windows on screen.  To do that, first go

to  the display mode <Alt>+"G".  Check that you are in window [0] (look

in the top RH corner),  and if you are not then press <F5>.  Next,  set

the  window size to the full screen by pressing "w" and then <Home> and

then <Return>. A box should outline the area of screen selected.  Next,

exit from the display mode by pressing <Esc>,  and you should  see  the

Graph menu.

Select  H  number  (the  number  of  boxes  horizontally)  and type 3

<Return>.  Next select V number (the number of  boxes  vertically)  and

enter 2.

Now go back to the display mode ("D" or <Alt>+"G"), and press <F7> to

get to the first window.

Two  parameters  have  to be set:  the text start point (for the file

name,  and information),  and the position of the scale bar.  They  are

both set relative to the first window, so having selected window [1] go

to  the  Laser  menu  by  typing <Alt>+"L",  "L".  From the Laser menu,

typing "T" will allow you to position the text start point.  If a cross

hair is not immediately visible on typing "T",  then  press  <Home>  to

bring  it  to  the  centre  of the screen.  Position it either above or

below window [1].

Next  press  "B" to position the scale bar also in window [1].  Next,

exit to the Copy menu by pressing <Esc>, and check that

i) an output file name is specified- it is better not to use  PRN  at

first.

ii) that Filter plots- is  OFF  (unless  you  really  want  to

filter  every  trace)

iii)  that  Auto windows is set to ONE WINDOW

iv) that Draw boxes iin ON

v) that Mode (HPGL/PCL) is set to  whichever  is  appropriate.  On  a

HPLiii,  PCL  mode  is  faster,  but  HPG files can be loaded into many

graphics programs and edited further.

vi) Reset the output file before starting

vii) Press "P" to  plot  all

the sweeps in this file.

When  this  file  has  finished  plotting,  go  to  the  display mode

<Alt>+"G",  and press <F7> to get window  [2].  Go  to  the  File  menu

<Alt>+"F"  and load the next file.  Go to the Copy menu <Alt>+"C",  and

just press "P".  Do not alter any other  parameters!  Repeat  with  all

six  files,  and  after  plotting the last,  press <Alt>+"E" to cause a

page-eject. Exit the program <Alt>+"X", and from the DOS prompt type:

NVPRINT plotfilename /B

NB neither the HPLii or  the  HPLiii  as  standard  is  designed  for

complex  graphical  plots.  Both  need more memory,  moreover the HPLii

runs into a "page formatting" problem after  about  2,000  points.  The

HPLii  avoids  this  problem  if  extra  memory  is  set aside in "page

protect" mode.

2.  PLOTTING A SAMPLE SWEEP AND THE AVERAGE (LAST SWEEP)  FROM  EVERY

FILE  IN A DIRECTORY.  Use the directory search mode <Alt>+"F",  "D" to

locate the first file in the directory (any file will do).

Go to the Display mode <Alt>+"G",  and  check  that  the  traces  wii

appear  on  the  screen.  If  in  doubt press <Home> to get the minimum

magnification.

Next go to the Copy-Laser menu <Alt>+"C",  "L" to check  on  the  Dot

size (eg 2), and the Plot style (Fast dots is usually best).

Return to the Copy menu <Esc>, and

i) press "O" to select a destination for the plot (preferably not PRN).

ii) Reset the output,

iii)  check that the Extension is the same as the extension of all the

files you want to print out,

iv) check Filter plots is OFF

v) set Auto windows to AUTO

vi) set Draw boxes ON

vii) check Mode (HPGL,LJ+): see above.

Lastly,  type :"C".  You are prompted with the number of files with the

extension given and offered the option to continue.

An Eject page is automatically sent after each page.

Exit to DOS, and see above how to print out the file.

_____________________________________________________________________________

(e) SETTING UP DISPLAY WINDOWS

This  is  a  task  that  is likely to cause some grief,  but bear the

following in mind:

1) the maximum number of windows allowed is 50

2) the windows are altered whenever H or V is changed

3) H and V are set so that they appear within the 0'th window

4) changing the 0'th window has no effect on subsequent windows until

H or V are reset.

5) the windows are defined in terms of screen pixels,  so if you swap

monitor  modes,  the display will be different.  Start from the display

mode (<Alt>+G) and select the 0'th window  (<F5>).  Make  it  fill  the

whole screen ("W" <Home> <Return>), then reset H and V.

_______________________________________________________________________________

(f) FILTERING

There are three parts to filtering, they are 1) choosing a filtering

method,  2)  setting  the  roll-off frequency and attenuation rate,  3)

setting the program to work filtering the data.

1) Assuming you have some data loaded, and displayed on screen,  type

<Alt>+"Q"  to  get  to the filter menu.  Press "T" to get a list of the

types of filter available.  These may be categorized as a)  causal,  or

simulations of analogue filters- these are the Bessel,  Butterworth and

Tchebyshev filters.  b) non-causal filters (those that have no hardware

counter-part)-  Sinc,  Gaussian,  band-pass.  c)  special  filters that

differentiate the data- Dsinc, 3 pt diff.

So if you want to simulate the effects of an  analogue  filter,  then

pick  a  Bessel,  Butterworth or Tchebyshev filter.  If speed is of the

essence,  then use a Gaussian filter,  if the best high frequency noise

reduction is what you are after, then use a Sinc filter.

A  Gaussian filter has a response that is similar to a Bessel filter,

there  is  no  "ringing"  following  an  abrupt  step.   However,   the

attenuation rate is poor.

A Sinc filter has a theoretically infinite roll-off rate, but it does

produce  the  effect  of  "ringing"  (low amplitude oscillations in the

region of  an  abrupt  step).  However  aesthetically  displeasing  the

"ringing"  effect  may  be,  the  Sinc filter is by definition the best

filter for retaining the signal waveform while cutting  out  components

over a given frequency. This is because the best least-squares fit of a

terminating  Fourier  series  to  any given signal is just the infinite

Fourier series truncated at the terminating frequency.

2) Having chosen the filter, you will need to set the roll-off. For a

causal  filter,  this  is  the  frequency  at  which  the  response  is

attenuated  by  3dB.  The  only  filter  that  will  allow you to use a

frequency greater than 1/10'th of the  digitization  frequency  is  the

Sinc filter, but 1/5'th is the highest you can go.

You  also need to set the attenuation rate for all filters apart from

the Gaussian filter.  You do this by setting either  the  window  width

(Sinc) or the number of poles (Bessel, Butterworth, Tchebyshev).  A one

pole filter is simply a R-C network,  so the causal filters start at 2-

poles.  An 8-pole filter gives the fastest attenuation rate,  but  also

takes the longest.

The  Sinc filter function has significant amplitude out to many times

the fundamental filter period,  so  to  be  practical,  it  has  to  be

artificially  terminated.  The  termination  point  is  called here the

"Window width" since a modified "Hann window" is used to terminate  the

filter  function.  A  window width of 2 means that the first two minima

to each side of the filter function are passed unchanged, and a further

two minima are passed truncated by a raised cosine.  Note that a window

width of  1  gives  a  pretty  good  attenuation  rate,  of  around  50

dB/octave.

3) Lastly,  filter the data either by pressing "F",  which causes the

entire sweep or trace held in the displayed buffer to be  filtered,  or

by pressing "L" which causes only the part of the loaded trace which is

displayed  on  screen  to  be  filtered.  The  Local filter function is

useful for looking at a small part of a data trace in detail.  From the

display mode,  typing <Alt>+"Q",  "L" performs  the  local  filter  and

returns the program automatically to the display mode.

______________________________________________________________________________

(g) INTEGRATING

The  major difficulties associated with digital integration are first

that the integrated signal readily  goes  out-of-range,  and  that  the

presence   of   a  small  baseline  offset  can  swamp  the  signal  on

integration.

Both of these difficulties are overcome here:  setting  the  baseline

for integration is easy, and a special feature allows automatic scaling

to prevent an out-of-range error.

There  are two ways of running the integration feature,  the first is

from the Display mode,  the  second  is  from  the  menu  tree  (Graph-

Integrate menu).

RUNNING INTEGRATION FROM THE DISPLAY MODE

First  load  a data file,  and display it by pressing <Alt>+"G".  The

entire business of selecting  the  integration  limits,  adjusting  the

baseline, and doing the integration is started by typing "N".

First  a pair of cursors appear on screen.  They may be adjusted with

the  arrow  keys,  and  <Ctrl>+arrows.  Toggle  between  the  two  with

<Enter>.  Move  on to the next part with <Esc>.  Next a horizontal line

appears,  with the "Y" value in box.  The line can be moved up and down

with  the  vertical  arrows,  or  you  can type in a number for the "Y"

value.  Again,  press <Esc>  to  move  on.  Almost  immediately  a  red

integrated trace appears on screen.  What has actually happened is that

successive integrations have been performed until a reduction factor is

obtained  that  avoids  an out-of-range error.  The Y scale is adjusted

for this display to account for the  reduction  factor.  You  can  swap

between  the  buffer  containing the data and the buffer containing the

integrated trace by pressing "X".

RUNNING INTEGRATION FROM THE MENU

The individual parts of performing an integration  have  to  be  made

separately.  It  is  assumed that you have a data file loaded,  and the

required  sweep  selected.  The  first  thing  to  do  is  to  set  the

integration limits, either by entering numbers for A: start int. and B:

end  int.,  or by selecting the Mark range feature,  and moving cursors

about on the screen.  Next,  you have to set a baseline- a  value  that

will  be  subtracted  from the data before integrating.  You do this by

selecting the Int.  baseline feature.  A  horizontal  line  appears  on

screen that show the mean Y value between the integration limits (so if

you  accepted  this value for the baseline,  the integrated trace would

end up at zero at the right hand limit.  You can adjust  this  baseline

value,  as  described  above.  It might be a good idea to initially set

the integration limits around a section of data that you wish to use to

estimate the baseline,  the set  the  baseline,  then  re-position  the

integration limits before integrating.

Having  set  the integration limits and the baseline,  you can choose

either to use the Multiple  tries  option  to  automatically  obtain  a

reduction  factor that keeps the integral within range,  or you can set

the reduction factor yourself,  and do a  single  try.  Note  that  the

Multiple  tries  option does NOT start with the reduction factor set to

1,  but starts at whatever you last had it set at.  In this it  differs

from the integration run from the Display mode.

______________________________________________________________________________

(h) Transient subtraction.

This  feature is designed for the benefit of electrophysiologists who

wish to remove that part of a current response to  a  potential  change

which  is  due  to  the  passive  electrical properties of the cell and

recording apparatus.  In general,  there will be an Ohmic component  to

this passive current, related to the existence of a leak to ground from

the recording pipette and cell,  and there will be a reactive component

that is mainly of the form of a capacitive transient.

The  options  are  to remove the transient part only,  the Ohmic part

only or to remove  both  parts.  Removal  of  the  transient  component

depends  on the existence of a response to a voltage pulse that elicits

a passive response only-  no  channels  open.  This  response  is  then

scaled  according  to the stimulus size and added to or subtracted from

each raw data sweep.

The procedure is quite well described  in  MANUAL1.doc  under  Graph-

Transient/Control-Transient  menu.  However,  it  should  be emphasized

that for a successful subtraction  a  number  of  criterion  should  be

satisfied:

1)  the  data  are  digitized at a frequency adequate to describe the

transient part of the response.

2) the passive response is a smooth curve- ie it is an average.  This

is  particularly  important  if  the  response  is  to  be  scaled   up

significantly before being added/subtracted.

3)  that  there is NO constant current offset prior to the test pulse

in the passive response.  If necessary it can be  subtracted  from  the

passive response prior to beginning the subtractions.

______________________________________________________________________________

(i) Response from Control subtraction.

This  is  also a straightforwards procedure that is well described in

MANUAL1.doc in  the  section  Graph-Transient/Control-Control  subtract

menu.   The   important  points  to  note  are  that  the  digitization

frequency,the number of points and the gain must be  identical  between

the control and response recordings.

Note that the control and response can be in the same file,  but must

be in sequence.

______________________________________________________________________________

(j) MEAN MINIMUM AND MAXIMUM

I will get around to writing this eventually!

______________________________________________________________________________

(k) EDITING A DATA FILE

You have a data file containing a number of sweeps.  You do not wish

to  keep  all  of the sweeps,  and the sweeps that you wish to keep you

would like to alter in some way (filter, invert, leak subtract).

First load up the data file  (<Alt>+"L").  Next  select  the  sweeps

that  you  wish  to  keep.  This may be done in one of two ways:  1) by

tagging each sweep from the display  mode,  2)  by  creating  an  "edit

list".

1)  Tagging  sweeps.  Go  to the display mode <Alt>+"G".  Advance to

the required sweep.  Press "t",  and you will see "TAGGED"  appear  (or

disappear)  from  the  top  of  the  screen.   Note  that  the  program

automatically advances to next sweep.  To change the "tagged" status of

all the sweeps, press "T".  This will make all the sweeps either tagged

or not depending on the status of the current sweep.  Next press <ESC>,

and "E" to get to the "Edit file" menu.  Give the file name of the file

you would like to write to (NB do not make this the same  as  the  data

file),  and reset it.  Select the options to filter,  leak subtract and

invert as necessary,  and then type "A" to "write  tagged  swps"  to  a

file.

2) Edit list.  From the Edit menu, type "M" to make a list.  Enter a

list  of  sweep  numbers  spaced with commas.  Use two stops (..) for a

range.  For example 1,2,3..8 will  select  sweeps  1  to  8  inclusive.

Select the filter,  invert, and leak subtract options as necessary, and

then type "B" to run the list.