How to build a Jughead's D/A Converter

(C) 1989,1990 by Tomi Engdahl

Writing date: 20th May, 1990

You need:

Connectors: D25 male

D25 female (optional)

Resistors: 640k,320k,160k,80k,40k,20k,10k,5k,390  (+-1%)

(You can use different values of resistors, but

the ratio of the values of the resistors must be

same. 1, 2, 4, 8, 16 etc.)

Capacitor: Electrolytic 10 yF 10V

CHEMATIC

------

l  2 l ---<640k>---+

l  3 l ---<320k>---l

l  4 l ---<160k>---l

l  5 l ---<80k>----l

l  6 l ---<40k>----l

l  7 l ---<20k>----l-----+---- 10 yF -------> Out

l  8 l ---<10k>----l     l

l  9 l ---<5k>-----+   <390>

l    l                   l

l 25 l ------------------+------------------> Ground

------

How to build

Solder all D25 male connector pins to corresponding D25 female

connector pins (pin 1 to pin 1 etc.). Connect the resistors

according the chematic. It is maybe not possible to obtain exact

resistor values. If you can't obtain them, try nearest values.

<1% deviation is acceptable.

How to use

Connect this circuit to the centronics printer port of your IBM

PC or compatible. Connect the printer cable to the D25 female

connector of the circuit. Your printer should work correctly

with this circuit and you can keed this circuit connected all

the time. Connect the sound output to your stereo system. Line

in and mic lines are suitable because the output level of this

circuit is adjustable (about 0-2V PP). The sound quality is very

good.

DISCLAIMER

I don't consider myself as responsible for anything considering

this circuit. I am not responsible to any losses caused by this

circuit diagram to anybody/anyone. If you can cause some damage

with this circuit, it is your fault. If this this circuit does

not work, it is not my fault (you have made something wrong if

this circuit does not work).

Contact the author

If you have made interesting software which supports this device

or you have ideas how to improve this circuit, please write to:

Tomi Engdahl

Kalliomntie 8bF

01800 Klaukkala, Finland