Press [Page Up] and [Page Down] to view this document a screen

at a time. Press the up and down direction keys to scroll

through the document. To exit the document at any time, press

[Esc].

CL-GD542X Display Drivers and Utilities

User Guide

Product: GDS542X-X-D130P

Software revision: 1.30

Document release date: April 12, 1993

Document revision: 1.30

Page 2

Notice

Cirrus Logic, Inc. believes the information

contained in this document is accurate and

reliable. However, Cirrus Logic, Inc. reserves

the right to make corrections, improvements, or

changes to this manual at any time and without

notice. No responsibility is assumed by Cirrus

Logic, Inc. for its use; nor for infringements

of patents or other rights of third parties.

This document implies no license under patents

or copyrights, other than those explicitly

stated.

This manual is copyrighted. All rights reserved. This document

may not, in whole or part, be copied; photocopied, reproduced,

translated, or reduced to any electronic medium or machine

readable form without prior consent, in writing, from Cirrus

Logic, Inc.

Copyright 1992 - 1993, Cirrus Logic, Inc.

All rights reserved.

Trademarks

Windows, MS-DOS, and MS Word are trademarks of Microsoft

Corporation. AutoCAD is a registered trademark, and AutoShade

and AutoSketch are trademarks of Autodesk, Inc. Display List

Driver-VGA, TurboDLDClassic, CustomColors, are trademarks of

Panacea Inc. GEM is a trademark of Digital Research, Inc.

Lotus, 1-2-3, and Symphony are trademarks of Lotus Development

Corporation. PC-DOS. Ventura Publisher is a trademark of Xerox

Corporation. WordPerfect is a trademark of WordPerfect

Corporation. WordStar is a trademark of WordStar Corporation.

RenderMan is a registered trademark of Pixar. All other brand

and product names are trademarks or registered trademarks of

their respective companies.

Page 3

Contents

Introduction                                        Page 5

Utility software                                    Page 7

Clmode                                            Page 8

SetRES                                            Page 12

Display drivers                                     Page 13

Microsoft Windows                                 Page 14

Autodesk - ADI 4.2                                Page 16

GEM/3                                             Page 39

Lotus 123 v2.x, Symphony, and Report Writer       Page 41

Lotus 123 v3.X                                    Page 42

Ventura Publisher                                 Page 44

WordPerfect 5.1                                   Page 45

WordStar                                          Page 47

Page 4

This page has been intentionally left nearly blank

Page 5

Introduction

This manual provides operating instructions for user utilities

and installation instructions for the display drivers supplied

with your Cirrus Logic VGA.

The Cirrus Logic VGA graphics system is 100% VGA compatible.

The utilities and drivers supplied with your Cirrus Logic VGA

provide support for additional features of your Cirrus Logic

VGA, such as greater speed, higher resolution, and more

available colors.

Before you begin

The following instructions assume that the user is familiar

with DOS and certain DOS commands. Please review the

associated DOS commands before performing the installation.

Notational conventions

Throughout this manual, the term 'DOS' refers to both MS-DOS

and PC-DOS, except when noting features that are unique to one

or the other.

Table 1 shows the typographic conventions that are used

throughout this manual:

Type Style                    Description

user input      This text must be typed exactly as it

appears. Text within brackets indicates

certain keyboard keys (such as [Enter],

[F10], etc.).

ALL CAPITALS    Directory names, files and acronyms.

Table 1: Notational conventions

Easy installation

The installation utility is provided to facilitate the smooth

installation of the display drivers and utility software. The

installation program is menu-driven and allows you to select

and install only those display drivers for software and

applications currently in use.

It is important to note that some display drivers need to have

the associated vendor's application program already installed

on the system prior to loading the Cirrus Logic VGA display

drivers. In other cases, the loading of the display driver may

be an integral part of the vendor's product installation

process. Please review the driver product section below for

specific instructions prior to running the installation

program.

The installation utility is located on the diskette labeled

Page 6

Disk 1. To install the desired display drivers and utilities,

insert the diskette into the A: drive, type

A:[Enter]

INSTALL[Enter]

Follow the instructions on the screen to install the listed

display drivers. At any time you may press [Esc] to abort the

installation process and go back to DOS.

Selected drivers are simply copied to the specified disk and

directory. Applications may require additional installation as

described in the Display drivers section of this manual.

Cirrus Logic BBS

If there are any newer versions of the software provided with

your VGA adapter, they will be available on the Cirrus Logic

BBS. It is a WILDCAT! BBS. The phone number is (510) 226-2365.

It can handle modems running up to 9600 baud. The modem should

be set to NO parity, 8 data bits, 1 stop bit. Before you are

able to download software, you must join the conference(s) for

the product(s) that you are interested in.

Page 7

Utility software

Several utility programs are supplied with you Cirrus Logic

VGA. The following section describes these utilities and how

to use them.

Clmode is a program supplied with your Cirrus Logic VGA to

configure it's various options.

SetRES lets the user change the operating resolution and the

number of colors from within Windows.

Page 8

Clmode

The Clmode utility allows the user to define the type of

monitor attached and set the video modes supported by the

Cirrus Logic VGA.

Using Clmode's menu-driven interface

At the DOS prompt type:

CLMODE[Enter]

The main popup window consists of a number of buttons. Each

button represents a different option or menu. The underlined

letter of a button name specifies the hot key combination for

that item. For example, press the [Alt] and the underlined

letter keys simultaneously or just the underlined letter key

to select an option. Note if you intend to use a mouse with

Clmode, then a mouse driver should first be installed prior to

running the Clmode utility.

Choosing the attached monitor type

Selecting the proper monitor type will allow the Cirrus Logic

VGA to display the highest quality output that it is capable

of with the attached monitor. The monitor type determines what

video modes will be available to your system. It will also

determine the vertical refresh rates available. Generally, the

higher the refresh, the better the screen will look. A

description of the available monitor types is given in table 2

at the end of this section.

Select the Monitor Type button. The Monitor Type setup window

will be displayed. The current monitor type will be

highlighted initially. Use the cursor keys or the mouse to

choose the type of monitor attached. Selecting the Advanced

monitor type will enable you to customize the monitor timings

for the monitor you are using.

To customize the monitor timings, use the mouse or the tab key

to select the gray Set Advanced button. It will only be

available if the Advanced monitor type is selected. This will

bring up a new dialog box with drop down menus for each

available resolution. Select the desired refresh rate for each

resolution. Consult the manual for your monitor to discover

the correct rate. When all of the settings are correct, select

the OK button. If there are any problems or you change your

mind, select the Cancel button.

Before saving the new monitor type, you may want to select the

Verify button to view each of the new video timings that you

have selected to verify that they are compatible with your

monitor. After each test screen is displayed, press [Enter] or

the left mouse button to see the next video mode. If there is

Page 9

a problem, press [Esc] or the right mouse button will return

you to the Monitor Type Setup dialog box. Try a slower setting

for the resolution that did not work and try verifying your

choices again.

Once you are sure that everything is correct, press the Save

button to set the new monitor type. Press the Cancel button to

discard any changes. Press the Help button to get a detailed

explanation on the monitor types and the VGA refresh rates.

On some systems the monitor type will be remembered from one

session to the next. To test this, select a monitor type and

exit the Clmode program. Turn the computer off for ten

seconds. After rebooting the computer, run Clmode and verify

whether or not the monitor that you selected is still enabled.

If it is, it should not ever need to be set again in normal

use. If the monitor type was not kept then you should select

the option to have Clmode save the monitor type in you

AUTOEXEC.BAT file. You will be given this choice after

selecting the Save button.

Available video modes

Select the Video Mode button. The Video Mode Preview window

displays all the modes supported according to the monitor type

attached and the amount of video memory present. This list of

video modes will tell you which are available in your current

configuration for use with extended resolution drivers. To see

what different video modes look like on your monitor, select

the Preview button. After each test screen is displayed, press

[Enter] or the left mouse button to see the next video mode.

If you don't want to see any more video modes press [Esc] or

the right mouse button will return you to the Video Modes

Preview window.

Retrieving the current VGA controller status

The information in the main window displays the VGA controller

type, the BIOS version number and the amount of video memory

present.

Getting help

Clmode provides Help for the following items: Monitor Type,

Video Mode Preview, Mouse, Keyboard and About Clmode. The

Monitor Type help window explains the different capabilities

of each monitor type. The Video Mode help window defines the

information given in the Video Mode window. The Keyboard and

Mouse help windows explain how to use the keyboard and the

mouse to make selections. The About window displays the Cirrus

Logic copyright message and the Clmode version number.

Page 10

Exiting the Clmode

To exit Clmode at any time, press [Alt] and [F4] keys

simultaneously, or click the left mouse button on the system

button of the main window (i.e. The top left corner button of

the window which is shown as a dot), or select the Exit

button. When the Clmode utility exits, the current video mode,

monitor type, and VGA refresh rate will be displayed.

Using Clmode's command line options

When command line options for CLMODE.EXE are given at the DOS

prompt, the menu-driven windows will not be displayed.

Instead, monitor type, video mode and refresh rate will be set

at the DOS prompt. The command line options for CLMODE.EXE

are:

CLMODE [[modenum][+*-]] [m[montype] | t6=x t8=x t1=x t2=x]

[modenum]  mode number

[+*-]        + selects 400 lines (default)

* selects 350 lines

- selects 200 lines

[montype]    monitor type

t6=x(Hz)     640x480   @ (0 = 60, 1 = 72)

t8=x(Hz)     800x600   @ (0 = 56, 1 = 60, 2 = 72)

t1=x(Hz)     1024x768  @ (0 = 87i, 1 = 60, 2 = 70, 3 = 72)

t2=x(Hz)     1280x1024 @ (0 = 87i, 1 = Not available)

For example, to select mode 3 for a Super VGA monitor (montype

2), type the following command at the DOS prompt:

CLMODE 3+ m2[Enter]

To select custom monitor timings with 640x480 at 60Hz and

800x600 at 72Hz refresh type:

CLMODE t6=0 t8=2[Enter]

Typing an invalid option will display the command line help

text. Typing [S] as a command line option will display the

current Clmode settings.

On the next page is a table of common monitor types.

Page 11

Monitor   Examples    Horizontal     Vertical        Display

Type                Frequency   Frequency (Hz)   Resolutions

(kHz)

0      IBM 8512,      31.5           60           640x480

8513,

8503

1      IBM 8514,      31.5           60           640x480

8515         35.5    43.5 - interlaced  1024x768

2       NEC 2A        31.5           60           640x480

35.2           56           800x600

3       NEC II        31.5           60           640x480

35.2           56           800x600

35.5    43.5 - interlaced  1024x768

4       NEC 3D        31.5           60           640x480

37.8           60           800x600

37.8    43.5 - interlaced  1024x768

5      Sony CPD-      31.5           60           640x480

1304,        48.0           72           800x600

NEC 3FGx,      48.0           60          1024x768

Nanao 9065S,     48.0    43.5 - interlaced  1280x1024

9070U

6       NEC 4D,       31.5           60           640x480

4FG,         48.0           72           800x600

Nanao T240i     56.0           70          1024x768

48.0    43.5 - interlaced  1280x1024

7       NEC 5D,       31.5           60           640x480

5FG, 6FG,      48.0           72           800x600

Nanao        58.3           72          1024x768

T550i,        48.0    43.5 - interlaced  1280x1024

T560i,

T660i,

F550i,

F750i,

Table 2: Monitor type examples

Page 12

SetRES

SetRES is a utility that runs under Windows 3.x. It allows the

user to change the screen resolution, number of screen colors,

and select either large or normal fonts and system resources.

After new options have been selected, the user can either

immediately restart Windows, in which case the new resolution

will take effect immediately, or continue working in the

current resolution, in which case the new resolution will take

effect the next time Windows is started. Note that under

Windows 3.0 large and normal font selection is not allowed.

This application assumes that the Windows drivers have been

correctly configured using Windows Setup. For information on

installing and configuring Windows display drivers using

setup, please refer to the Microsoft Windows section in this

manual. From Windows Setup select the Multi-Resolution entry.

This will correctly configure the driver for Windows 386

enhanced mode operation, and copy the necessary font files to

your hard disk.

Installation

Start Windows.

In the Program Manager, choose File Run.

Specify the diskette drive where the Windows Drivers and

Utilities Diskette is inserted, and run the install

program. For instance if the floppy is in drive A:, type

A:\INSTALL[Enter]

When prompted, specify to copy the files to the Windows

directory.

SetRES may now be run by selecting the SetRES icon.

Use the SetRES utility to select supported colors and

resolutions.

If your are unsure what resolutions and colors your video card

can support, refer to the Section of this manual on Clmode for

information on how to determine supported resolutions.

For help on the SetRES utility, select Help from the program

menu.

Page 13

Display drivers

The Cirrus Logic video controller is 100% VGA compatible. The

display drivers described in this manual are supplied to

improve the resolution for each supported software application

package.

It can support improved text resolution, providing greater

readability when using the supplied drivers in text-based word

processing programs. It also supports higher graphics

resolutions, providing greater detail when using the supplied

drivers in graphics-based programs.

In each application section, there will be a brief

introduction describing the application and the revision level

of the application supported by the supplied display drivers.

The installation instructions for each display driver will

follow the introduction section. Follow the instructions

carefully to be sure that each display driver is correctly

installed. All of the installation instructions assume that

the Cirrus Logic Drivers Diskette is located in drive A:. If

drive B: is used, the instructions should changed

appropriately.

The installation utility (INSTALL.EXE) should be used to copy

display drivers directly to the appropriate application

directories where they may be configured by the application

software.

Before you begin

It is assumed that the user is familiar with DOS and certain

DOS commands. Please review the installation instructions and

the associated DOS commands before attempting the actual

installation.

Not all video modes will be available on all systems. If an

extended mode driver is installed for a video mode that is not

available, the application program will not function properly.

There are a number of things that determine the list of

available video modes. Some of these include the current

monitor type, the amount of installed memory, and the revision

of the VGA controller. To determine which modes are available

before beginning the driver installation, it is recommended

that the user run the Clmode program and examine the list of

available video modes.

Page 14

Microsoft Windows

Installing Windows 3.1 display drivers

To install the Windows 3.1 drivers from the DOS prompt,

proceed as follows:

Insure that Windows 3.1 is already installed on your

computer.

From your Windows directory, at the DOS prompt, type

SETUP[Enter] to run the Windows SETUP.EXE program. Follow

the instructions on the screen. When you come to the screen

which lists the hardware and software components such as

display adapter (e.g. VGA, CGA, etc.), keyboard type, mouse

type, etc., go to the Display selection by using cursor

keys to move the highlighted bar and press [Enter].

From the next menu listing of display options, scroll to

the bottom of the list, and highlight the following text:

Other (Requires disk provided by a hardware manufacturer)

Press [Enter], and when prompted, insert the Windows

Display Driver diskette into drive A: and type A:\[Enter].

You will see the list of drivers and their associated

resolutions, such as:

Cirrus 54xx v1.3, 1280x1024x16

Cirrus 54xx v1.3, 1024x768x16

Cirrus 54xx v1.3, 640x480x16

Cirrus 54xx v1.3, 640x480x256

Cirrus 54xx v1.3, 640x480x64K

Cirrus 54xx v1.3, 800x600x16

Cirrus 54xx v1.3, 800x600x256

Highlight the desired choice by moving the cursor to the

correct display driver, and then press [Enter].

Continue with the remainder of the setup procedure.

To install the Windows 3.1 drivers from within Windows,

proceed as follows:

Insure that Windows 3.1 is already installed on your

computer and start Windows.

From the Main window of the Program Manager run the Windows

3.1 Setup program.

Select Change Systems Settings... from the Options menu of

Page 15

Setup.

Click on the down arrow at the right side of the Display:

line. Scroll to the end of the list of available display

drivers and select Other display (Requires disk from

OEM)....

Insert the Windows display driver diskette into drive A:

and type A:\[Enter] as the pathname, then click on OK.

You will see the list of available drivers and their

associated resolutions, such as:

Cirrus 54xx v1.3, 1280x1024x16

Cirrus 54xx v1.3, 1024x768x16

Cirrus 54xx v1.3, 640x480x16

Cirrus 54xx v1.3, 640x480x256

Cirrus 54xx v1.3, 640x480x64K

Cirrus 54xx v1.3, 800x600x16

Cirrus 54xx v1.3, 800x600x256

Highlight by moving the cursor to the desired display

driver, and then click on OK.

Continue with the remainder of the setup procedure. The

changes will not take effect until Windows is restarted.

Page 16

Autodesk - ADI 4.2

The ADI 4.2 driver provided with your Cirrus Logic VGA is the

TurboDLDClassic display list driver from Panacea. It has only

two purposes:

To speed up AutoCAD REDRAWs, PANs, and ZOOMs

To provide a more productive, user-friendly, interface to

AutoCAD (via the features described later in this manual).

The driver is memory-resident and inserts itself between

AutoCAD and the graphics board. It has no other effect on

AutoCAD's operation besides speeding the program up; it runs

with AutoShade 2 with RenderMan and 3D Studio to provide

enhanced rendering support, but does not affect the speed of

these programs, since they do not support Display Lists.

Installing TurboDLDClassic does not change any of the AutoCAD

program files or alter any of the drawing files stored on

disk. TurboDLDClassic was designed to be an easy-to-use,

feature-filled product that makes using AutoCAD faster and

more productive, not one that makes AutoCAD more complicated

and difficult to use.

How does TurboDLDClassic make AutoCAD run faster? There are

three things the driver does to speed operation:

AutoCAD stores drawings in a hierarchical structure, with

simple elements intermixed with complex ones. Every time

the screen is updated, AutoCAD must decode this structure.

TurboDLDClassic works differently. While you are working,

it translates the normal hierarchical structure into a

Display List, a series of vectors or polygon fills. When

you pan or zoom, TurboDLDClassic uses the Display List,

then writes the resulting vectors to the video board

hardware. Since the hierarchical structure does not have to

be decoded, drawing proceeds very quickly.

TurboDLDClassic also maintains a Drawing Cache. The Drawing

Cache is a compressed list which contains the current

contents of a viewport. This pre-scaled portion of the

Display List allows for even faster pans and zooms and

redraws.

TurboDLDClassic gives you numerous new user-interface

features, such as the bird's eye view. (via the features

described later in this manual).

By how much does TurboDLDClassic increase the speed of

AutoCAD? PANs and ZOOMs, aided by only the Display List, run

from two to twelve times faster than a non-display list

driver. The Drawing Cache further speeds things up to the

point that REDRAWs can be up to twenty-five times faster with

Page 17

TurboDLDClassic, compared to the graphics drivers shipped with

AutoCAD.Features

The features provided by TurboDLDClassic include:

Bird's eye view.

Accelerated redraws, pans, and zooms.

Easy to use - no new commands or special menus to learn.

Protected-mode ADI 4.2 driver - completely compatible with

AutoCAD Release 12, Release 11/386, 3D Studio and AutoShade

2 with RenderMan.

No memory conflicts. Works with AutoCAD's built-in Virtual

Memory Manager

Includes CustomColors_, which lets you interactively

customize your logical and physical colors from within

AutoCAD.

Completely compatible with all Autodesk ADI 4.2 compatible

third party software.

Supports all AutoCAD Release 12 features, including

rendering to viewports and 31-Bit regen space.

TurboDLDClassic requires a '386, '486 or Pentium based PC

which supports AutoCAD Release 12, 11/386, AutoShade 2 with

RenderMan, or 3D Studio V1.x/2.x. Additional extended memory

is recommended for optimal performance.

Quick start guide

This section summarizes the basics of installing and using

TurboDLDClassic. Please read the rest of the manual for

details.

What does it do?

TurboDLDClassic 's main function is to speed up AutoCAD pans,

zooms and redraws. The driver accomplishes this by creating

and maintaining a Display List - a fast-displaying object list

of the current drawing - dramatically increasing performance

on pans and zooms.

In addition to offering display list speed enhancement,

TurboDLDClassic provides a bird's-eye view to allow you to

move around large drawings.

What's the catch?

The only resource used by TurboDLDClassic is memory. The

driver actively uses about 300Kbytes of RAM from extended

memory for its operation. This memory is drawn from AutoCAD's

memory pool and therefore does not affect normal DOS

operation. The driver is loaded by AutoCAD at AutoCAD load

time and is unloaded when AutoCAD is exited.

Additionally, the Display List size can vary from one tenth to

three times the size of the current drawing file, particularly

when using AutoCAD 12's 31-bit regen space.

Page 18

You may wish to purchase and install additional RAM before

installing TurboDLDClassic, since it shares memory with

AutoCAD. If AutoCAD is using a lot of memory, TurboDLDClassic

may not have enough. If there is significant hard disk

activity while you are using TurboDLDClassic, it may be an

indication that you should add more memory to your system.

Installation

Insert the Cirrus Logic drivers diskette into Drive A: (or

B:). Run the Cirrus Logic installation program (INSTALL.EXE),

and choose the option for the Autodesk/AutoCAD drivers and

specify the drive and directory where you want them copied to

(such as C:\ACAD\DRV).

Since the start-up is a little bit different for AutoCAD

Release 12 than it is for older versions, please follow the

instructions for the version of AutoCAD you will be using

TurboDLDClassic with.

To configure AutoCAD 12

Begin AutoCAD with the reconfigure switch by typing

ACAD -R[Enter]

Choose option 3, Configure Video Display from the AutoCAD

configuration menu.

Type Y at the Do you want to select... message to display the

available video options for AutoCAD.

Select TurboDLD Classic by Panacea Inc. from the list of

display options.

If you chose to install TurboDLDClassic into a sub-directory

other than ACAD\DRV, be sure to modify the ACADDRV environment

variable to include that sub-directory. Otherwise, the

TurboDLDClassic selection will not appear in the list of

available drivers.

To configure AutoCAD 11/386

Run the FASTACAD.BAT that was copied to the TurboDLDClassic

sub-directory by typing:

C:\TURBODLD\FASTACAD[Enter]

The above example shows that TurboDLDClassic was installed on

drive C: in the TURBODLD sub-directory.

Start AutoCAD and reconfigure it to use TurboDLDClassic by

selecting option 5, Configure AutoCAD from the AutoCAD main

menu. From the next menu, select option 3, Configure Video

Display. Choose Item 1, P386 ADI 4.0/4.1 (R11) as your display

device. (For more information, see your AutoCAD 'Installation

Page 19

and Performance Guide.')

TurboDLDClassic configuration

After selecting the proper display device, the TurboDLDClassic

driver configuration menu will appear.

Set-up of TurboDLDClassic requires configuration of its

various operating parameters. These parameters have been

logically grouped into menus based on their interaction with

AutoCAD. A quick trip through each menu will complete the

configuration process. Context sensitive help can be obtained

by typing [?] at any menu option. Please note that the

DLDSETUP.HLP file must be present in order for help to appear.

If you get an error message, be sure that the file

DLDSETUP.HLP is in the ACAD\DRV sub-directory (R12), or in the

TURBODLD sub-directory (R11).

If at any point during the configuration process you want to

change an item on a previous screen, just press the [ESC] key

to back up. Selecting NO SAVE, EXIT at the main configuration

menu will return you to the AutoCAD configuration menu without

making any changes.

At a minimum, a graphics board and screen resolution must be

chosen from the Select Graphics Board/Resolution menu. If no

display options are configured, the driver will be

automatically configured for Generic VGA, 640 x 480 resolution

at 16 colors.

The first menu, Select Graphics Board/Resolution, configures

TurboDLDClassic for the graphics board, and display and

rendering resolutions to be used. Prior to configuring

TurboDLDClassic, run the Clmode utility and check the first

screen to determine which Cirrus Logic chip and how much

memory is on your VGA adapter.

Select Graphics Board - Selects the graphics chip being

used

Select Display Resolution - Selects AutoCAD, 3D Studio and

AutoShade main display screen resolutions

Select Rendering Resolution - Selects AutoCAD 12, 3D Studio

and AutoShade rendering resolutions

When configuring TurboDLDClassic's display options, the

Rendering Board/Resolution options described below will

automatically appear ONLY when configuring AVE Render, 3D

Studio or AutoShade. Otherwise, the Display Board/Resolution

options will be the only menu choices available.

Basic Configuration options set AutoCAD screen

characteristics, number of text lines in the command-line

area, font size and dual screen mode.

Page 20

AutoCAD Text Lines - Selects number of lines in AutoCAD

command prompt area. The default is 3 lines, but values

from 1 to 10 can be specified. A number larger than 3 might

be useful if you are using this driver in a high resolution

mode with small fonts, eliminating the need to frequently

swap back and forth between the text and graphics screens.

If you do not want any lines of text at the bottom of the

display, use AutoCAD to disable the command area (see the

AutoCAD 'Interface, Installation and Performance Guide' for

more information). Press [Return] to continue with font

selection.

Font Size - Selects AutoCAD display screen font or font

file to be used. Choose the Font Size you would like to use

for your AutoCAD menus, pull-downs, and dialog boxes:

8x8/8x14/8x16/12x20/12x24. For resolutions over 800x600 we

recommend the 12x20 or 12x24 fonts. The default is the 8x14

font.

Dual Screen Mode - Enables or disables dual screen

operation of AutoCAD.

User Interface options include the parameters for maneuvering

within AutoCAD. Using these options, the Big Picture,

Panacea's bird's-eye view, can be configured to suit your

needs or allow for better differentiation within a complex

drawing.

Double Click - The Double Click option allows you to set

the delay for TurboDLDClassic's Big Picture pop-up. This

time represents the largest amount of time that can elapse

between the release of the digitizer button and the

subsequent press of the same button in order to detect a

double click and display the Big Picture. Setting this

number too low will result in the computer seeing two

separate clicks, while setting the time too high may slow

down response time for single button presses. Values

between 10 and 30 are recommended.

The Double Click time is measured in hundredths of seconds.

PC systems have a timing resolution of 5/100ths of a

second, therefore, the entered time is rounded by the

driver to most closely approximate the nearest multiple of

5. This means that a time setting of 23 is the same as a

setting of 27, both of which get rounded to 25 (or one-

quarter of a second). In simplest terms, when adjusting the

timing value, it is best to use multiples of 5.

BP Button - Sets mouse/digitizer button to use for Big

Picture. With the BP Button option, TurboDLDClassic gives

you the flexibility to use a double click on an available

digitizer/mouse button as a Big Picture pop-up button. On

the BP Button menu line, enter the number of any available

digitizer button other than button 1 - button 1 is reserved

Page 21

as the pick button. Within AutoCAD, simply double click the

chosen button to invoke the Big Picture.

BP Highlight Mode - Controls how the Big Picture will

appear on the screen. Patt Line will display the Big

Picture using dotted lines to form the outer bounding box

and the zoom crosshairs. XOR Rect will use a contrasting

rectangle to display the Big Picture. The Both option will

use a combination of Patt Line and XOR Rect to distinguish

the Big Picture.

BP Refresh - In it's fastest mode, TurboDLDClassic does not

refresh the Big Picture until a DLDREFRESH command is

issued. Depending on a drawing's complexity, in this mode,

the Big Picture could become confusing, displaying vectors

that are now actually erased, and no longer part of the

drawing. To configure the Big Picture to be updated as

objects are drawn or erased, enable the BP Refresh. If

speed is a major concern, disable it.

BP Cache - Enables or disables TurboDLDClassic's internal

memory cache for Big Picture functions. This cache will

speed up Big Picture operations on some graphics platforms,

primarily on TIGA-based graphics boards. For this reason,

the BP Cache is normally disabled. As a reference, on VGAs,

the performance benefit of the BP Cache is as little as 1%.

With a TIGA-based graphics board, however, the performance

benefit of the BP Cache is as high as 400%.

The Expert Configuration menu sets the function of

TurboDLDClassic itself. This menu allows for customization of

the driver for speed or to adjust for memory constraints.

Display List - Enables or disables the display list feature

of TurboDLDClassic. This option should always be set to

enable, disabling the Display List will cause

TurboDLDClassic to run as an ordinary non-display list

driver.

Drawing Cache - Enables or disables TurboDLDClassic's

internal drawing cache, a compressed list of the current

viewport, which speeds up pans zooms and redraws. As in the

Display List option, Drawing Cache should normally be

enabled. In low memory situations, it may be desirable to

disable the Drawing Cache. Disabling the drawing cache will

free-up memory for AutoCAD but may or may not have a

visible effect on your ZOOM and PAN performance. For

example, on VGAs, the performance benefit of the drawing

cache is as little as 5%. With a TIGA-based graphics board,

however, the performance benefit of the drawing cache is as

high as 400%.

AutoCAD Logical Drawing Space - Using AutoCAD 31-Bit Space,

configures TurboDLDClassic for use with AutoCAD R12's 31-

Page 22

Bit logical drawing space. When set to Yes, the driver will

use AutoCAD's extended 31-Bit drawing space. Selecting No,

will use a 15-Bit drawing space, similar to that of AutoCAD

R11.

The 31-Bit logical space allows you to extend your Regen-

less zooming ability by a factor of several million, at the

cost of more memory. Additional memory is used by AutoCAD

for the drawing space, and by TurboDLDClassic for the

Display List. 31-Bit zooming and panning is about 10%-20%

slower than for 15-Bit logical space. The Use AutoCAD 31-

Bit Space setting is ignored for AutoCAD R11 installations.

Internal Command Echo - The echo of TurboDLDClassic

internal commands can be enabled or disabled with the

Internal Command Echo option. If you would like to see

TurboDLDClassic's internal commands display at the AutoCAD

command line as they are executed, enable this option.

Big Picture Zoom Mode - Sets the Big Picture (BP) zoom

definition area display options. In a zoomed view of the

static BP, as the current viewport is zoomed or panned,

Float mode causes the image in the BP to move around within

the bird's-eye window, keeping the zoomed viewport area

fixed in the center of the bird's-eye. Freeze mode will

lock the current BP contents into place to provide a better

overall frame of reference.

Regen Mode - As a convenience feature, a Regen Mode option

has been added to TurboDLDClassic. A Fast Regen will store

the AutoCAD drawing until the Display List has been

created, and then display it, all at once. The Incremental

mode displays the drawing in 'chunks' as the display list

is created. The Fast mode causes Regens to process

approximately 5%-10% faster than the incremental. Neither

mode changes memory requirements.

After all options have been set, use the arrow keys to scroll

down to the Save and Exit option and then press the [Enter]

key to continue.

Next, configure the AutoCAD screen display characteristics and

then type [Y] to accept the changes.

Exit to the AutoCAD drawing editor to begin using

TurboDLDClassic.

The last step in setting-up TurboDLDClassic is to configure

the colors for AutoCAD. From the AutoCAD Command Line, type

DLDCOLOR[Enter], to start CustomColors, TurboDLDClassic's

color configuration utility. Make any desired color changes

and then Save the new color palette and then Exit to return to

the drawing editor.

Verifying your TurboDLDClassic installation

To verify that TurboDLDClassic is running and installed

Page 23

correctly, follow one of these two simple tests.

If you have the AutoCAD side menu enabled, look for the

Panacea Logo in the lower right-hand corner.

If you are running AutoCAD without a side menu, type

DLDVER[Enter] at the AutoCAD command prompt. If

TurboDLDClassic is loaded and running, this command should

return your current version, and serial number.

Reconfiguring TurboDLDClassic

To reconfigure TurboDLDClassic follow the instructions below

for your version of AutoCAD, Release 12 or Release 11.

AutoCAD Release 12

If you need to make changes to your Release 12 TurboDLDClassic

configuration, type CONFIG[Enter] at the AutoCAD command

prompt or use ACAD -R[Enter] when starting the program. Select

option 3, Configure Video Display. Answer No[Enter] to the

Select a new videodriver ... prompt to start the

TurboDLDClassic configuration program. Make the desired

changes to the driver and then Save and Exit to continue to

the AutoCAD drawing editor.

AutoCAD release 11

To change a Release 11 configuration, select option 5,

Configure AutoCAD from the AutoCAD main menu, and then from

the configuration menu, select option 3, Configure Video

Display. Answer no to the Select a new video driver ... prompt

to start the TurboDLDClassic configuration program. Change the

desired driver options. Save and Exit to return to the AutoCAD

configuration menu. Open or begin a new drawing.

To completely reconfigure TurboDLDClassic using Panacea's

defaults, delete DLDSETUP.DAT from either the \ACAD\DRV sub-

directory (R12) or the TURBODLD sub-directory (R11) and then

follow the Driver Configuration instructions earlier in this

chapter.

Configuring an AutoCAD 11 environment

During the TurboDLDClassic installation process, a

FASTACAD.BAT file is created and placed into the TURBODLD sub-

directory. FASTACAD.BAT contains four lines that set four

separate environment variables: DLDCFG - used by

TurboDLDClassic to find all of its configuration files; and

DSPADI, RCPADI, RDPADI - used by AutoCAD, 3D Studio and

AutoShade 2 w/RenderMan, respectively, to find the driver

file. FASTACAD.BAT must be run prior to starting AutoCAD R11

and only needs to be run once per system boot. For automatic

loading of the environment variables, FASTACAD.BAT may be

added to the AUTOEXEC.BAT file or an AutoCAD start-up batch

file.

Page 24

To add FASTACAD.BAT to your AUTOEXEC.BAT file, insert the line

CALL D:\TURBODLD\FASTACAD

anywhere in the file. The above example assumes that

FASTACAD.BAT resides in a sub-directory on drive D: called

\TURBODLD.

If you don't wish to put FASTACAD in your AUTOEXEC.BAT file,

you may put it in a batch file which also starts AutoCAD, or

simply remember to run the file before starting AutoCAD.

If you get the message Out of Environment Space when you run

FASTACAD.BAT, you will need to enlarge your system's

environment. This is accomplished by adding the following

line,

SHELL=C:\COMMAND.COM /P /E:768

to your CONFIG.SYS file. The /E:768 specifies an environment

size of 768 bytes. Change this number as appropriate. You will

need to reboot after modifying your CONFIG.SYS file, in order

for the changes to take effect.

Configuring AutoShade, 3D Studio and AVE Render

AutoShade

To configure AutoShade v2.0 to use TurboDLDClassic, first run

the FASTACAD.BAT file from your TurboDLDClassic sub-directory

to set the AutoShade environment variables. Next, start

AutoShade with SHADE /R[Enter], which will allow you to

reconfigure AutoShade. For the display device, select P386

Autodesk Device Interface display driver, and for the

rendering display select the P386 Autodesk Device Interface

rendering driver. If you are running the display and rendering

screen on the same monitor (i.e. single monitor), make sure to

tell AutoShade this. A single monitor approach will require a

redraw of the display screen after a rendering screen.

Next, follow the same installation steps that were used to

select the graphics board and display and rendering

resolutions for AutoCAD.

3D Studio

Configuring 3D Studio requires three steps. First, you need to

set the environment variables for 3D Studio by running the

PANA3DS.BAT file that was copied into your TurboDLDClassic

sub-directory. Next, edit the 3DS.SET file, located in your

3DS directory. The following three lines may be changed to use

the RCPADI rendering driver. Locate the lines that begin with

RENDER-DISPLAY

Page 25

MAIN-DISPLAY

MATERIAL-DISPLAY

and change them to read

RENDER-DISPLAY=RCPADI

MAIN-DISPLAY=RCPADI

MATERIAL-DISPLAY=RCPADI

Make sure to remove the ';' or any spaces that may be present

at the beginning of the line.

It is only necessary for the RENDER-DISPLAY line to be set to

RCPADI in order to render at high resolution. If you do not

need a high resolution main display screen or if you will not

be using the materials editor, you may keep MAIN-DISPLAY and

MATERIAL-DISPLAY set to their defaults.

For 3D Studio 1.x, there is no RENDER-DISPLAY line. The

corresponding line is DEFAULT-DISPLAY and should be changed to

read

DEFAULT-DISPLAY="RCPADI"

The quotes around RCPADI must be used for this version of 3D

Studio.

The MATERIAL-DISPLAY should only be configured for RCPADI when

BOTH the Display AND Rendering Resolutions of TurboDLDClassic

are configured for 256 color resolutions. An unpredictable

Materials Editor screen will appear if anything other than 256

colors is selected. If you are unsure about the function of

the Materials Editor screen, use the Materials Editor as VGA,

you should not have any problems at all. Please note too, that

the use of the Materials Editor will not give you any more

colors to choose from.

Save the above changes and start 3D Studio to begin the last

part of the configuration procedure.

From your 3D Studio directory, delete the file 3DADI.CFG by

typing

DEL 3DADI.CFG[Enter]

This will cause 3D Studio to start in its reconfiguration

mode. Start 3D Studio by typing 3DS[Enter].

During the 3D Studio reconfiguration start-up, you are

prompted with a series of questions. After the first 3D Studio

question appears and is answered, the TurboDLDClassic

configuration program will appear. Select a graphics board and

display and rendering resolutions as you would for AutoCAD

use.

Because RCPADI device drivers, by definition, are combined

display and rendering devices, during 3D Studio

Page 26

reconfiguration, you will be brought to the TurboDLDClassic

configuration menu more than one time, once each for Display

and Rendering. It is not necessary to select display and/or

rendering resolutions a second time. Simply press the [Enter]

key to remove the help screen and then highlight Save and Exit

to continue to the next question. Also if FASTACAD.BAT is used

instead of PANA3DS.BAT to define the AutoCAD operating

environment, the TurboDLDClassic configuration menu will also

appear for RDPADI and DSPADI devices if they are present.

TurboDLDClassic only provides still rendering support for 3D

Studio. TurboDLDClassic will not play back rendered .FLI or

.FLC files. This is a limitation of 3D Studio.The Mapping Icon

colors, which are usually yellow and green, will appear as

black when using any external ADI driver. Also, 3D Studio has

problems with large fonts. It is recommended that you use the

default font settings.

AVE Render

AutoCAD 12's AVE Render uses TurboDLDClassic's rendering

capabilities to render objects and drawings. If AVE Render has

never been configured, when you first select the AutoCAD

render command, you will be forced to run through the

configuration process. You will be prompted as follows.

Select a Rendering Display Device. Since TurboDLDClassic is

a combined display/rendering device, choose item 1, P386

ADI Combined Display/Rendering Driver from the available

choices.

Configure the Rendering Graphics Board and Resolution. Here

the TurboDLDClassic configuration program will appear on

the screen. Press the [Enter] key to continue past the help

screen and display the Rendering Configuration menu. Choose

Select Graphics Board/Resolution to display graphics board

and resolution menu selections. First, choose Select Render

Graphics Board to select the graphics board to be used for

renderings. Next, choose Select Render Resolution to select

the desired rendering resolution from the list of available

choices. Select Return to Previous Menu and then select

Save and Exit.

Select Render Mode. Then select the rendering mode for

TurboDLDClassic. Select either Render to Viewport or Render

to Screen depending on how you wish to view your

renderings. Note that in order to render to a viewport you

must be using a display resolution of at least 256 colors.

Otherwise, AVE Render will not allow a render to viewport

selection.

Select a Render Hard Copy Device. If you are using a render

hard copy device select your device type from the list of

choices. If you are not using a hard copy device, accept

Page 27

the default of NULL.

After configuring AVE Render, you will return to the drawing

to render the current drawing.

If you have previously configured AVE Render, type

RCONFIG[Enter] at the AutoCAD command prompt to manually

display the Render Configuration menu. Follow these steps to

re-configure AVE Render.

Select option 2, Configure Rendering Device to choose a new

rendering driver. Answer [Yes] to the Select Different

Rendering Device question.

Select option 1, P386 Combined Display/Rendering Driver.

Configure the rendering graphics board and resolution. Here

the TurboDLDClassic configuration program will appear on

the screen. Press the [Enter] key to continue past the help

screen and display the Rendering Configuration menu. Choose

Select Graphics Board/Resolution to display graphics board

and resolution menu selections. First, choose Select Render

Graphics Board to select the graphics board to be used for

rendering. Next, choose Select Render Resolution to select

the desired rendering resolution from the list of available

choices. Select Return to Previous Menu and then Save and

Exit.

Select the rendering mode for TurboDLDClassic. Select

either Render to Viewport or Render to Screen depending on

how you wish to view your renderings. Note that in order to

render to a viewport you must be using a Display resolution

with at least 256 colors. Otherwise, AVE Render will not

allow a render to viewport selection.

Select Exit to the Drawing Editor from the Render

Configuration menu and then type [Y] to keep the changes

you've just made. Press the [F1] key to change to the

graphics screen if necessary.

Changing colors

Included with TurboDLDClassic is the CustomColors color

configuration utility. What is so unique about CustomColors is

that it gives you the ability to modify all of your changeable

AutoCAD colors, including menu colors, text colors, dialog box

colors, and even drawing colors, while running AutoCAD.

CustomColors simulates an AutoCAD screen, complete with all

possible objects. At the AutoCAD drawing editor command line,

type DLDCOLOR[Enter] to edit the color configuration. Once you

enter the utility, you are provided with the following

configuration menu at the bottom of the screen:

Page 28

(O)bject,

(D)rawing,

(P)hysical,

(A)DI reset,

(V)GA reset,

(L)oad,

(S)ave,

(E)xit

The menu items perform the following functions.

Object - Selecting this option will allow you to change the

color of any AutoCAD screen object, such as the graphics

area background color, the menu area text color, or the

border line color.

Select the object whose color you want to change by moving

the highlight box around with the left and right cursor

control keys and pressing [Enter] when you have highlighted

the desired object. Note that a one line description of the

object type is displayed at the top of the screen as you

move the box around.

Once you have selected the object, another highlight box

will appear around the color boxes nearest to the bottom of

the screen. These are the physical colors that the video

board supports. Use the left and right cursor keys to

select the physical color to be used for the object you

have selected and press [Enter]. The screen will quickly

redraw with the new color selection for the object you have

chosen, and you will be back at the start of object

selection in case you want to change the color of another

object.

Note that at any time during color configuration, you can

press the [Esc] key once or twice and return to AutoCAD.

Also, pressing the question mark [?] key will provide you

with context sensitive help.

Drawing - This option allows you to modify AutoCAD drawing

colors 0, 8 and 9, in a fashion similar to the way you

change object colors.

When this option is selected, a highlight square appears in

the drawing color area of the simulated AutoCAD display.

The square can be manipulated using the four cursor keys.

You may notice that the drawing color portion of the

display is laid out just like the CHROMA drawing supplied

with AutoCAD. When you press [Enter], the highlight moves

down to the 16 physical colors, just as it does during the

object color selection. Using the arrow keys, pick the

physical color you want to represent the selected drawing

color. Press [Enter] to have your selection take effect.

Page 29

Physical - choosing the Physical option from the menu

allows you to alter the red, green, and blue components of

the physical colors. A highlight box will appear in the row

of 16 physical colors at the bottom of the screen.

Use the arrow keys to select the color you wish to edit and

press [Enter]. Three sliders will appear near the bottom of

the display, with the horizontal position of the slider for

each of the color components (red, green, or blue - RGB)

indicating the relative intensity of the component. You can

use the left and right arrow keys to move the slider for

the selected component, or type in a number from 0 to 255.

Many graphics boards do not support 255 different

intensities for each color component. Therefore, when you

enter an intensity, Custom Colors will round it to the

nearest intensity which your graphics board supports. For

example, VGAs support 64 intensities per RGB color

component. This means intensities increase in multiples of

4 (256/64), therefore an intensity of 0 is the same as an

intensity of 3.

The up and down arrows allow you to select which component

you want to alter. As you manipulate the sliders, all

objects on the display that are of the same color as the

selected physical color will change in hue. This way you

can visually determine the most appropriate setting for

your display. Pressing [Enter] will set the RGB values you

have selected for the physical color you were modifying.

ADIreset - constructs a default ADI color palette, as

defined in the Autodesk Device Interface Driver Development

Kit.

VGAreset - constructs a default VGA color palette in the

first 16 color entries. The remaining palette colors remain

identical to those used for ADI devices.

Load - reloads the color information from the file

DLDCOLOR.DAT. It will be loaded from the directory pointed

to by DLDCFG (AutoCAD R11), or from the AutoCAD R12 \DRV

directory. If DLDCOLOR.DAT cannot be found, an error

message will be displayed.

This command is useful because it allows you to go back to

your previously edited color configuration in case you have

made some mistakes in configuring your colors that you want

to undo.

Don't Accidently Wipe Out Your Changes. Selecting ADIreset,

VGAreset, or Load will irrevocably wipe out any current color

changes you may have made, unless you have just saved your new

setting, so use them only when you really need to, such as

when you have made so many color changes that you can't seem

Page 30

to get back to a reasonable place and just want to start over.

Save - Saves the current color palette to DLDCOLOR.DAT. It

will be saved to the directory pointed to by the DLDCFG

environment variable in FASTACAD, or to the \ACAD\DRV (R12)

directory if FASTACAD is not used.

Exit - Exits CustomColors. If you have made palette

changes, but not saved them, you will be asked if you want

to save your changes before exiting.

When running at more than 8 bits-per-pixel (256 colors),

setting physical colors via DLDCOLOR will have no visible

effect until a SAVE and EXIT occurs.

TurboDLDClassic commands

TurboDLDClassic offers AutoCAD users many features and

productivity options. This section alphabetically lists the

basic commands found in TurboDLDClassic and provides the

correct syntax for their usage. For a brief summary of

TurboDLDClassic commands type DLDHELP at the AutoCAD command

prompt.

The most important feature of TurboDLDClassic is the addition

of a world view called The Big PictureTM or BP for short.

Bringing up the BP allows you to view where in the current

drawing you are currently located in your active viewport, and

it also gives you the ability to dynamically move to another

part of the drawing without having to exit the function you're

currently executing.

DLDBIGPIC

To call up the Big Picture, at the AutoCAD command line, type

DLDBIGPIC[Enter], or double click the right mouse button or

button 2 on your digitizer puck. (if you haven't redefined

your BP Button double click). This will bring up the BP on the

display. You will see cross-hairs and a highlighted section

indicating which part of the whole drawing you're currently

viewing - the size of the image will be determined by the

AutoCAD logical drawing space being used, 15-Bit or 31-Bit.

The selection or pick box will have an 'X' through it. Move

the mouse/digitizer around to locate your pick box. If you

can't see a small box moving, click the left mouse button (the

pick button) to size down the pick area. In size mode, the

pick box will have a right pointing arrow in it ('-->'). If

the contents of the BP are too small to work with, you may

increase the size of the image in the window by pressing the

[+] key on your numeric keypad this performs an incremental

zoom within the BP. Likewise, pressing the [-] key will reduce

the size of the contents of the BP. Pressing the [Home] and

[End] keys will put the BP into its smallest and largest size,

respectively. The smallest BP size is defined as being exactly

the same view as that in the viewport referenced by the BP.

Page 31

The largest BP size is defined as the largest image possible

without causing a Regen.

The BP pick box is always proportioned to the proper aspect

ratio for the current viewport. Clicking the pick button again

will put you back into the pick box move mode ('X'). This

operation is very similar to using the Zoom Dynamic feature of

AutoCAD. Once you have positioned the pick box on the area you

would like displayed in the active viewport, click any button

other than the pick button to initiate the zoom, or press the

[Enter] key. If you want to abort the operation, just hold any

button down, other than the pick button, until the BP

disappears. You can also hit any key on the keyboard, other

than [Enter], to cancel the BP. Note that the BP is updated

with new drawing commands, but that moves or erases will not

update the image unless BPREFRESH is enabled. A DLDREFRESH or

a REGEN will manually update the BP with the current drawing

changes.

DLDBPCACHE*

Enables or disables TurboDLDClassic's internal cache for BP

features. This cache speeds up TurboDLDClassic's Big Picture

functions. DLDBPCACHE is normally disabled. Performance

benefits of the BP Cache with will vary from one graphics

platform to the next. For example, on VGAs, the performance

benefit of the BP Cache is as little as 1%.

DLDBPDIM

Resizing and repositioning the BP can be accomplished by

typing this command at the AutoCAD prompt. DLDBPDIM allows you

to tell the driver where you want the BP placed and how large

you want it to be. The size is limited to being between one-

quarter and one-half the width of the display. This command

functions like the pan/zoom selection box in the BP, as far as

positioning and dimensioning goes. To abort, hold the right

mouse button, or any other digitizer button, down until the

moving box disappears.

DLDBPFREEZE

Sets the Big Picture (BP) zoom definition area display

options. DLDBPFREEZE toggles the Big Picture Zoom Mode

parameter in TurboDLDClassic's Expert Configuration Menu.

In a zoomed view of the static BP, as the current viewport is

zoomed or panned, Float mode causes the image in the BP to

move around within the bird's-eye window, keeping the zoomed

viewport area fixed in the center of the bird's-eye. Freeze

mode will lock the current BP contents into place to provide a

better overall frame of reference.

DLDBPHILIGHT*

Changes the highlight used to display the Big Picture among

three modes, Patt Line, XOR Rect, and Both. Patt Lines will

use dotted lines to outline the Big Picture pick box. XOR Rect

Page 32

will use a contrasting rectangle to highlight the pick box,

and Both will use a combination of Patt Lines and XOR Rect.

DLDBPREFRSH*

This convenience feature of TurboDLDClassic, toggles the Big

Picture update mode. If disabled, the BP is updated manually

by issuing a DLDREFRESH command. When enabled, DLDBPREFRSH

will cause the BP to refresh automatically when an object is

drawn or erased. The BP operations therefore will be slower

with DLDBPREFRSH enabled.

DLDBPSTATIC

Toggles the BP to stay on the screen at all times. Use this

command if you want the BP on the screen in order to

conveniently move into it whenever you want to zoom or pan.

Typing DLDBPSTATIC will put up the BP at the position you set

with DLDBPDIM. Note that the static BP will disappear when

other menus pop-up on the screen, and it will also disappear

if you draw any objects which may overwrite the BP. With these

exceptions, the BP will remain on-screen until you either exit

the drawing screen or you type DLDBPSTATIC again.

DLDCOLOR

Invokes CustomColorsTM, Panacea's color configuration program.

DLDCOMPACT

Forces a manual garbage collect of Display List memory thereby

returning unused display list memory back to the AutoCAD

memory pool.

DLDDCACHE*

This command toggles the TurboDLDClassic drawing cache on and

off. The Drawing Cache is a compressed form of the current

viewport which speeds pans, zooms and redraws

DLDDLIST*

This command toggles the display list function of

TurboDLDClassic on and off.

Please note that if the display list is turned off, you will

be running AutoCAD as though you were using a standard non

display list driver - pans, zooms and redraws will be MUCH

slower with DLDDLIST disabled.

DLDECHO*

Toggles internal TurboDLDClassic command echoing at the

AutoCAD command line. When TurboDLDClassic commands are

executed via the digitizer or pop-up menus, they generate

internal commands which will be displayed at the AutoCAD

command line if DLDECHO is enabled. Disable DLDECHO to

simplify the command line.

DLDHELP

Provides a list of TurboDLDClassic commands with one-line

description of each, within AutoCAD. It's recommended that you

Page 33

flip to the text screen by pressing the <F1> key to view the

output.

DLDREFRESH

Refreshes the Big Picture to reflect the most current changes

to a drawing. If the static BP is up, it will be updated. If

not, the next time the BP is brought up, it will contain a

current representation of your drawing, provided no changes

were made between the DLDREFRESH command and the DLDBIGPIC

command. For Automatic update of the Big Picture, see the

DLDBPREFRSH command above.

DLDSTAT

Displays the current TurboDLDClassic status. A listing of the

current TurboDLDClassic parameters will be displayed at the

AutoCAD command line. A flip to the graphics screen is

recommended for this command.

DLDUSAGE

Use DLDUSAGE if you want a to-the-byte breakdown of how memory

is being used, specifically for display list processing.

DLDUSAGE returns information regarding the memory each

viewport is occupying. Since AutoCAD supports multiple

viewports, it is possible to have multiple display lists.

DLDVER

Displays the TurboDLDClassic version, serial number and

registered user's name at the AutoCAD command line.

DLDVISREGEN*

Toggles between the Fast and Visible Regen modes of

TurboDLDClassic. A Fast Regen will create the display list and

then display the drawing all at once. A Visible Regen will

display the drawing in chunks as the display list is created.

This command is a dynamic form of the Regen Mode parameter in

the Expert Configuration Menu. Since TurboDLDClassic's Fast

Regen mode is faster than AutoCAD's, we highly recommend its

use.

Please note that the commands above with a '*' following them,

when issued within AutoCAD, will override the selections made

during TurboDLDClassic configuration, for the current drawing

session only. Exiting AutoCAD and subsequently restarting will

cause all feature settings to revert back to those selected in

the TurboDLDClassic configuration menu. If you wish to make

the current changes permanent, reconfigure TurboDLDClassic.

Memory usage and lists

TurboDLDClassic shares extended memory with AutoCAD via the

PharLap Virtual Memory Manager. This means that

TurboDLDClassic will automatically page to disk if it uses up

all the RAM that AutoCAD has left for its use. See the AutoCAD

'Installation and Performance Guide' for more information on

Page 34

Virtual Memory Management.

Please note that if you start seeing excessive hard disk

accesses during PANs, REDRAWs, and ZOOMs while using AutoCAD

with TurboDLDClassic, try using the DLDCOMPACT command. If

this doesn't affect the amount of disk access it's probably

time to add more memory to your system. Contact your AutoCAD

dealer for assistance in upgrading your memory.

Regarding display list memory, it's important to realize that

TurboDLDClassic speeds up AutoCAD operations by creating a

Display List in memory, and sending that list to the graphics

board for pans, zooms and redraws. But a display list takes up

memory.

How much memory?

For production use, we recommend that at least 1 megabyte is

available for TurboDLDClassic. To determine how much memory

AutoCAD is using, use the status command while in AutoCAD

(refer to the AutoCAD 'Installation and Performance Guide' for

more information). The Display List for a simple drawing like

the shuttle Columbia might only require 20 Kbytes for the

Display List. Complex drawings may require several megabytes.

We have seen Display Lists for a drawing range from one-tenth

the size of the drawing file to three times the size; in

general, the Display List averages about the same as the DWG

file size. This means that you should count on the Display

List generally taking up as much as twice the DWG file size.

Also, complex objects such as circles and text ``                                                               expand'' when

translated into display list format, so a drawing with a lot

of complex objects and text will have a larger Display List

than a simpler drawing.

Keeping the Display List small

Text takes up a disproportionate amount of space in the

Display List. To keep the list small, put text in its own

layer of the drawing. Then don't display the text layer when

editing the rest of the figure. This will keep memory

consumption down and speed up PANs, REDRAWs, and ZOOMs.

Read the AutoCAD manuals

The AutoCAD manuals have an excellent section on performance,

concentrating on memory usage. To get the most performance out

of the program, read the appropriate sections of those manuals

as well as this one.

Tips and tricks

Upgrading from older Panacea DLD drivers

Be sure to remove any reference to previous DLD driver

Page 35

commands (FASTACAD calls or SET parameters) or sub-directories

that may be in your AUTOEXEC.BAT file or in AutoCAD start-up

batch files. Such references could cause AutoCAD and

TurboDLDClassic to look in the wrong place for setup

information.

If have been using a previous Panacea DLD driver and wish to

use the color palette you customized for it with

TurboDLDClassic, you may copy the DLDCOLOR.DAT file from your

older DLD's sub-directory to the sub-directory you specified

during the TurboDLDClassic installation process.

Maneuvering Through AutoCAD

Zooming

When zooming into an image, be aware of your AutoCAD grid snap

setting. If you are zoomed extremely far into a drawing and

you are having trouble moving your digitizer cursor, you may

be snapping to a point which is not part of the zoomed

viewport. If the cursor only moves to a single point, or is

not on screen at all, turn the grid snap off.

TurboDLDClassic and Paper Space

TurboDLDClassic features do not work in AutoCAD's Paper Space.

The execution of any TurboDLDClassic command in Paper Space

will result in an error message at the command prompt.

Zoom Dynamic

We do not recommended using ANY TurboDLDClassic features while

in the Zoom Dynamic mode of AutoCAD. Because TurboDLDClassic

cannot determine when Zoom Dynamic has been initiated,

unpredictable results will occur. Use the Big Picture feature

of TurboDLDClassic to accomplish the same task as Zoom

Dynamic, in a much more efficient fashion.

Switching Color Modes

When switching color modes, (i.e. from 16 to 256 or vice

versa) the DLDCOLOR command should be run in order to

reconfigure your color palette for the number of colors

selected. A black cursor and disappearing crosshairs are both

symptoms of a color palette problem.

AutoCAD R12's CONFIG command

Configuring TurboDLDClassic in the middle of a drawing session

using the CONFIG command is a bit like exiting and re-starting

AutoCAD. If are using the static Big Picture, you will have to

re-initialize it after returning from the Configuration menu.

Sticky Cursors

Page 36

Sometimes, when using the Big Picture, the digitizer cursor

will appear to stick to the edge of the bird's-eye. This is

normal and is the result of the digitizer puck responding to

the whole screen area while the Big Picture only occupies a

small portion of the screen. Because of the difference in

resolutions between the screen and the digitizer, when the

screen cursor enters the Big Picture area, the digitizer puck

has more drawing area to cover in order to get to the same

location as the screen cursor.

Using AutoCAD Commands

Since TurboDLDClassic is totally transparent to users with

respect to using normal AutoCAD commands to REDRAW, PAN, and

ZOOM, you still have to suffer from some of AutoCAD's nuances.

One of these nuances is that ZOOM ALL and ZOOM EXTENTS both

force a REGEN, because AutoCAD does not keep track of various

boundaries necessary to avoid the REGENs. And REGENs are

rather time-consuming and don't use any display list

processing to speed themselves up. One way around this problem

is to use another of AutoCAD's built-in features, namely the

VIEW command.

When you first load your image and see the whole drawing on

the screen at once, just type VIEW Save ALL[Enter], which will

save the display position you see under a view named 'All'.

Then, after you've done some detailed editing and want to

return back to the big picture, type VIEW Restore ALL[Enter],

instead of ZOOM All[Enter] or ZOOM Extents[Enter], and the

full drawing will be restored to the display at display list

speeds, without a REGEN.

Another nuance of AutoCAD's is that if you zoom in too far or

pan over too far, you may inadvertently cause a REGEN. AutoCAD

again provides a very simple solution: the REGENAUTO command.

Just type REGENAUTO Off[Enter] at the AutoCAD command prompt,

and automatic REGENs will be disabled. The REGENAUTO setting

is also saved as part of your drawing file, so you only need

to execute it once per drawing. You may even want to set

REGENAUTO off in your ACAD.DWG drawing template so that all

your drawings are created with REGENAUTO set off.

Third-Party Software

If you are having trouble with TurboDLDClassic and third-party

AutoCAD applications, be sure that the third-party application

supports ADI 4.2. In order to use the ADI 4.2 specification,

third-party applications require new T-Drivers and therefore

must be revised. If an application does not specifically say

that it is ADI 4.2 compatible, it probably is not. Check with

the manufacturer to be sure.

If your third-party application is ADI 4.2 compatible and you

Page 37

are having trouble using TurboDLDClassic, try running AutoCAD

without the third-party application to try to isolate the

problem. Also, try the third-party application with the VESA

compatible driver shipped with AutoCAD, this will also help to

isolate the source of the problem.

Also note that any third-party TSR that needs to access the

display may not work properly when using AutoCAD with any

advanced ADI display driver, especially if the TSR switches

graphics modes. Most TSRs will not support the same graphics

platforms as TurboDLDClassic and therefore, will not be able

to accommodate mode switching back and forth.

Command summary

This section alphabetically lists the basic commands found in

TurboDLDClassic. Commands have been listed alphabetically.

DLDBIGPIC calls up the Big Picture - use digitizer clicks to

define an area and zoom into it.

DLDBPCACHE enables or disables the Drawing Cache for

TurboDLDClassic advanced features.

DLDBPDIMallows resizing and repositioning of the Big Picture.

DLDBPFREEZE toggles the Big Picture Zoom Mode between Float

and Fixed modes.

DLDBPHILIGHT selects the highlight mode used to display the

Big Picture.

DLDBPREFRSH enables and Disables the Big Picture automatic

update mode.

DLDBPSTATIC toggles the Big Picture to stay on the screen at

all times.

DLDCOLORinvokes CustomColorsTM, Panacea's color configuration

program.

DLDCOMPACT forces a manual clean-up of Display List memory

returning any unused memory back to the AutoCAD memory pool.

DLDDCACHE toggles the TurboDLDClassic drawing cache on and

off.

DLDDLIST toggles the display list function of TurboDLDClassic

on and off.

DLDECHO toggles internal TurboDLDClassic command echoing at

the AutoCAD command line.

DLDHELP provides a list of TurboDLDClassic commands with one-

line description of each, within AutoCAD. It's recommended

that you flip to the text screen to view the output.

DLDREFRESH refreshes the Big Picture to reflect the most

current changes to a drawing.

DLDSTAT displays a listing of the current TurboDLDClassic

parameters. A flip to the graphics screen is recommended for

this command.

DLDUSAGEgives a to-the-byte breakdown of how memory is being

used for display list processing.

DLDVER displays the TurboDLDClassic version, serial number and

registered user's name at the AutoCAD command line.

DLDVISREGEN toggles between the Fast and Visible Regen modes

Page 38

of TurboDLDClassic.

Page 39

GEM/3

Installing GEM/3 display drivers

To install the GEM drivers the first step is to build a GEM

driver pak.

In the following three steps, you create a disk known as

the GEM DRIVER PAK. Using the standard DOS FORMAT utility,

format a blank diskette by inserting it in drive A and

typing the following:

FORMAT A: /V [Enter]

The DOS format utility will then prompt for the volume

label by displaying:

Volume Label? (11 characters max) Press Enter for none.

At this prompt, type the following volume-label name in

upper case:

GEM DRIVRPK [Enter]

Using the DOS COPY utility, copy the following files from

the GEM/3 system master diskette (comes with GEM/3) to the

newly formatted GEM DRIVRPK diskette. Type the following

sample commands assuming that the GEM/3 system master

diskette is in drive A: and the GEM DRIVRPK diskette is in

drive B:

COPY A:GEMVDI.EXE B: [Enter]

COPY A:MDGEM?.SYS B: [Enter]

Insert the Cirrus Logic drivers diskette into Drive A:. Run

the Cirrus Logic installation program (INSTALL.EXE), and

choose the option for the GEM/3 drivers and specify the

drive and directory where you want them copied to (such as

B:\). GEM/3 requires that the drivers be installed from the

root directory of a floppy disk in drive A:. Be sure to

install the drivers to the root of drive A: or drive B: and

then place the diskette in drive A: before proceeding.

Install GEM/3 (following the instructions provided in the

product's manual), selecting the standard IBM VGA Color(IBM

PS/2) driver.

After successfully completing installation, insert the

GEM/3 System Master Disk in drive A.

Move to drive A by typing A:[Enter]. Please note that you

must install from the A: drive.

Page 40

Then type GEMSETUP[Enter].

Follow the GEM/3 installation instructions up to the CHANGE

EXISTING CONFIGURATION stage. When prompted for a screen

driver choice, highlight:

OTHER (DRIVER PACK)

When prompted to insert a screen driver into drive A,

insert the newly created GEM DRIVRPK diskette.

Select one of the following:

Cirrus Logic VGA 16 Color 800x600

Cirrus Logic VGA 16 Color 1024x768

Continue with the remainder of the installation.

Page 41

Lotus 123 v2.x, Symphony, and Report Writer

To install a Lotus display driver, proceed as follows:

Run the Cirrus Logic installation program (INSTALL.EXE),

and choose the option for the Lotus 123 and Symphony

drivers and specify the drive and directory where you want

them copied to (such as C:\LOTUS).

From within the Lotus sub-directory type:

INSTALL [Enter]

From the next menu, select Advanced Options for Lotus 123

v2.2 or Change Selected Equipment for Lotus 123 v2.3.

For v2.2, select Add New Drivers to Library and go to step

3. For v2.3, select Modify Current Driver Set and go to

step 6.

From the next menu, select Modify Current Driver Set.

From the next menu, select one of the display and

resolution configurations.

From the next menu, select Return To Menu.

From the next menu, select Save Changes.

Enter the name you have selected for the driver set. We

recommend that the driver set name reflect or include the

driver's resolution (e.g. 132x43) NOTE: The default driver

set name assigned by the program is 123.SET.

You may create multiple driver sets with different display

resolutions. To run Lotus 1-2-3 or Symphony with a specific

driver set, type:

123 SETNAME [Enter] or

SYMPHONY SETNAME [Enter]

where SETNAME is the name of the desired driver set.

Page 42

Lotus 123 v3.X

To install a Lotus 3.0 or 3.1 display driver, proceed as

follows:

It is assumed that Lotus 1-2-3 v3.0 or 3.1 is already

installed on your hard disk, using the standard Lotus

installation utility. To install this driver, make the

directory containing 1-2-3 v3.0 the current directory. For

example, if 1-2-3 is in the directory named 123R3, then

type the following command:

CD \123R3[Enter]

One of the files used in the driver installation is named

INSTALL.DDF. This is a text file used by the 1-2-3

installation utility, containing information on each of the

drivers and modes of operation. If you have been using a

driver supplied with another graphics adapter, you may want

to make a backup copy of the INSTALL.DDF file currently in

your 1-2-3 directory.

COPY INSTALL.DDF INSTALL.BAK [Enter]

Run the Cirrus Logic installation program (INSTALL.EXE),

and choose the option for the version of Lotus 123 that you

are using and specify the drive and directory where you

want them copied to (such as C:\123R3). If the INSTALL.DDF

has been copied to a backup fille (see step 2 above),

answer YES to the prompt to confirm overwriting the file.

Now run the Lotus installation utility by typing:

INSTALL[Enter]

Select Change selected equipment from the list. Note: If

you are currently using another driver supplied with a

graphics board, it may be necessary to choose First-time

installation.

Select Modify Current DCF, or Choose another DCF to modify,

according to your preference. The following procedure is

valid for either selection.

Select Change Video Display. This will present you with a

list of all the display types provided with the 1-2-3

distribution, with two additional choices. GD542x 100

Column Display (800x600 resolution) supports 100x31,

100x42, and 100x75 16 color modes. GD542x 128 Column

Display (1024x768 resolution) supports 128x40, 128x54, and

128x96 16 color modes. Select a resolution that best meets

your requirements.

After you selection is made, choose the Save Changes option

Page 43

from the menu. Note that when some high-resolution modes

are selected, a message may appear asking for the drive

letter of the floppy disk. Before entering the drive

letter, insert the disk that is requested in the floppy

drive. After inserting the disk, you may enter the drive

letter containing that diskette.

Page 44

Ventura Publisher

Installing Ventura Publisher display drivers

To install the Ventura Publisher driver, proceed as follows:

Install Ventura Publisher V2.0 as instructed in the

product's manual, specifying the Hercules driver.

Insert the Cirrus Logic drivers diskette into Drive B:. Run

the Cirrus Logic installation program (INSTALL.EXE), and

choose the option for the Ventura Publisher drivers and

specify the drive and directory where you want them copied

to (such as A:\). Ventura Publisher requires that the

drivers be installed from the root directory of a floppy

disk indrive A:. Be sure to install the drivers to the root

of drive A: or drive B: and then place the diskette in

drive A: before proceeding.

Change to drive A: by typing A:[Enter].

After the A: prompt appears, run the VPDRV2_0.EXE program

by typing VPDRV2_0[Enter]. Note that VPDRV2_0.EXE can only

be executed from the A: drive.

Follow the step-by-step installation procedure as it

appears on the screen and select one of the following

drivers when prompted:

Cirrus Logic VGA 800x600 16 of 256K colors or greys.

Cirrus Logic VGA 1024x768 16 of 256K colors or greys.

Ventura Publisher V2.0 is now configured for the selected

screen resolution and 16 color graphics.

Page 45

WordPerfect 5.1

The high resolution WordPerfect version 5.1 driver supplied

with your Cirrus Logic VGA supports both high resolution text

and graphics. WordPerfect allows graphics and text drivers to

be set up separately, so be sure to install both of these

drivers.

Installing WordPerfect 5.1 display drivers

To install the WordPerfect high resolution text driver

supplied with your Cirrus Logic VGA, follow these

instructions:

If you have not already done so, install WordPerfect onto

your computer system. Follow the instructions supplied with

WordPerfect.

Run the Cirrus Logic installation program (INSTALL.EXE),

and choose the option for the WordPerfect drivers and

specify the drive and directory where you want them copied

to (such as C:\WP51).

Start WordPerfect, and go into SETUP by typing [Shift-F1].

Choose Display [2], then text screen type by pressing [3].

Move the cursor to where GD542X appears on the list, and

choose SELECT.

A sub-menu will appear, listing all the choices of

resolutions supported by the Cirrus Logic text driver.

Select the desired resolution.

To install the WordPerfect high resolution graphics driver

supplied with your Cirrus Logic VGA, follow these

instructions:

If you have not already done so, install WordPerfect onto

your computer system. Follow the instructions supplied with

WordPerfect.

Run the Cirrus Logic installation program (INSTALL.EXE),

and choose the option for the WordPerfect drivers and

specify the drive and directory where you want them copied

to (such as C:\WP51).

Start WordPerfect, and go into SETUP by typing [Shift-F1].

Choose Display [2], then graphics screen type by pressing

[2].

Move the cursor to where Cirrus Logic VGA appears on the

Page 46

list, and choose SELECT.

A sub-menu will appear, listing all the choices of

resolutions supported by the Cirrus Logic graphics driver.

Select the desired resolution.

Page 47

WordStar

The WordStar display drivers support both WordStar version 5.5

and 6.0 and above and WordStar 2000 version 3.5.

Installing WordStar drivers

To install the WordStar high resolution text driver supplied

with your Cirrus Logic VGA, follow these instructions:

If you have not already done so, install WordStar onto your

computer system. Follow the instructions supplied with

WordStar.

Run the Cirrus Logic installation program (INSTALL.EXE),

and choose the option for the WordStar drivers and specify

the drive and directory where you want them copied to (such

as C:\WS).

Using a text editor enter the file FONTID.CTL in the

WordStar directory and change the CRT_TYPE line to read:

CRT_TYPE=CL800.WGD

When WordStar is executed, the display driver will be used

for page preview mode.

.