����������������� ���ķ ��ķ ���ķ �  � ���������������������

�����������������   �   ��Ľ � � � �  � ���������������������

�����������������   �   ��Ľ � � � ���� ���������������������

�����������������������������������������������������������Ŀ

�  An ultimate TANDY, PCjr & PS/1 sound chip emulator for   �

�  PC-Speaker & Covox; also Disney Sound Source emulator.   �

������������������������������������͸�����������������������

� Version 3.03 �

����������������������������;������������Ŀ

� (C)opyleft 1992-1993 by FRIENDS software �

��������������������������������������������

Hello people!

At  last  I  got  the time and finetuned this program, so you can

appreciate  it  now.  What  it  does, you ask. Well, its very simple :

this  program  emulates  the 3- voice syntheser installed into some PC

clones,  such  as  PCjr, Tandy and PS/1. Besides that it emulates also

Disney  Sound  Source -  a  real  shit, but many programs supports DSS

and  doesnt  seem  to  support  Covox,  so  I added DSS emulation into

TEMU. It  works  with many games I tested and only thing that locks up

was  Wolf3D,  but hey! you don't have Roman Trunov's Covoxer to use it

along with Wolf3D?

System  requirements :  i80386SX  or  better - based machine, 20K

of  RAM,  MSDOS  3.30  or  higher (I  tested  it  only on 5.00, but it

must  work  everywhere);  program  will  not run if any software which

runs  in  V86  mode  (such  as  EMM386,  386MAX,  QEMM,  CEMM  etc) is

detected;  XMS  support  is  present,  so you can run games which uses

extended  memory  (if  an XMS driver, such as HIMEM, is loaded) or via

INT  15/Fn  87;  Some games which detects TANDY will not run; at least

you  can  find  where  it detects TANDY and patch them; PS/1 detection

is  present,  but  I  don't know how many programs detects PS/1 in the

way  I  used  to fool them. For example, Prince 2 checks bit 4 of CMOS

byte  at  address  2F;  if it is 1 they assume that this is PS/1; very

shitty, eh? I first tried to set this bit manually but after reboot my

poor CMOS was filled with default values, so I decided to patch Prince

manually.  Disney Sound Source also must be detected, at least SQ5 and

Prince 2 do it well.

Some   technical  info:  TEMU  emulates  musical  chip  on  ports

0C0h,   0C1h,  1E0h,  1E1h,  201h &  205h.  If  you  know  other  port

addresses  on  which  this  chip  can  be  located  let  me know. This

program  I  started to teach 386 - protected mode technique, thereby I

do  not  guarantee  that  program will work well on every computer and

every  system  configuration,  simply because it's my first program of

this  kind.  Besides  that it's  a alfa (beta, gamma ...) version, and

if  you  don't  know  what's a beta version you explicitely don't need

this   program.  Disney  Source  is  emulated  on  ports  378h .. 37Ah

(usually  LPT1),  but  if  your  LPT1  is on ports 3BCh..3BEh (if it's

located  EGA  mono  video  card)  you'll need to patch manually system

variable  at  0000:0408  to  378h, because all programs which uses DSS

will search for it on wrong address.

You can get short info about TEMU's options running TEMU /? or TEMU /H;

here are more detailed info about them:

/S - Set output to PC Squeaker. It sounds VERY bad, I think I'll

exclude this switch in the future, but while it is available.

/L - Set LPT port address of your Covox. If your Covox is on LPT1

and your LPT1 is on address 3BCh you can have troubles using

DSS emulation. You can patch system varable - LPT1 address -

to 378h, and LPT2 address (0000:040A) to 3BCh, then run TEMU

to use Covox in LPT2.

/T#- Set quantization frequence. The higher is frequence, more CPU

power is taken and higher is quality. Normally it's set to

something about 16KHz.

Hint:  if  music  sounds  interrupted  this  means that game uses

too  many  CLI  instructions.  Try  to  figure  them  out and to patch

them -  in  most  cases they're a overkill. For example, I patched the

CLI's  in  VGA320. DRV  from  SQ5 in palette changing routines without

any external effects.

At  last  some  programming  info  about TANDY sound chip, if you

wish  to  make  your  own  programs  which  will  use  TANDY for sound

output.  Standard  TANDY  have  this  chip  installed on hardware port

C0h  and  thought  Jordain  say  that  on  PCjr  it's  on  port A0 all

programs  which  support  PCjr  outputs  also  to  C0. Chip have eight

registers:

���������������������������������������������Ŀ

� N �    Bits     � Contains                  �

���������������������������������������������Ĵ

� 0 �     10      � First channel - frequence �

� 1 �      4      � First channel - volume    �

� 2 �     10      � Second channel - frequence�

� 3 �      4      � Second channel - volume   �

� 4 �     10      � Third channel - frequence �

� 5 �      4      � Third channel - volume    �

� 6 �      2      � Noise - quality           �

� 7 �      4      � Noise - volume            �

�����������������������������������������������

Noise  frequence  depend  of  two  lower  bits  in sixth register

Volume  registers  have  4  bits, so, volume have 16 different values.

To write something in a register:

���������������Ŀ

(I/O port C0)  <- �1 r r r�d d d d�

�����������������

where [rrrr] - Register number;

[dddd] - lower four bits of data

If register have more than 8 bits use second out to port C0:

���������������Ŀ

(I/O port C0)  <- �0 0�d d d d d d�

�����������������

Where: [dddddd] - higher 6 bits of frequence

Frequence  you  can  compute  in the next way: F{Hz}=~114000/x, where

x  is  a  10-bit in chip's registers.

Volume   TANDY  understands  negated,  i. e.  to  set  volume  to

maximal  you  must  use  [dddd]  =  0000b and vice versa - for maximal

volume [dddd]=1111b.

Here is a example:

mov  al,8Fh   ; lower four bits of frequence

out  C0h,al

mov  al,3Fh   ; the rest of frequence number

out  C0h,al

mov  al,90h   ; maximal volume

out  C0h,al

This  example  runs  channel  0  with  frequence  110Hz  and with

maximal volume.

If you wish to say me something you can contact me via FIDOnet

at address:

2:469/37

or 2:469/37.1@FIDOnet ; Andrew Zabolotny.

Home phone: 373-2-573656, from 09:00 to 10:00

or 23:00-24:00 local time (+1 MSK)

������������������������������������������������������������������������������

Version information.

3.00b, 05/09/93

---------------

First public release. Some strange bugs, but IT WORKS!

3.01, 15/10/93

--------------

Changed a bit DSS emulation because some games such as

Prince 2 have very slow IRQ0 handler and digital sounds

squeaks. Now when IRQ0 handler is invoked interrupt flag

is set to 1, this looks strange but all seem to be ok and

Prince 2 works also. Now TEMU detects XMS driver and if

it is so, enables A20 line via XMS driver. These changes

was done because rather TEMU conflicts with SmartDrive,

now it's all right. I think that Wolf3D will work also,

but I killed'em a month ago.

3.02, 26/12/93

--------------

DSS buffer size reduced to 16 bytes; also now VM86 supervisor

emulates some PM commands such as <mov e?x,cr#> and <mov e?x,dr#>.

Fixed some other bugz in supervisor.

3.03, 27/01/94

--------------

DSS buffer size again is 32 bytes; however if buffer is more than

16 bytes fill `status bit` is 1.

������������������������������������������������������������������������������