SDUMP INSTRUCTIONS

This program provides the ability to print the contents of the Z-

100  screen  graphically  and in color on an IDS  Prism  printer.

When executed,  SDUMP verifies that it is not already in  memory,

then, if not, will exit remaining in memory.  The segment address

to be used by calling programs is printed on the CRT.  It is then

available  for use by ZBASIC programs to do color screen dumps to

the  Prism printer.   Calling format from a BASIC program  is  as

follows:

DEF SEG =&H663      ' Segment containing subroutine

SDUMP = &H1000      ' Address of subroutine within segment

SDA$="06325417"     ' Prism printer color map

SDMX%=1             ' Dot multiplier - horizontal

SDMY%=2             ' Dot multiplier - vertical

CALL SDUMP (SDMX%,SDMY%,SDA$)

BE  SURE  TO USE THE CORRECT SEGMENT ADDRESS AS EXPLAINED  BELOW!

The dump may be aborted at any time by merely pressing any key on

the keyboard.  Color scheme is as follows:

Value          BASIC Color         Prism Color

0              Black               White (no printing)

1              Blue                Yellow

2              Green               Cyan

3              Cyan                Green

4              Red                 Magenta

5              Magenta             Red (Orange)

6              Yellow              Blue (Violet)

7              White               Black

The  SDA$  color map parameter to the subroutine maps  the  BASIC

color  (position  in  the 8-character array) to the  Prism  color

(value of the array element).   The size and the aspect ratio  of

the  resulting printed screen dump may be changed using the SDMX%

and SDMY% parameters.   Values provided in the above example give

a  good  representation of the screen  (i.e.   circles  print  as

circles and not ovals).

Until  a  better  method is devised through the study of  as  yet

unreceived  technical documentation on the Z-100 and  Z-DOS,  the

segment  address (DEF SEG) must be tied to a given release of  Z-

DOS.   Since the address can be different for different  versions

of Z-DOS,  and for different configurations with the same version

(for  example,  if you are using a Winchester),  we have provided

the FINDSEG.COM program which will print the correct address  for

your  configuration  when you run it.   Just enter

A:FINDSEG

The  address  663 as shown above is correct for  some  configura-

tions,  but not for all, so be sure to run FINDSEG and modify all

of the BASIC example files before running them.  Note that if you

are using the MENU program, the segment address changes while you

are under its control.

Flash: The better method is now available with the newest release

of Z-DOS (BIOS release 1.00,  version 1.10).   In their  infinite

wisdom,  the  powers that be deceided to restore the MS-DOS print

screen hook originally ommitted from Z-DOS (interrupt 5 generated

upon entry of shift-F12).   This utility need merely be implemen-

ted  as the interrupt 5 processor and programs need not  call  it

specifically  as it could be invoked by pressing shift-F12 (para-

meters would, however, be fixed).

The SDUMP program is written in assembler,  is structured as a Z-

DOS 'COM' file,  and occupies just under 5K of memory.   It  also

requires an IDS Prism color printer with mixing ribbon.

#M' file,  and occupies just under 5K of memory.   It  also

requires an IDS Prism color printe