%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%OP%DEMiscellaneous useful information

%CO:A,12,80%%C%%H1%%H2%M i s c e l l a n e o u s   u s e f u l   i n f o r m a t i o n%H2%%H1%

%H2%The expanded and non-expanded machine%H2%

When a Z88 has 128K or more of RAM in slot 1, it becomes expanded. The latest

V4 of the operating system is expanded automatically if internal RAM has been

upgraded to 128K or more. For previous versions of the Z88 operating system

only the first rule is obeyed. The differences between the expanded and

non-expanded computers are as follows:

%H4%Property%H4%

Size of BBC BASIC

Maximum map width

User characters

Value of EOF#-1

Some users will want to use extra memory, for filing for example, without the

burden of a 40K BASIC. If all your BASIC programs fit comfortably within the

ordinary 8K it is unhelpful to have to carry the extra 32K around, thus using

slot 2 or slot 3 to expand the memory does not expand the machine (V3 or

earlier operating systems), but just increases memory size (note that using

slot 3 for RAM causes a very heavy power drain on the Z88, due to the lack of

special power saving hardware in this slot). The unexpanded machine can use 64

user characters, but if an 80 pixel map is used, the last 48 of these will be

overwritten by map information when PipeDream is used (or another application

that write to the high resolution map). Reducing the map width to 64 pixels, or

not using the map at all, allows for free use of all 64 user characters. The

file attributes for BASIC's -1 channel ($FFFF) all hold interesting

information:

PTR#-1

EXT#-1

EOF#-1

This information can be accessed by applications by using the OS_FRM system

call, to read file attributes, and specifying a handle of $FFFF in the IX

register pair. Note that although the sequential pointer (PTR) and extent (EXT)

are returned as 4 byte values in DE and BC registers, the end of file (EOF)

result is returned in Fz, with Fz = 1 indicating the end of file (ie. TRUE or

1) and Fz = 0 otherwise (ie. FALSE or 0).

%H2%Extra CLI features%H2%

<SQUARE> + S

<SQUARE> + K

<SQUARE> + P

<DIAMOND><LEFT-SHIFT>

If there is an EPROM, which contains a file called "boot.cli", present in slot

3 at the time of a reset (soft or hard) then it will be loaded into :RAM.- and

executed as a CLI file.

Note: :RAM.- files are dangerous under the version 2.2 or 3.0 of the operating

system - see below for details.

%H2%Memory for files and applications%H2%

Each RAM card is given an allocation of space by the system which is useable by

the RAM filing device associated with that slot. This allocation is always less

than total RAM available in that card. Applications (and the special device

:RAM.-) can use the remaining memory as well as the memory allocated to files.

Because of this approach, the values given for free memory are sometimes

confusing. The free memory indicators return space available to applications,

which will generally be rather less than that available to files.

The device :RAM.- can use memory from anywhere in the system, and as such is it

very useful for large files. Due to a bug in versions 2.2 (3.0) in the

operating system, if any files are stored in the device when a soft reset

occurs, the system becomes badly confused, and it is very likely that a crash

will follow sometime later. If you need to use :RAM.- files, then it is vital

that they are deleted immediately after use. The features described in the

previous section all use :RAM.- and if these operations are used, then the

files generated should be deleted at the first opportunity.

NOTE: V4 has fixed the :RAM.- problem. Some foreign V3.xx versions contains

the fix as well. You will probably be aware of this on your own Z88.

%P0%

%H2%PipeDream format%H2%

The PipeDream file format is designed to facilitate manipulation by other

programs. No control characters, or characters above ASCII value 127 are used

in the file, unless they have been explicitly typed in (ISO characters), apart

from Carriage Return (CR) characters which are used as separators. The files

are sequential, and can be edited by any text editor.

PipeDream uses a special form of construct to add special information about the

structure of the file. All constructs begin and end with an ASCII percent sign

'%PC%'. Immediately after the first percent sign come one or more alphabetic

characters which identify the construct. Between these characters and the final

percent sign, can come any information which is relevant to the construct.

At the start of the file is a group of constructs that identify what the values

of Options Page Parameters are for the file. This is followed by the main body

of the file.

Each column in the file is preceeded by a column construct which gives details

about the column followed by all the slots in the column. Every slot that is

defined in that column has an entry.

Each entry is separated from the next one with a CR character. On each line

there can be a collection of constructs giving details about the slot.

For each column in the file there is a column construct, followed by all the

entries for that column, until the end of file.

%H2%Options Page construct%H2%

%PC%OP%PC%parameter value CR

parameter

value

CR

The various parameter values are:

AM

BM

BO

DE

DP

FM

FO

HE

HM

IW

JU

LM

LP

LS

MB

PA

PL

PS

RC

TM

TN

TP

WR

%H2%Column construct%H2%

%PC%CO: column, width, wrap-width%PC%

column

width

wrap-width

These constructs can appear anywhere in the entry for each slot:

%PC%B%PC%

%PC%C%PC%

%PC%Dn%PC%

%PC%DF%PC%

%PC%F%PC%

%PC%Hn%PC%

%PC%JL%PC%

%PC%JR%PC%

%PC%L%PC%

%PC%LC%PC%

%PC%LCR%PC%

%PC%Pn%PC%

%PC%PC%PC%

%PC%R%PC%

%PC%TC%PC%

%PC%V%PC%

%H2%General format%H2%

construct text construct text ... CR

construct

text

CR

%H2%Diary format%H2%

Each day the diary uses (an active day) is headed by a date. This date is in

the form:

%PC%dd,mm,yyyy

The %PC% character is represented as %PC%%PC%

The remainder of a diary file is simply straight text.

%P0%

%H2%EPROM format%H2%

The following is the format used in file EPROMs. Unfortunately this is not the

conventions used by the DOR system. However, it is sufficient for traversing

through the file contents. With this information you can program an application

that can retreave previously 'deleted' (overwritten) files. The format is as

follows:

$0000

...

...

...

...

$3FC0

$3FF7

$3FF8

$3FFC

$3FFD

$3FFE

$3FFF

A file entry has the form:

1 byte

1 byte

n-1 bytes

4 bytes

m bytes

When a file is saved to EPROM which has the same name as an exising file, the

byte following the filename length, is set to zero, thus marking the file

deleted. However, the name length and contents of the old file are all intact

and so by directly reading the EPROM, old files can be recovered.

%P0%

%CO:B,12,68%

high word = no. of free handles in the system. low word = ROM

version code (=3 for 2.2 and 3.0 versions).

free memory. This is an approximation and should be interpreted

with care. It is similar to the memory free indicators given by

PipeDream and the Diary (Z80asm uses it too).

-1 = machine is expanded. 0 = standard 32K machine

two characters identifying the parameter

the value of the parameter

Carriage Return ($0D)

Auto/manual

Bottom margin

Borders

Title

Decimal places

Footer margin

Footer

Header

Header margin

Insert on wrap

Justify

Left margin

Leading characters

Line spacing

Minus/Brackets

Pages

Page length

Page start no.

Rows/Columns

Top margin

Text/Numbers

Trailing characters

Wrap

letter identifying the column, eg. A

decimal number giving the column width

decimal number giving the wrap-width

bracket format

centre format

decimal places. n gives the number

floating format

free align

highlight character: n gives the number (1 to 8)

justify left

justify right

left align

leading character format

LCR align

page break: n gives the argument, 0 for unconditional

per cent character

right align

trailing character format

slot is a number slot

one of the constructs above

ASCII characters being the slot contents

Carriage Return ($0D)

File entry

File entry

Latest file entry

$FF's until

$00's until

$01

4 byte random id

size of card in banks (2=32K, 8=128K, 16=256K)

sub-type, $7E for 32K cards, and $7C for 128K (or larger) cards

'o'

'z' (file eprom identifier, lower case 'oz')

n

x

'xxxx'

m

%CO:C,12,56%

%H4%Expanded%H4%

40K

256 pixels

64

-1

T-direct output to ":RAM.-/s.sgn".

T-direct input to ":RAM.-/k.sgn".

T-direct output to :PRT.0

Freeze screen output

length of filename

'/' for latest version, $00 for old version (deleted)

filename

length of file (least significant byte first)

body of file

%CO:D,12,44%%CO:E,12,32%

%H4%Non-expanded%H4%

8K

96 pixels

16 (but see below)

0

%CO:F,12,20%%CO:G,12,8%